3GPP TSG-SA WG2 Meeting #132	S2-1903513

# Xi'an, China, April 8 – 12, 2019												 (revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1216 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration and Service Request for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the Registration and Service Request message, the UE includes List Of PDU Sessions To Be Activated and PDU Session Status. Because MA PDU Session is established over both accesses, the UE and network behaviour is not clear when the UE included those IEs in the Registration and Service Request message. According to the discussion in S2-1903512, following is proposed: 1) The network activates user plane resources of the access the Service Request / Registration request is received if the UE indicated activation in the List Of PDU Sessions To Be Activated. 2) The PDU Session Status indicates status of the MA PDU Session. The UE and network completely releases the MA PDU Session if there is mismatch. The SMF shall not indicate "skip indicator" if the SMF releases MA PDU Session over a single access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Registration and Service Request procedure for ATSSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network behaviour is not clear when and how handle List Of PDU Sessions To Be Activated and PDU Session Status IE in the Registration and Service Request message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.X (new), 4.22.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.22.X	Connection, Registration and Mobility Management procedures

4.22.X.1	Registration procedures

The signalling flow for a Registraion is based on the signalling flow in Figure 4.2.2.2.2-1 with the following differences and clarifications:

-	In step 1, if the UE want to re-activate the MA PDU Session over the access the Registration message is sent, the UE indicates PDU Session ID of the MA PDU Session in the List Of PDU Sessions To Be Activated.

If the UE locally released the MA PDU Session in both accesses, the UE indicates it in the PDU Session Status. If the AMF received the indication, the AMF invokes Nsmf_PDUSession_ReleaseSMContext service towards the SMF in order to release any network resources related to the MA PDU Session.

-	In step 22, if the AMF indicated the PDU Session Status to the UE in Registration Accept message, the UE removes locally any internal resources related to the MA PDU Sessions that are not marked as established.

4.22.X.2	UE Triggered Service Request

The signalling flow for a UE Triggered Service Request is based on the signalling flow in Figure 4.2.3.2-1 with the following differences and clarifications:

-	In step 1, if the UE want to re-activate the MA PDU Session over the access the Service Request message is sent, the UE indicates PDU Session ID of the MA PDU Session in the List Of PDU Sessions To Be Activated.

If the UE locally released the MA PDU Session, the UE indicates it in the PDU Session Status. If the AMF received the indication, the AMF invokes Nsmf_PDUSession_ReleaseSMContext service towards the SMF in order to release any network resources related to the MA PDU Session.

-	In step 11, the SMF sends N2 SM Information over the access the request is received.

-	In step 12, if the AMF indicated the PDU Session Status to the UE in Service Accept message, the UE removes locally any internal resources related to the MA PDU Sessions that are not marked as established.

* * * * Start of 2nd Change * * * *

4.22.7	Adding / Re-activating User-Plane Resources

If the UE has established a MA PDU Session but the user-plane resources over one access of the MA PDU Session have not been established, then:

-	If the UE wants to add user-plane resources over this access, the UE shall initiate the UE Requested PDU Session Establishment procedure over this access, as specified in clause 4.3.2.2. The PDU Session Establishment Request message shall contain the "MA PDU Request" indication and the PDU Session ID of the established MA PDU Session.

-	The PDU Session Establishment Accept message received by the UE may contain updated ATSSS rules for the MA PDU session.

If the UE has established a MA PDU Session and the user-plane resources over one access of the MA PDU Session have been established but are currently inactive (e.g. because the UE is CM-IDLE over this access), then:

-	If the UE wants to re-activate the user-plane resources over this access, then the UE shall initiate the Registration or UE Triggered Service Request procedure over this access, as specified in clause 4.22.X.1 and clause 4.22.X.2.

Editor's note:	It is FFS if and how the network can request the re-activation of user-plane resources for the Non-3GPP access of the MA PDU Session.

* * * * End of Changes * * * *