SA WG2 Meeting #132 	S2-1903660

08 – 12 April, 2019, Xi’an, PRC

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1218 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Standalone NPN – NID Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 3/30/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | We defined NID for use by Standalone NPNs. However following items are not clear for reader (thus also potential standalone network operators): - Reason for introducing NID for SNPN is that the PLMN IDs are assumed not to be unique rather it is assumed to be shared (e.g. ITU assigned PLMN ID with MCC 999 MNC xxx can be used by a private network operator, similarly regionally assigned shared HNI (PLMN ID) can be used by private network operators). It needs to be clarified that the PLMN ID is assumed to be shared. - For universally managed NIDs, such a clarification will also help trigger an external (regional or global) organization to step up for managing unique NID values. Thus NID needs to be unique within the scope of the PLMN ID if the combo {PLMN ID, NID} needs to be universally unique. - There is a need to differentiate locally managed NIDs and universally managed NIDs to avoid collision between them. Otherwise, locally managed NIDs could use the encoding space from universally managed NID space. (collision can cause errors in connectivity rather a nuisance for NPNs with universally managed NIDs) *Also NPNs planning to use locally managed NIDs need not wait until some external organization steps up for universally managing the NIDs rather 3GPP spec can already provide guidance on what not to use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify PLMN IDs are assumed to be shared in case of standalone NPN - Clarify locally managed NID shall come from a different encoding space |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NID management aspects will be unclear, slowing down deployments of NPNs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.30.2	Stand-alone non-public networks

#### 5.30.2.1	Identifiers

The combination of a PLMN ID and Network identifier (NID) identifies an SNPN.

NOTE 1:	The PLMN ID used for SNPNs is not required to be unique. PLMN IDs reserved for use by private networks can be used for non-public networks, e.g. based on mobile country code (MCC) 999 as assigned by ITU [78]). Thus, the same PLMN ID is assumed to be shared by multiple NPNs.

The NID shall support two assignment models:

-	Locally managed NIDs are assumed to be chosen individually by SNPNs at deployment time (and may therefore not be unique) but use a different number space than the universally managed NIDs.

-	Universally managed NIDs are assumed to be globally unique.

NOTE 2:	Which legal entities manage the number space is beyond the scope of this specification. It is assumed that the locally managed NIDs does not set the first bit to 1.

An optional human-readable network name helps to identify an SNPN during manual SNPN selection.