SA WG2 Meeting #S2-132	S2-1904591

08 - 12 April, 2019, Xi'an China

3GPP TSG-CT WG4 Meeting #90	C4-191313

# Xi'an, P.R.China; 08th – 12th April 2019

# Title:	LS on Status of FS_eIMS5G

# Response to:	LS (S2-1901397) on Status of FS_eIMS5G from SA2

# Release:	Release 16

# Work Item:	FS_eIMS5G

Source:	CT4

To:	SA2

Cc:

Contact Person:

Name:	Ulrich Wiehe

Tel. Number:

E-mail Address:	ulrich.wiehe@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

CT4 thank SA2 for their LS on Status of FS_eIMS5G, and answer SA2’s questions as follows:

Q1)	On the matter of repository(transparent) data between the IMS-AS and applicable database; what is the feasibility and the impacts to stage 3 specifications of directly routing from a SBA enabled IMS-AS to the UDR?

Answer: CT4’s understanding is that the applicable database where IMS-AS transparent data are stored is the User Data Repository as specified in 23.335 and is not the Unified Data Repository as specified in 23.501. The User Data Repository can be accessed by the HSS via the non-fully standardized Ud interface. Direct routing from an (SBA enabled) IMS-AS to the User Data Repository is not feasible as this requires to fully specify the LDAP-based Ud interface (29.335).

Q2)	For non-transparent data for Sh; what is the impact of splitting UE context data from subscription data?

Answer: Defining separate services (Nudm_Sh_SDM, Nudm_Sh_UECM) for subscription data / context data does not allow retrieval of both with a single Get request, however, this impact does not justify to interdict the splitting of UE context data from subscription data.

Q3)	On the matter of CSCF interaction with the IMS-HSS, one solution has suggested using implicit subscription instead of explicit subscription. What are the stage 3 benefits and impacts of specifying implicit subscription?

Answer: Within one service (e.g. Nhss_ims_UECM) one operation (e.g. Registration) could imply a subscription to notifications (e.g. subscription to deregistration notifications). The benefit is that only one message is sent for two operations (Registration + Subscription), as the subscription is implied by the registration. Note that the two operations cannot belong to two different services, and that a safe HTTP method (e.g. GET) cannot imply a subscription to notifications.

2. Actions:

CT4 kindly ask SA2 to take the above information into account when completing normative stage 2 work on eIMS5G.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#91		13th – 17th May 2019	Reno, US

3GPP TSG CT4#93		26th – 30th August 2019	Wroclaw, Poland