3GPP TSG-SA2 Meeting #133	S2-1904983

# May 13 - 17, 2019, Reno, NV, USA

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.734 | CR | 0023 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  |  | UICC apps |  |  | ME |  | X | Radio Access Network |  |  |  |  | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Conclusion proposal for key issue#3.2 TSN Time Synchronization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | vivo, ORANGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Vertical_LAN |  |  |  |  |  |  |  |  |  |  | Date: |  | 2019-04-29 |  |  |  |
| Category: | C |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-16 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explan7ations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | SA2#132 has agreed solution 28A as the working assumption. 28A requires a bigger payload size for gPTP message size, since it needs to carry the ingress time stamp except for normal gPTP payload. However, no scalability analysis for gPTP message transmission in 28A has been considered so far. TSN is based on an extremely precise time (e.g. in the granularity of 50ns), which will require really short synchronization period (e.g. 160ms) and frequent (g)PTP SYN messages transmission. The overhead of the continuous SYN message transmission will increase along with number of the UE. In order to support multiple TSN time domains, - Broadcast method in solution#11.2 is more efficient than unicast method in solution#28A when the number of the TSN time domains is under a limitation (e.g. <=19). Broadcast method in solution#11.2 especially can be applied in cell with all kinds of bandwidths and has no restriction on the number of TSN UE. - When the number of the time domain is bigger than the limitation (e.g. >19) and the cell is configured with a large bandwidth (e.g. >5MHz), solution#28A can be applied. Otherwise, support solution#28A only will strongly restrict the bandwidth deployment for NR cell and cause great resource overhead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | To enable support of TSN time synchronization, it is recommended to agree Solution#28A and Solution#11.2 (broadcast method only) for normative work. Whether the reference time of the TSC AI parameters, is based on the 5G GM clock or the external TSN GM clock will be resolved in normative phase. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key issue #3.2: Time synchronization aspect is not concluded. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

8.X	Conclusion for key issue 3.2

To enable support for TSN time synchronization, the following solutions and principles are agreed for normative work:

-  Solution# 28A is agreed for time synchronization of multiple (e.g. >certain number of TSN time domains in the cell with bandwidth higher than certain bandwidth) external TSN time domains via (g)PTP protocols between TSN translator functions at the side of UPF and UE.

-  Solution# 11.2 (broadcast method only) is also agreed for time synchronization of single or multiple (e.g. <= certain number of TSN time domains) external TSN time domains between the RAN node and UE.

-  Whether the reference time of the TSC AI parameters, is based on the 5G GM clock or the external TSN GM clock will be resolved in normative phase.

* * * * End of Changes * * * *