3GPP TSG-SA2 Meeting #133	S2-1905491

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1271 | rev | 8 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCP deployment option describing a name-based Service Router concept |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding a point of comparison between two distinct deployment scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a Service Router deployment option to Annex X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The community would miss on a point of comparison between two viable options. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** Change (all new) **************

Annex X (informative):
An SCP Deployment Example based on name-based Routing

# X.3	General Information

This Annex provides a deployment example for the SCP. It describes an SCP offering an SBA-platform to interconnect 5GC Services (or a subset of the respective services). The Name-based Routing is the core part of the SCP. The 5GC Services are running as microservices on cloud/deployment units (clusters). A Service Router is the communication node (access node/gateway) between the SCP and the 5GC Services and resides as a single unit within a Service Deployment Cluster. This Service Router serves multiple 5GC Service Endpoints within that cluster. For direct communication the Service Router is not used.

The Functionalities within the Service Deployment Cluster are containerized Service Functions.

Depicted in Figure X.3-1, the Service Router act as SCP termination point and offer the SBI to the respective 5GC Service Functionalities. In this example, Service Routers and 5GC functionality, although co-located, are separate components within the Service Deployment Cluster. Multiple Functionalities can exist within the Service Deployment Cluster, all served by the respective Service Router when needed to communicate to other Service Functionalities within different clusters.

Figure X.3-1: Deployment unit: 5GC functionality and co-located Service Agent(s) implementing peripheral tasks

In Figure X.3-1, the two depicted 5GC Service Functionalities (realized as Network Function Service Instances) may communicate in two ways. However, before the communication can be established between two 5GC Functionalities, Service Registration and Service Discovery need to take place, as described in Figure X.3.1-1.

# X.3.1	Service Registration and Service Discovery

Service discovery entails Function A requesting a resolvable identifier for Functionality B. This resolve request is received by the Service Router which performs the task with the help of the SCP. After the resolve is done, the 5GC Functionalities may communicate either directly without any further interaction through the SCP, when the targeted address is resolved within the same Service Deployment Cluster; or via the Service Router when the Functionality resides outside of the originator’s Service Deployment Cluster. The Service Router then acts as gateway towards the underlying SCP platform.

Figure X.3.1-1: Registering 5GC Functionalities in the SCP

# X.3.2	Overview of Deployment Scenario

Figure X.3.2-1 shows an overview of this deployment scenario. For SBI-based interactions with other 5GC functionalities, a client/consumer entity (e.g. 5GC functionality B in the cluster on the left side) communicates through the cluster’s Service Router with other entities in other clusters (e.g. 5GC Functionality D in the cluster on the right side). The target selection is performed through the NRF via the SCP. From the client’s perspective, the Service Router is the first and only contact point to the SCP. The platform resolves the requested Service identifier and aligns the results with the platform’s policies. The Service Delivery Controller performes path computation between the consumer and the producer (e.g., calculate the shortest path).

Figure X.3.2-1: (NbR-) SCP interconnects multiple deployment clusters with external NRF

************** End changes **************