3GPP TSG-SA WG2 Meeting #133 	S2- 1905060

Reno, NV, USA, May 13 - 17	(revision of S2-19xxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1325 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF context transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | eSBA, SMF context transfer |  |  |  |  |  |  |  |  |
| Summary of change: |  | A SMF Service Context Transfer procedures is proposed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not following the decision from the eSBA study phase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 4.xx.4	SMF Service Context Transfer procedures

#### 4.xx.4.2	General

In case of decommission of the Old SMF, the SMF may inform NRF that it is about to be decommissioned. NRF will in this case not include the NF profile of the Old SMF in any discovery result from the NF/NF service discovery procedure.

#### 4.xx.4.2	I-SMF Context Transfer procedure

Old I-SMF triggered from O&M procedure sends Nsmf_PDUSession_StatusNotify (I-SMF transfer indication, New SMF ID) to AMF.

Steps 2-25 in clause 4.23.4.3 follows, where in step 2 AMF selects the indicated I-SMF

#### 4.xx.4.3	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF

In case of dynamic IP address assignment (IPv4 address and/or IPv6 prefix), the procedure in figure 4.xx.4.1.1-1 assumes that the control of the IP address(es) assigned by Old SMF is moved to New SMF by O&M procedures. New SMF is in full control of the conserned IP address(es) when the transfer is complete.

Note: IF UPF has the IP point of presence from the DNN, the same UPF is used.

Figure 4.xx.4.1.1-1: Context transfer of a PDU session

1.	SM context transfer is triggered, e.g. by OAM including SUPI, PDU session ID and New SMF ID.

2.	From Old SMF to AMF Nsmf_PDUSession_StatusNotify (SMF transfer indication, New SMF ID, transfer timer). Transfer timer is used to give AMF a waiting time for UE to become IDLE or RRC inactive. If UE does not become IDLE or RRC inactive during this time, the AMF will anyway go on with the transfer.

3.	The AMF waits until UE becomes IDLE or RRC inactive or tranfer timer times out.

4.	AMF uses New SMF ID to select New SMF and sends Nsmf_PDU_Session_CreateSMContext request (PDU Session ID, SM Context ID, UE location info, Access Type, RAT Type, Operation Type, SMF transfer indication)

5.	From New SMF to Old SMF SMF Nsmf_PDUSessionContextRequest request (SM Context type, SM Context ID, SMF transfer indication).

6.	The Old SMF releases the N4 session.

7.	From Old SMF to New SMF Nsmf_PDUSessionContextRequest respons (SM Context). The SM Context includes the IP address(es) in case PDU session is of typ IPv4, IPv6 or IPv6v4, or the Ethernet MAC address(es) in case PDU session type Ethernet.

8.	(Contitional) 	If dynamic PCC is used for the PDU Session, the ew SMF sets up a new policy association towards PCF

9.	New SMF establishes N4 session

10.	From New SMF to AMF: Nsmf_PDU_Session_CreateSMContext response

11.	New SMF registers to UDM,

12.	New SMF subscribes to subscription changes for the UE

13.	(Conditional) Old SMF removes its policy association with PCF (triggered by 7)Any changes to the QoS rules need to be sent to the UE when it becomes active.

*** END CHANGES ***