3GPP TSG SA WG2 Meeting #133	S2-1906429

13th – 17th May 2019, Reno, Nevada, USA                                             (was S2-1905065)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1328 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Serving 4G only UEs by SMF+PGW-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems, T-Mobile USA, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-04-30 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF+PGW-C is allowed to be selected for a 4G only UE whose subscription has been updated to enable access to 5GS (SMF+PGW-C, PCF, CHF) via MME/SGW/HSS. A UE that does not support N1 NAS does not support any 5GS related parameters and cannot include PDU Session Id in PDN connection request. But PDU Session Id is needed for interaction with UDM, policy and charaging functions from SMF+PGW-C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PGW-C+SMF needs to generate a unique PDU session Id for each PDN connection associated with a UE that has 5GS subscription but does not support 5G NAS. Additionaly, PGW-C+SMF will not send any 5G related parameters to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SMF+PGW-C not able to support 4G only UEs. This will hinder migration plans of operators who want to anchor PDN connections in PGW-C+SMF for UEs that do not support N1 NAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.1, 4.11.0a.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 4.11.0a	Impacts to EPS Procedures

#### 4.11.0a.1	General

This clause captures changes to procedures in TS 23.401 [13] that are common to interworking based on N26 and interworking without N26.

In section 4.11, UEs are assumed to support both 5GC NAS and EPC NAS unless explicitly stated otherwise.

* * * Next Change * * * *

#### 4.11.0a.5	PDN Connection Establishment

During PDN connection establishment in the EPC, the UE and the PGW-C+SMF exchange information via PCO as described in TS 23.501 [2] clause 5.15.7. In case the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the PGW-C+SMF provides an S-NSSAI to the UE, the PGW-C+SMF should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in TS 23.501 [2] clause 6.3.8). If the PGW-C+SMF is in a VPLMN, the PGW-C+SMF uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values.

If PGW-C+SMF is selected for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/EUTRAN or ePDG, the PGW-C+SMF creates unique PDU Session ID for each PDN connection of the UE for interaction with UDM, policy and charaging functions. The SMF+PGW-C shall not provide any 5GS related parameters to the UE.

* * * End Of Change * * * *