3GPP TSG- SA WG2 Meeting #133	S2-1906640

# Reno, NV, USA, 13-17 May 2019													       (was S2-1905399)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1393 | rev | 1 | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Context Transfer Support in SDSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Deutsche Telekom, Telecom Italia, Sprint, AT&T, Verizon, Telstra |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#132, Service Context Transfer procedures (S2-1904843) were apporved in TS23.502. SDSF shall support Service Context Transfer with Any NF/NF service. To specify this, we update the specification of SDSF. Service Context identifier is assigned by SDSF in order to identify Service Context. We also propose that Service Context identifier should be only unique in the same NF or NF service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initial Rev - To add the new service in UDSF service Rev 1 - Use of SDSF for data exposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SDSF does not support Service Context Transfer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.14.1, 5.2.14.3(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * First change * * * *

5.2.X	SDSF Services

5.2.X.1	General

The following table illustrates the SDSF Services.

Table 5.2.X.1-1: NF services provided by SDSF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| SMF Context | Query | Request/Response | SMF |
|  | Push | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | Update | Request/Response | SMF |

The following table shows the data that may be stored in the SDSF along with a time stamp. The Data keys uniquely identify the data to be retrieved from the SDSF.

:

Table 5.2.12.1-X: Data stored in the SDSF

| Category | Services | Description | Data key |
| --- | --- | --- | --- |
| SMF Context | SMF Context Transfer | A SM Context that is exposed for e.g. Context Transfer by a Source SMF | SUPI, PDU Session ID |

5.2.X.2	Nsdsf_SMContext service

5.2.X.2.1	General

The operations defined for Nsdsf_SMContext service use following set of parameters defined in this clause:

-	Data Keys defined in Table 5.2.12.1-X

5.2.X.2.2	Nsdsf_SMContext_Query service operation

Service operation name: Nsdsf_SMContext _Query.

Description: NF service consumer intends to query SM Context from SDSF.

Inputs, Required: Data Key(s)

Inputs, Optional: None.

Outputs, Required: Requested SM Context.

See Table 5.2.8.2.10-1 for single SM Context that may be transferred, and Table 5.2.2.2.2-1 for single UE Context that may be transferred.

.

Outputs, Optional: None.

5.2.X.2.2	Nsdsf_SMContext_Push service operation

Service operation name: Nsdsf_SMContext_Push.

Description: NF service consumer intends to push SM Context into the SDSF.

Inputs, Required: Data Key(s), SM Context.

Inputs, Optional: None

Outputs, Required: Result indication.

Outputs, Optional: None.

5.2.X.2.3	Nsdsf_SMContext_SMF_Delete service operation

Service operation name: Nsdsf_SMContext _Delete.

Description: NF service consumer intends to delete SM Context in the SDSF.

Inputs, Required: Data Key(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

5.2.X.2.4	Nsdsf_SMContext _Update service operation

Service operation name: Nsdsf_SMContext _Update.

Description: NF service consumer intends to update SM Context in the SDSF.

Inputs, Required: Data Key(s), SM Context.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

* * * * End of Changes * * * *