# SA WG2 Meeting #S2-134	S2-1907100

24 - 28 June, 2019, Sapporo, Japan

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1505 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 23.501: Clarifications to Slice-Specific Authentication and Authorisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS |  |  |  |  |  | Date: |  |  | 2019-06-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A few clarifications to the text are required to correctly describe the functionality. For revocation of network slice-specific authentication and authorization, the case where the revocation applies to the “last” S-NSSAI in the Allowed NSSAI is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.2.9.2 is clarified to normative text. Clause 4.2.9.4 is completed to indicate that for revocation of network slice-specific authentication and authorization applying to the “last” S-NSSAI in the Allowed NSSAI is missing, the AMF shal trigger UE de-registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and incomplete functionality for network slice-specific authentication and authorization |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9.2, 4.2.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.2.9.2	Network Slice-Specific Authentication and Authorization

Figure 4.2.9.2-1: Network Slice-Specific Authentication and Authorization procedure

1.	For S-NSSAIs that are requiring Network Slice-Specific Authentication and Authorization, based on change of subscription information, or triggered by the AAA-S, the AMF may trigger the start of the Network Slice Specific Authentication and Authorization procedure.

If Network Slice Specific Authentication and Authorization is triggered as a result of Registration procedure, the AMF may determine, based on UE Context in the AMF, that for some or all S-NSSAI(s) subject to Network Slice Specific Authentication and Authorization, the UE has already been authenticated following a Registration procedure on a first access. Depending on Network Slice Specific Authentication and Authorization result (e.g. success/failure) from the previous Registration , the AMF may decide, based on Network policies, to skip Network Slice Specific Authentication and Authorization for these S-NSSAIs during the Registration on a second access.

If the Network Slice Specific Authentication and Authorization procedure corresponds to a re-authentication and re-authorization procedure triggered as a result of AAA Server-triggered UE re-authentication and re-authorization for one or more S-NSSAIs, as described in 4.2.9.2, or triggered by the AMF based on operator policy or a subscription change and

if S-NSSAIs that are requiring Network Slice-Specific Authentication and Authorization are included in the Allowed NSSAI for each Access Type, the AMF selects an Access Type to be used to perform the Network Slice Specific Authentication and Authorization procedure based on network policies.

2.	The AMF may request the UE User ID for EAP authentication (EAP ID) for the S-NSSAI in a NAS MM Transport message including the S-NSSAI. This is the S-NSSAI of the H-PLMN, not the locally mapped S-NSSAI value.

3.	The UE provides the EAP ID for the S-NSSAI alongside the S-NSSAI in an NAS MM Transport message towards the AMF.

4.	The AMF sends the EAP ID to the AUSF in a Nausf_Communication_EAPMessage_Transfer (EAP ID Response, AAA-S address, GIPSI, S-NSSAI).

5.	If the AAA-P is present (e.g. because the AAA-S belongs to a third party), the AUSF invokes the Nausf_Communication_EAPmessageTranfser servce to forward the message to the AAA-P otherwise the AUSF forwards the message directly to the AAA-S.

6.	The AAA-P associates AAA-S address with the S-NSSAI and forwards the EAP Identity message to the AAA-S addressable by the AAA-S address together with S-NSSAI and GPSI.

7-14.	EAP-messages are exchanged with the UE. One or more than one iteration of these steps may occur.

15.	EAP authentication completes. An EAP-Success/Failure message is delivered to the AAA-P (or if the AAA-P is not present, directly to the AUSF) with GPSI and S-NSSAI.

16.	If the AAA-P is used, the AAA-P sends the Nausf_Communication_EAPmessageTranfser(EAP-Success/Failure, S-NSSAI, GPSI) to the AUSF.

17.	The AUSF sends the Namf_Communication_N1N2messageTranfser (EAP-Success/Failure, S-NSSAI, GPSI) to the AMF.

18.	The AMF transmits a NAS MM Transport message (EAP-Success/Failure) to the UE.

19.	If a new Allowed NSSAI or new Rejected NSSAIs needs to be delivered to the UE, or if the AMF re-allocation is required, the AMF initiates the UE Configuration Update procedure, for each Access Type, as described in clause 4.2.4.2. If the Network Slice-Specific Authentication and Authorization fails for all S-NSSAIs in the Allowed NSSAI, the AMF shall execute the Network-initiated Deregistration procedure described in clause 4.2.2.3.3, and it shall include in the explicit De-Registration Request message the list of Rejected S-NSSAIs, each of them with the appropriate rejection cause value

NEXT CHANGE

#### 4.2.9.4	AAA Server triggered Slice-Specific Authorization Revocation

Figure 4.2.9.4-1: AAA Server-initiated Network Slice-Specific Authorization Revocation procedure

1.	The AAA-S requests the revocation of authorization for the Network Slice specified by the S-NSSAI in the Revoke Authorization Request message, for the UE identified by the GPSI in this message. This message is sent to AAA-P if it is used.

2.	The AAA-P, if present, relays the request to the AUSF.

3.	The AUSF request the relevant AMF to revoke the S-NSSAI authorization for the UE.

4.	The AMF updates the UE configuration to revoke the S-NSSAI from the current Allowed NSSAI, for any Access Type for which Network Slice Specific Authentication and Authorization had been successfully run on this S-NSSAI. The UE Configuration Update may include a request to Register if the AMF needs to be re-allocated. The AMF provides a new Allowed NSSAI to the UE by removing the S-NSSAI for which authorization has been revoked. The AMF provides new rejected NSSAIs to the UE including the S-NSSAI for which authorization has been revoked. If UE has only one S-NSSAI in the Allowed NSSAI for an access, and the revocation of authorization is triggered for this S-NSSAI, the AMF shall execute the Network-initiated Deregistration procedure for the access as described in clause 4.2.2.3.3, and it shall include in the explicit De-Registration Request message the list of Rejected S-NSSAIs, each of them with the appropriate rejection cause value.

END OF CHANGES