SA WG2 Meeting #134 	S2-1907378

# June 24-28th, 2019 ; Sapporro, Japan		                                    (revision of S2-1907378)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 0029 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining support of slicing for Wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Slicing support for Wireline access is currently undefined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Slicing support for Wireline access SRVCC is not applicable to 5G-RG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Slicing support for Wireline access is currently remains undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new) ; 7.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.X Support of slicing

Slicing as defined in TS 23.501[2]  is supported with following clarifications and modifications:

- 5G-RG may receive USRP rules mapping application flows to S-NSSAI. Even though the corresponding PDU Session type is IP, layer 2 (e.g. VLAN identifiers) related traffic filters may be used to identify application flows in the corresponding URSP,

- For 5G-RG access over 3GPP access (FWA), slicing is supported as described in TS 23.501[2],

- For 5G-RG access over Wireline, the Wireline access is assumed to be able to carry slicing information together with NAS signalling tarnsported between the 5G-RG and the W-AGF,

- The W-AGF shall support the same requirements for AMF selection based on slicing request from the UE than defined for N3IWF / TNGF in TS 23.501 [2] clause 15.

NEXT CHANGE

### 7.6.1	General

This clause includes the differences for 5G-RG comparing to TS 23.502 [3] clause 4.9.

SRVCC is not applicable to RG.

END OF CHANGES