3GPP TSG-SA WG2 Meeting #134	S2-1908221

# Sapporo, Japan,  June 24 - June 28, 2019	(was S2-1908207, 8179, 7434)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0034 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on Potential QoS Change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Orange, Samsung, Tencent, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA, eV2XARC |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Remove FFS on description of Potential QoS Change and update parameters descriptions for subscriptions to reflect requirements of Notification of Potential QoS Changes feature of TS23.287. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove FFS in Clause 6.9.1; Clarify text about location information parameters included in the subscription/request to NWDAF, so that these parameters can be directly used by NWDAF without requiring further NWDAF capabilities to translate/convert such data. Updated name and description of Analytics ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements of Notification of Potential QoS Changes feature of TS23.287 are not fully supported in Rel. 16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1, 6.9.2, 6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 6.9	QoS Sustainability Analytics

### 6.9.1	General

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding  the QoS change statistics for an observation period in the past in a certain area or the likelihood of a QoS change for an observation period in the future in a certain area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e., a Request-Response model). The request includes the following parameters:

-	Analytics ID = "QoS Sustainability".

-	Analytics Filter Information:

-	QoS requirements:

-	Standardized 5QI.

-	Location information: could be an area or a path of interest. The location information could reflect a list of waypoints.

NOTE:	In this release, the consumer of the "QoS Sustainability" Analytics ID will provide location information in the area of interest format (TAIs or Cell IDs) which is understandable by NWDAF.

-	Observation Period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested.

-	Threshold(s): The threshold(s) indicate level(s) used for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification. The level(s) relate to the QoS KPIs, i.e the RAN UE Throughput, the QoS flow Retainability, etc., for the relevant 5QI(s) defined in TS 28.554 [10]:

-	for a 5QI of GBR resource type, the Threshold(s) refer to the QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Threshold(s) refer to the RAN UE Throughput KPI;

-	S-NSSAI (optional).

The NWDAF collects the corresponding statistics information on the QoS KPI for the relevant 5QI of interests from the OAM, i.e. the QoS flow Retainability or the RAN UE Throughput as defined in TS 28.554 [10].

If the Observation Period refers to the past,

-	The NWDAF verifies whether the triggering conditions for the notification of QoS change statistics are met and if so, generates for the consumer one or more notifications.

-	The analytics feedback contains the information on the location and the time for the QoS change statistics and the threshold(s) that were crossed.

If the Observation Period is in the future,

-	The NWDAF detects the need for notification about a potential QoS change based on comparing the expected value for the KPI of the target 5QI against the threshold(s) provided by the consumer in any cell in the requested area for the requested Observation Period. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

-	The analytics feedback contains the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed.

* * * * Start of 2nd Change * * * *

### 6.9.2	Input data

Table 6.9.2-1: Data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [10] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

* * * * Start of 3rd Change * * * *

### 6.9.3	Output analytics

The NWDAF outputs the QoS Sustainability analytics. Depending on the Observation Period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.9.3-1.

Table 6.9.3-1: "QoS Sustainability" Analytics

| Information | Description |
| --- | --- |
| Analytics |  |
| >Applicable Area | A list of TAIs or Cell IDs within the Location information that the analytics applies to |
| >Applicable Time Period | The time period within the Observation Period that the analytics applies to |
| >Crossed Threshold(s) | The threshold(s) that are met or exceeded by the statistics value or the expected value of the QoS KPI |
| >Confidence | Confidence of the analytics |

NOTE 1: 	The meaning of Confidence is based on the SLA, i.e., the cosumer has to understand the meaning of the different values of Confidence.

NOTE 2: 	The Analytics can contain multiple sets of the above information in case the location information reflected a list of waypoints.

* * * * Start of 4th Change * * * *

### 6.9.4 Procedures

Figure 6.9.4-1 depicts a procedure for "QoS Stainability" analytics provided by NWDAF.

Figure 6.9.4-1: "QoS Sustainability" analytics provided by NWDAF.

1.	The consumer requests or subscribes to analytics information on "QoS Sustainability" provided by NWDAF. The parameters included in the request are described in clause 6.9.1.

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Observation Period" when requesting QoS Sustainability analytics for monitoring for a path of interest.

2.	The NWDAF collects the data specified in clause 6.9.2 from the OAM.

3.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the threshold(s) provided by consumer in any cell over the requested Observation Period.

4.	The NWDAF provides response or notification on "QoS Sustainability" to the consumer.

* * * * End of Changes * * * *