3GPP TSG-SA2 Meeting #134	S2-1908033

# Sapporo, Japan, 24th Jun 2019 - 28th Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0010 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Specification clean-up |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE, Huawei, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some clean-up is needed for the specification to be fully correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updates according to the fact that AF is an NF. - Removal of the second Editor's Note in clause 6.7.2.4 because it is already solved in clause 6.2.2.1. - Fix some wording and spelling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.1.3, 6.2.2, 6.2.2.1, 6.2.2.2, 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4] apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

* * * Next change * * * *

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7 may provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of parameter types and values enables to select which type of analytics information is requested (e.g. subset of all available analytics produced by NWDAF for the given Analytics ID value) and may in addition indicate a maximum number of results expected.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	Analytics Reporting Parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	Observation period: time interval [start..end], either in the past or in the future, expressed as positive or negative offsets to the present. An interval in the past is a request for statistics. An interval in the future is a request for predictions.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer, applicable to Nnwdaf_AnalyticsInfo_Request service operation.

NOTE:	The feasibility of the parameter "Time when analytics are needed" will be checked by stage 3.

The NWDAF provides the consumer the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7, the output information listed below:

-	The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested observation period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: level of certainty, degree of confidence in statistics/prediction.

* * * Next change * * * *

### 6.2.2	Data Collection from NFs

#### 6.2.2.1	General

The Data Collection from NFs is used by NWDAF to subscribe/unsubscribe at any NF to be notified for data on a set of events.

The Data Collection from NFs is based on the services of AMF, SMF, UDM, PCF, NRF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in TS 23.502 [3] clause 4.17)

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability), and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Table 6.2.2.1-1: Services consumed by NWDAF for data collection

| Service producer | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure | 5.2.2.3 |
| SMF | Nsmf_EventExposure | 5.2.8.3 |
| PCF | Npcf_EventExposure | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| AF | Naf_EventExposure | 5.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |

Editor's note:	The exact reference in TS 23.502 [3] for Naf_EventExposure service would need to be updated when new version for TS 23.502 [3] is made available.

NOTE:	How NWDAF collects data from UPF is not defined in this Release of the specification.

To retrieve data related to a specific UE, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | FFS |  |  |
| AF | FFS |  |  |

Editor's note:	The question of determining which NEF and AF instances are serving a UE is left for further study.

The UDM instance should be determined using NRF thanks to optional request parameters as stated in clause 4.17.4 of TS 23.502 [3].

Editor's note:	The parameter(s) to include in the request to the NRF to determine the UDM holding the UE subscription are FFS.

The AMF, SMF instances should be determined using a request to UDM providing the SUPI. To determine the SMF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple SMFs (e.g. one per PDU session).

The BSF instance should be discovered using NRF thanks to optional request parameters (e.g. DNN list, IP domain list, IPv4 address range) as stated in clause 4.17.4 of TS 23.502 [3].

Editor's note:	The question of providing or provisioning these parameters to the NWDAF is left for further study.

The PCF instance serving UE PDU Session(s) should be determined using a request to BSF request providing the SUPI. To determine the PCF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple PCFs (e.g. one per PDU session).

Editor's note:	Whether there is a need to determine the PCF that serves the AMF or provides policies to the UE, and how to do it, is FFS.

Editor's note:	The question of providing or provisioning these parameters to the NWDAF is left for further study.

Editor's note:	The question of discovery of groups of UEs is left for further study.

#### 6.2.2.2	Procedure for Data Collection from NFs

The procedure in Figure 6.2.2.2-1 is used by NWDAF to subscribe/unsubscribe at NFs in order to be notified for data collection on a related event (s), using Event Exposure Services as listed in Table 6.2.2.1-1.

Figure 6.2.2.2-1: Event Exposure Subscribe/unsubscribe for NFs

1.	The NWDAF subscribes to or cancels subscription for a (set of) Event ID(s) by invoking the Nnf _EventExposure_Subscribe/ Nnf _EventExposure_Unsubscribe service operation.

NOTE 1:	The Event ID (s) are defined in TS 23.502 [3].

2.	If NWDAF subscribes to a (set of) Event ID(s), the NFs notify the NWDAF (e.g. with the event report) by invoking Nnf _EventExposure_Notify service operation.

NOTE 2:	The NWDAF can use the immediate reporting flag as defined in Table 4.15.1-1 of TS 23.502 [3] to meet the request-response model for data collection from NFs.

* * * Next change * * * *

#### 6.7.2.4	Procedures

The NWDAF can provide UE mobility related analytics, in the form of statistics or predictions or both, to another NF or to an AF. When the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to NWDAF.

Figure 6.7.2.4-1: UE mobility analytics provided to an NF

1.	The NF or trusted AF sends a request to the NWDAF for analytics on a specific UE or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF or AF can request statistics or predictions or both. The type of analytics is set to UE mobility information. The NF or AF provides the UE id or Internal Group ID which is the target for analytics.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes. This step may be skipped when e.g. the NWDAF already has the requested analytics available.

Editor's note:	Details for NWDAF collecting data from other NFs, AFs and OAM are FFS.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested UE mobility analytics to the NF or trusted AF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE mobility analytics provided by NWDAF are defined in clause 6.7.2.3.

5-6. If at step 1, the NF or trusted AF has subscribed to receive notifications for UE mobility analytics, the NWDAF may generate new analytics and provide them to the NF or trusted AF.

* * * End of changes * * * *