3GPP TSG SA WG2 Meeting #135	S2-1909055

# Split, HR, October 14-18, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1723 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UDM Cancelling the previously registered serving node during initial Registration in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per clause 5.17.2.4 of TS 23.501, upon mobility from GERAN/UTRAN to 5GS, the UE shall do initial Registration. When the AMF performs Nudm_UECM_Registration to UDM, it’s not clear whether the UDM will cancel the old SGSN if registered in HLR, as a result, the HLR may still have registered SGSN while the UE is already registered in the UDM, leading to terminating voice call failure if the HLR/SGSN is not upgraded for T-ADS data retrieval. DP S2-1909053 may be referenced for more discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify during initial Registration the UDM sends cancellation to the previously registered serving node (e.g. SGSN) via HSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminating voice call may fail as the UE may be considered in 2G/3G when in fact in AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11a.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR1744, TS23.502 CR1725 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The change of this CR needs to be implemented after the new clause 4.11a.2 in 23.502 CR1725 (S2-1909058) is introduced. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First changes ***************

#### 4.11a.2	Registration procedure

-	Step 14d

When the UDM stores the associated Access Type (e.g. 3GPP) together with the serving AMF as indicated in step 14a, it will also cause the cancellation of any previously registered serving node (e.g. SGSN) via HSS.

NOTE x: The previously registered SGSN needs to be cancelled from HLR upon mobility from GERAN/UTRAN to 5GS as specified in clause 5.17.2.4 of TS 23.501 [2].

*************** End of changes ***************