SA WG2 Meeting #136AH	S2-2000687

# Jan 13th-17th, 2020 ; Incheon, South Korea		                         			  (revision of S2-200)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0107 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to abnormal behaviour related network data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For abnormal behaviour related network analytics, TS 23.288 clause 6.7.5.1 specifies that "- The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier;" However, procedure in clause 6.7.5.4 is only detailed for target of analytics being a specific UE or a group of UEs: "1a. A consumer NF subscribes to NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID set to "Abnormal behaviour", Target of Analytics Reporting = Internal-Group-Identifier or SUPI). A consumer NF may subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic." and there is no description for how NWDAF can provide such analytics for any UE. Additionally, in clause 6.7.5.2, it is specified that NWDAF collects UE behavioural information listed in clauses 6.7.2.2 and 6.7.3.2 to derive abnormal behaviour related network data analytics, but for UE mobility and UE communication analytics the target is either a specific UE or a group of UEs. It is unclear which information NWDAF can collect for deriving abnormal behaviour analytics for a target being "any UE". Statistics / predictions specified in Table 6.7.5.3-1 / Table 6.7.5.3-2 include an "amount" information defined as "Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE")" This information is only vaid if "any UE" is a possible target for the analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the possibility to derive abnormal behaviour analytics for any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unspecified how NWDAF can derive such analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.1, 6.7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (1)

### 6.7.5	Abnormal behaviour related network data analytics

#### 6.7.5.1	General

This clause defines how to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF.

NOTE 1:	The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

The consumer of this analytics could be a 5GC NF. The 5GC NF subscribes analytics on abnormal behaviour from a NWDAF based on the UE subscription, network configuration or application layer request.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription.

The consumer of this analytics shall indicate in the request:

-	Analytics ID set to "Abnormal behaviour";

-	The Target of Analytics Reporting can be one UE or an Internal Group Identifier;

-	Optionally an Application ID, DNN or S-NSSAI;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information: optional expected UE behaviour, expected analytics type or list of Exception IDs with associated thresholds, where the expected analytics type can be mobility related, communication related or both, Area of interest, maximum number of results;

NOTE 2:	The expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer, and the list of exception IDs indicates what specific analytics are expected by the consumer, the expected analytics type and the list of Exception IDs are not presented simultaneously.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

When the NWDAF detects those UEs that deviate from the expected UE behaviour, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.7.5.3.

NEXT CHANGE (2)

#### 6.7.5.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1 and Table 6.7.5.3-2. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer of the exception with an appropriate exception ID if the exception has an identity within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer. The NWDAF shall provide the Exception Level and determine which of the other information elements to provide, depending on the observed exception.

Abnormal behaviour statistics information is defined in Table 6.7.5.3-1.

Table 6.7.5.3-1: Abnormal behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..N) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Additional measurement | Specific information for each risk |

Abnormal behaviour predictions information is defined in Table 6.7.5.3-2.

Table 6.7.5.3-2: Abnormal behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..N) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Additional measurement | Specific information for each risk |
| > Confidence | Confidence of this prediction |

The UE characteristics may provide a set of features common to all UEs affected with the exception.

The length of the SUPI list shall be lower than the parameter "maximum number of results" (see clause 6.1.3).

If PCF subscribes notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.7.5.3-2 gives examples of additional measurement, AM/SM policies and corresponding actions for solving each risk.

Table 6.7.5.3-3: Examples of policies and actions for risk solving

| Exception ID and description | Additional measurement | AM/SM policy | Actions of NFs |
| --- | --- | --- | --- |
| Unexpected UE location | Unexpected UE location (TA or cells which the UE stays) | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Unexpected long-live/large rate flows | Unexpected flow template (IP address 5 tuple) | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Time of unexpected wake-up | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Victim's address (target IP address list) | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Wrong destination address (target IP address list) | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Ping-pong UE | Numbers, frequency, time and location, assumptions about the possible circumstances | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust the UE (e.g. a stationary UE) registration area. |
| Too frequent Service Access/Abnormal traffic volume | Volume, frequency, time, assumptions about the possible circumstances | NWDAF notifies AF (Service Provider) |  |
| Unexpected radio link failures | Numbers, frequency, time and location, assumptions about the possible circumstances | Not applicable | If the unexpected radio link failures are per UE location bases, the AMF may allow the use of CE (Coverage Enhancement) in the affected location. Also, the Operator may improve the coverage conditions in the affected location. If the unexpected radio link failures are per UE bases, then the AMF and/or the AF may allow the use of CE for the affected UE. |
| Ping-ponging across neighbouring cells | Numbers, frequency, time and location information, assumption about the possible circumstances of the ping-ponging | Not applicable | If the amount of ping-ponging across neighbouring cells is above the thresholds set by the service provider, the service provider may adjust and improve the antenna tilts of the neighbouring cells or the overlapping coverage conditions in the affected location. If the ping-ponging are per UE, then the AMF and/or the AF may allow the use of Coverage Enhancement for the affected UE. |

END OF CHANGES