3GPP TSG-WG SA2 Meeting #136AH 	S2-2001035

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-2000657)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2081 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support MA PDU Session in ETSUN scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN, ATSSS |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1, According to the current description in clause 4.2.10, it describes that “UPF” may support MPTCP Proxy functionality or ATSSS-LL functionality. It is not clear which UPF it refers to in case of I-UPF or V-UPF exists. This proposal modifies “UPF” to “PSA UPF” for clarification. 2, As discussed in S2-2000656, it proposed that AMF interacts with SMF via I-SMF in control plane and UE connects PSA via I-UPF in user plane to support non-3GPP access in ETSUN scenario. This proposal adds the corresponding architecture figure in clause 4.2.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Change “UPF” to “PSA UPF”. 2, Add the architecture figure to support MA PDU session in ETSUN scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1, Descriptions on UPF functionality is not clear. 2, The architecture figure of MA PDU session in ETSUN scenario is missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 502 CR 2046 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.10	Architecture Reference Model for ATSSS Support

In order to support the ATSSS feature, the 5G System Architecture is extended as shown in Figure 4.2.10-1, Figure 4.2.10-2, Figure 4.2.10-3 and Figure 4.2.10-4. The additional functionality that is supported by the UE and the network functions shown in these figures is specified in clause 5.32 below. In summary:

-	The UE supports one or more of the steering functionalities specified in clause 5.32.6, e.g. MPTCP functionality and/or ATSSS-LL functionality. Each steering functionality in the UE enables traffic steering, switching and splitting across 3GPP access and non-3GPP access, in accordance with the ATSSS rules provided by the network. The ATSSS-LL functionality is mandatory in the UE for MA PDU Session of type Ethernet.

-	The PSA UPF may support MPTCP Proxy functionality, which communicates with the MPTCP functionality in the UE by using the MPTCP protocol [81].

-	The PSA UPF may support ATSSS-LL functionality, which is similar to the ATSSS-LL functionality defined for the UE. There is no user plane protocol defined between the ATSSS-LL functionality in the UE and the ATSSS-LL functionality in the PSA UPF. The ATSSS-LL functionality in the PSA UPF is not shown in the following three figures.

NOTE 1:	ATSSS-LL functionality is needed in the 5GC for MA PDU Session of type Ethernet.

-	In addition, the PSA UPF supports Performance Measurement Functionality (PMF), which may be used by the UE to obtain access performance measurements (see clause 5.32.5) over the user-plane of 3GPP access and/or over the user-plane of non-3GPP access.

-	The AMF, SMF and PCF are extended with new functionality that is further discussed in clause 5.32.

Figure 4.2.10-1: Non-roaming and Roaming with Local Breakout architecture for ATSSS support

NOTE 2:	The interactions between the UE and PCF that may be required for ATSSS control are specified in TS 23.503 [45].

NOTE 3:	A PSA UPF that supports the MPTCP Proxy functionality and the PMF can be connected via an N9 reference point, instead of the N3 reference point.

When I-SMF and I-UPF are inserted over 3GPP access, the 5G System Architecture is shown in Figure 4.2.10-2.

Figure 4.2.10-2: Non-roaming and Roaming with Local Breakout architecture for ATSSS support with I-SMF

Figure 4.2.10-3 shows the 5G System Architecture for ATSSS support in a roaming case with home-routed traffic and when the UE is registered to the same VPLMN over 3GPP and non-3GPP accesses. In this case, the MPTCP Proxy functionality and the PMF are located in the H-UPF.

Figure 4.2.10-3: Roaming with Home-routed architecture for ATSSS support (UE registered to the same VPLMN)

Figure 4.2.10-4 shows the 5G System Architecture for ATSSS support in a roaming case with home-routed traffic and when the UE is registered to a VPLMN over 3GPP access and to HPLMN over non-3GPP access (i.e. the UE is registered to different PLMNs). In this case, the MPTCP Proxy functionality and the PMF are located in the H-UPF.

Figure 4.2.10-4: Roaming with Home-routed architecture for ATSSS support (UE registered to different PLMNs)

# * * * * End of changes * * * *