3GPP TSG-SA WG2 Meeting #136AH	S2-2001163

# Incheon, Korea, 13 – 17 January 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0398 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on policy decision based on UE related analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-01-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PCF shall be able to take UE related analytics including UE mobility analytics, UE communication analytics and abnormal behaivour analtyics into account to make policy decision. However, in current specification 23.503, Other UE related analytics than abnormal behaviour are missing. Furthermore, the terminology “abnormal UE behavioiur” is not used in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE related analytics are considered for policy decision. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Other UE related analytics than abnormal behaviour are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.11, 6.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

5.3.11	Interactions between NWDAF and PCF

The Nnwdaf enables the PCF to subscribe to and be notified on slice load level analytics. The following information are notified by the NWDAF:

-	Identifier of network slice instance;

-	Load level information of network slice instance.

NOTE:	How this information is used by the PCF is not standardized in this release of the specification.

The Nnwdaf enables the PCF to request or subscribe to and be notified on observed service experience (i.e. the average observed Service MoS) as described in clause 6.4 of TS 23.288 [24].

The Nnwdaf enables the PCF to request or subscribe to and be notified on UE related analyticsas described in clause 6.7 of TS 23.288 [24].

The N23 reference point is defined for the interactions between NWDAF and PCF in the reference point representation.

* * * Fourth change * * *

6.1.1.3	Policy decisions based on network analytics

Policy decisions based on network analytics is a function that allows PCF taking actions related to the network analytics reported by the NWDAF as defined in TS 23.288 [24]. The requested and reported analytics type are identified by an analytics ID and the scope of the analytics type is identified by the Analytics Filter, as defined in TS 23.288 [24].

The PCF may subscribe to notifications of slice load level analytics using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "load level information", the Analytics Filter "network slice instance" and the Analytics Reporting Information set to either a threshold value or a time period.

When the Analytics Reporting Information is set to a threshold value, then the NWDAF notifies the PCF when the load level for the network slice instance is above the threshold and then when the load level is below the threshold. When the Analytics Reporting Information is set to a time period, then the NWDAF notifies the PCF at the time the time period is reached. Notifications are performed using Nnwdaf_AnalyticsSubscription_Notify service operation including the Analytics ID "load level information", the Analytics Filter "network slice instance" and the load level information. The PCF may cancel a subscription to network analytics using Nnwdaf_AnalyticsSubscription_Unsubscribe service operation.

The PCF may request a report of a specific Analytics ID from the NWDAF using the Nnwdaf_AnalyticsInfo_Request service operation. The NWDAF provides the load level information to the PCF at the time of the request.

The PCF may use the NWDAF service to retrieve the observed service experience (i.e. the average observed Service MoS) as described in clause 6.4 of TS 23.288 [24].

The PCF may use the NWDAF service to retrieve UE related analytics as described in clause 6.7 of TS 23.288 [24].

The PCF uses the network analytics as input to its policy decision to apply operator defined actions for example for the UE context(s) or PDU session(s).

* * * End of changes * * *