SA WG2 Meeting #136AH	S2-2000242

# Jan 13th-17th, 2020 ; Incheon, Korea		                               		  (revision of S2-1912045)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1848 | rev | 11 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the Inter PLMN UP functionality in the architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2020-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | S3-193096/S2-1908713 “LS to SA2 on UP gateway function” Having a pure stand-alone Inter PLMN UP Security Gateway (IPUPS) raises the operational difficulty that when many such IPUPS are deployed it becomes a burden for the IP backbone traffic forwarding to ensure that UL / DL and N4 traffic for a PDU Session reaches the same IPUPS Gateway ; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe inter PLMN User Plane security at PLMN borders |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing description of inter PLMN User Plane security at PLMN borders |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X; 6.2.3 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

Annex X (normative):
Inter PLMN User Plane security at the PLMN border

When deployed, Inter PLMN User Plane security at the PLMN border corresponds to following features:

- For N9 traffic received from outside of the PLMN: forward valid GTP-U traffic and discard the remaining, non-valid GTP-U traffic.

Such features are further specified in TS 33.501 [29].

Operators may deploy at the border of their network UPF(s) supporting inter PLMN User Plane security on top of other features for a N4 Session / PDU Session.

Alternatively, operators may deploy UPF(s) dedicated to supporting inter PLMN User Plane security.  Such UPF(s) act as a GTP-u switch i.e. the UP flows on inter PLMN N9 reach these UPF(s) based on their own IP address; These UPF(s) are controlled over N4 by the SMF (at least for their GTP-u switching functionality).

.

NEXT CHANGE (2)

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	TSN Translator functionality to hold and forward user plane packets for de-jittering when 5G System is integrated as a bridge with the TSN network.

-	High latency communication, see clause 5.31.8.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

- Traffic filtering used e.g. for inter PLMN N9 interface.

NEXT CHANGE (4)

NEXT CHANGE (2)

NEXT CHANGE (3) all text is new

END OF CHANGES