SA WG2 Meeting #136 	S2-2000920

# 18-22 November 2019, Reno, Nevada, USA	 	(revision of S2-1911521, S2-1909346)

Source:	Nokia, Nokia Shanghai Bell

Title:	New key issue for minimization of the load generated by data collection

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Proposal for a key issue on minimization of the load generated by data collection in single NWDAF or multi-NWDAFs environment.

# Discussion

One of the objectives listed in FS_eNA_Ph2 study is:

  - "Rel-16 NWDAF features enhancement:

    - Minimization of the load generated by NWDAF data collection"

To be able to provide analytics, NWDAF needs to collect an important amount of data. This is true in particular for statistics, but this also applies to predictions. And the amount of data can be quite important considering that analytics can be provided on a per UE basis.

Rel-17 is expected to bring new use cases, and so potentially new input data will be required for NWDAF to provide the requested service.

Solutions are needed to allow minimizing the load due to data collection process, for a single NWDAF as well as in multi NWDAF environment.

# Proposal

It is proposed to update TR 23.700-91 as follows.

FIRST CHANGE – ALL NEW TEXT

## 5.2	Key Issues

### 5.2.Y	Key Issue #Y: Minimization of the load generated by data collection

#### 5.2.Y.1	Description

To be able to provide analytics, NWDAF needs to collect an important amount of data. This is true in particular for statistics, but this also applies to predictions. The amount of data can be quite important considering that analytics can be provided on a per UE basis.

Rel-17 is expected to bring new use cases, and so potentially new input data will be required for NWDAF to provide the requested service, in addition to data already needed for analytics defined in Rel-16.

In a multiple NWDAFs environment, some NWDAF may collect the exact same data, for the same analytics ID or different analytics IDs, thus increasing further the load from data collection phase in the network.

Within the context of this key issue, it is proposed to study:

-	how to minimize the load due to data collection for a single NWDAF; and

-	how to minimize the load due to data collection in a multiple NWDAFs environment.

The study should also consider data storage aspects.

END OF CHANGES