SA WG2 Meeting #Ad-hoc 136	S2-2001718

13 - 17 January 2020, Incheon, Korea	(e-mail revision 7 of S2-2001565)

Source:	China Telecom, Huawei, China Mobile, China Unicom, ZTE, CATT

Title:	Solution for Key Issue #12: NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution for Key Issue #12: NWDAF-assisted RFSP policy.

# 1. Introduction

This solution is proposed to address key issue #12.

The RFSP mechanism has been supported in the current specification. It can be beneficial for the PCF to derive suitable RFSP index based on UEs behavior/service analytics data provided by NWDAF. Then the RFSP index could be transferred to RAN via AMF to support radio resource management e.g. help RAN determine to redirect active mode UEs to different RATs or frequency layers.

# 2. Discussion

An algorithm for NWDAF to provide suggestions of policy used in this solution is introduced as following:

A= u × VT,

-	 A and u are known parameters which could be collected from AF or NEF,

-	While VT is unknown and requires to be calculated through a specific algorithm according to above formula.

Matrix of indicating user accessing to applications (i.e. user-item matrix): A= 0111⋯0110⋮⋱⋮0010⋯1110,

-	the column of the matrix represents user,

-	the row of the matrix represents the history of user accessing to the applications,

-	and the 1 or 0 represents the user uses the application or not.

NOTE: The 0-1 matrix is an example. In reality, the A could also be fulfilled by the frequency of application used by the user.

Matrix of indicating user profile: u =   uuuu⋯uuuu⋮⋱⋮uuuu⋯uuuu,

-	the column of the matrix represents user profile (e.g. gender, category/class/level, education, age, career, etc.),

-	the row of the matrix represents user application preference corresponding to specific user profile.

Matrix VT   vvvv⋯vvvv⋮⋱⋮vvvv⋯vvvvis intermediate results.

With VT and a specific user profile u, NWDAF could provide service preference suggestions u*T for the user by using the formula:

u*T= u × VT

Matrix of indicating user accessing to applications: u*T = u1⋮11 , the column of the matrix represents the service preference suggestions on the probability of user using specific services or applications and suggestions on choosing which frequency or RAT.

# 3. Proposal

The following changes are proposed for TR 23.700-91.

* * * * First Change (All new text) * * * *

## 6.X	Solution #X: Service Behaviour analytics and Service Experience analytics for NWDAF-assisted RFSP policy

### 6.X.1	Description

#### 6.X.1.1	General

This solution is proposed to address Key Issue #12: NWDAF-assisted RFSP policy.

This solution specifies for how NWDAF can provide service behavior analytics and/or service experience analytics to a service consumer to derive a suitable RFSP index.

#### 6.X.1.2	Input data

The service data collected from the AF/NEF, the network data from other 5GC NFs and the network data from OAM for service behavior analytics and service experience analytics are defined in Table 6.X.1.2-1, Table 6.X.1.2-2 and Table 6.X.1.2-3, respectively.

Table 6.X.1.2-1: Service Data from AF related to the Service Behaviour analytics and/or Service Experience analytics

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [x1] or a customized MOS. |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

Editor’s note:	Which information is respectively exploited to generate Service Behaviour analytics and Service Experience analytics is FFS.

Table 6.X.1.2-2: QoS flow level Network Data from 5GC NF

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | PCF/SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF | The observed number of packet retransmission. |

Editor’s note:	Whether more information is needed as input data is FFS.

Editor’s note:	Which information is respectively exploited to generate Service Behaviour analytics and Service Experience analytics is FFS.

Table 6.X.1.2-3: QoS flow level Network Data from OAM

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [x2] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [x3]. |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [x2] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [x3]. |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [x4]. |
| RAT type(e.g. NR or EUTRA ) | OAM | Indicating which RAT the UE uses/camps on. |
| Frequency | OAM | Indicating which Frequency the UE uses/camps on. |

Editor’s note:	Which information is respectively exploited to generate Service Behaviour analytics and Service Experience analytics is FFS.

#### 6.X.1.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3, TS 23.288 [5], are used to expose the analytics.

-	Service Experience statistics information per observed service experience information for each Application is defined in Table 6.4.3-1 of TS 23.288 [5].

-	Service Experience predictions information per observed service experience information for each Application is defined in Table 6.4.3-2 of TS 23.288 [5].

-	Service Behavior statistics information per UE for each application is defined in Table 6.X.1.3-1.

-	Service Behavior predictions information per UE for each application is defined in Table 6.X.1.3-2.

Table 6.X.1.3-1: Service Behavior statistics

| Information | Description |
| --- | --- |
| UE ID | Identity of the UE (such as SUPI) for the Service Behavior analytics. |
| Service Behaviour (1..n) |  |
| > Application ID | Identification of the application. |
| > Occurrence probability | Probability for the application used by the UE. |
| > Duration Time | Period for the application used by the UE. |
| > Spatial validity | Area where the Service Behaviour applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |

Table 6.X.1.3-2: Service Behavior predictions

| Information | Description |
| --- | --- |
| UE ID | Identity of the UE (such as SUPI) for the Service Behavior analytics |
| Service Behaviour (1..n) |  |
| > Application ID | Identification of the application. |
| > Occurrence probability | Probability for the application used by the UE |
| > Duration Time | Period for the application used by the UE |
| > Spatial validity | Area where the Service Behaviour applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Probability assertion | Confidence of this prediction. |

Editor’s note:	Whether service behaviour analytics are required in addition to service experience analytics is FFS.

Editor’s note:	Whether more information is needed as part of service experience analytics and/or service behaviour analytics is FFS.

#### 6.X.1.4	Procedures

Figure 6.X.1.4-1: Procedure for NWDAF providing Service Behaviour analytics

1.	Consumer NF sends an Analytics subscribe (Analytics ID = Service Experience/Service Behaviour, Target of Analytics Reporting =/UE ID, Analytics Filter information = (Application ID, S-NSSAI, DNN, Area of Interest), Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	NWDAF subscribes the service data from AF in the Table 6.X.1.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = /UE ID) as defined TS 23.502 [3].

NOTE 1:	In the case of external AF, NEF translates the requested Area of Interest into a list of geographic zone identifier(s) as described in clause 5.6.7.1 of TS 23.501 [2].

3.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.X.1.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

4.	With these data, the NWDAF derives the subscribed Service Experience analytics and Service Behaviour analytics.

5.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) and service behaviour to the consumer NF by means of Nnwdaf_AnalyticsSubscription_Notify.

NOTE 2:	The call flow only shows a subscription-notify model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model as defined in clause 6.1, TS 23.288 [5].

If the consumer NF is PCF, it could use the data analytics to derive suitable RFSP index (specified in clause 5.3.4.3 of TS 23.501 [2]), i.e. Service Experience analytics and Service Behaviour analytics which are defined in clause 6.x.1.3.

### 6.X.2	Impacts

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

PCF:

-	Using the information of analysis result of NWDAF to make decision on the RSFP index.

* * * * End of Changes * * * *