3GPP TSG-SA WG2 Meeting #136	S2-1911284

# Reno, NV, USA, November 18 – 22, 2019							      revision of  S2-1910720

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1753 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ETSUN: Mobility event management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-11-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the mobility, both AMF and I-SMF may be changed. So the subscription information, e.g. subscription ID and notification information need be updated accordingly. For the subscription to AMF, i.e. mobility event: According to current description, when the AMF change, the AMF update the subscription ID stored at the I-SMF/SMF only in case of no I-SMF change/removal. So in case if AMF change and I-SMF change, the AMF will not notify the new subscription ID to the SMF/I-SMF respecitively. This is correct for the I-SMF as the new I-SMF trigger a new subscription. However this is incorrect for SMF as the SMF always need be notified of the new subscription ID in the new AMF. To simplify the AMF behaviour, if the AMF change, the AMF always notify the I-SMF/SMF the new subscription ID information per the notification information. If the I-SMF has been changed and the context in old I-SMF has been deleted, the update may fail with a response code “404 not found”, the AMF can delete the subscription in this case. If I-SMF change, per existing description, the new I-SMF may update the subscription or trigger a new subscription. However due to the AMF may also be reallocated, the subscription update only apply the case the AMF is not reallocated. A new subscription triggered from the new I-SMF apply to all case. To simplify the I-SMF behaviour, it is proposed only the new subscription activation is defined. I-SMF insertion case is similar as adding a new I-SMF without removal of old I-SMF. It can do in the similar way. In a short, the AMF do the subscription ID update if the AMF change. The SMF trigger a new subscription if the SMF change. For the subscription to SMF: For the implicit subscription requested by AMF, the AMF can always update the SMF with the new notification information. In all case for the subscription from the old NF, the subscription can be implicitly removed after the subscription timer expired per TS29.501. To simplify the I-SMF design the mobility event and SMF event are not transferred between I-SMFs. So the SMF always re-subscribe the SMF event if the I-SMF change or insertion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the target AMF update the subscription with I-SMF if AMF change. And the I-SMF trigger a new subscription if a new I-SMF is changed. Also how to do the SMF subscription from AMF is clarified. SMF event and related action when the I-SMF change/Insertion are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is complexity for the SMF/AMF to handle the subscription ID /notification information change in all case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.7, 5.34.7.1(new), 5.34.7.2(new), 5.34.7.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev3: - adding the content from S2-1910274, remove “Change of Access Type” as it can be interacted between the SMF and AMF directly. - The subscription event are not transferred between I-SMFs. |
| --- | --- |

1st CHANGE

5.34.7	Event Management

5.34.7.1	UE's Mobility Event Management

When an I-SMF is involved in a PDU Session, the SMF and I-SMF independently subscribe to "UE mobility event notification" service provided by AMF. The AMF treats the SMF's and I-SMF's subscription separately and notifies the event directly to the SMF or I-SMF. If the SMF does not know the serving AMF address, the SMF gets the serving AMF address from the UDM as described in clause 5.2.3.2.4, TS 23.502 [3] and subscribes directly with the serving AMF.

In the case of AMF change (e.g. Inter NG-RAN node N2 based handover), the target AMF receives mobility event subscription information from the source AMF and updates the mobility event subscription information with the SMF and I-SMF independently (i.e. target AMF allocates the Subscription Correlation ID for each event and notify the respective SMFs or I-SMF as described in clause 5.3.4.4).

In the case of I-SMF change or I-SMF insertion (e.g. at Inter NG-RAN node N2 based handover), the subscription of mobility event is not transferred from the old I-SMF or SMF to the new I-SMF respectively, the new I-SMF triggers a new subscription event if the new I-SMF wants to receive the  corresponding mobility event . In case of I-SMF removal, the subscription of mobility event is not transferred from the old I-SMF to SMF, the SMF triggers a new subscription event if the SMF wants to receive the corresponding mobility event.

NOTE 1: 	The subscription from the old SMF entity (old SMF, old I-SMF) can be implicitly removed after the subscription timer expires or removed via the explicitly request from the old SMF entity.

2nd CHANGE

5.34.7.2	SMF event  exposure service

Consumers of SMF event exposure services do not need to be aware of the insertion / removal / change of an I-SMF as they always subscribe to the SMF of the PDU Session.

Except for the events documented in the present clause, the I-SMF does not need to support the events defined in TS 23.502 [2] clause 5.2.8.3.1.

For Events "First downlink packet per source of the downlink IP traffic (buffered / discarded / transmitted)", when an I-SMF is involved in the PDU Session, the SMF subscribes / unsubscribes onto I-SMF on behalf of the event consumer (e.g. at I-SMF insertion or when a consumer subscribes or un subscribes while an I-SMF serves the PDU Session) and the I-SMF directly notifies the event consumer. At I-SMF change, the related SMF event subscriptions are not transferred from old I-SMF to target I-SMF, the SMF may trigger new subscription events to the target I-SMF if the SMF wants to maintain the corresponding SMF events

5.34.7.3	SM context status notification

When creating an association with a SMF or I-SMF for a PDU Session, the AMF implicitly subscribe the SM context status notification by providing a notification information, i.e. callback URI, to SMF or I-SMF respectively. In case of I-SMF insertion/change, the AMF provides the notification information to new I-SMF. In case of I-SMF removal, the AMF provides the notification information to SMF.

END OF CHANGES