3GPP TSG-WG SA2 Meeting #139E e-meeting  	S2-2003891

# Elbonia, June 1 – 12, 2020	(revision of S2-2003244)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2297 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing important assumptions on how Npcf_PolicyAuthorization_Notify is used in TSN context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, [Intel] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 4.16.5.1 (SMF initiated SM Policy Association Modification): step 2 refers to the use of a pre-configured AF session for delivering TSN-related information when there is no established AF session corresponding to the DS-TT MAC address. If the SMF has reported that a manageable Ethernet port has been detected and no AF session exists for this PDU session yet, then the PCF informs a pre-configured AF using the Npcf_PolicyAuthorization_Notify service operation of 5GS Bridge ID, the port number of the DS-TT Ethernet port, and MAC address (if available) of the DS-TT Ethernet port for the PDU Session. What is currently not stated (neither here nor anywhere else in Stage 2 specifications) is that upon reception of the Notify message over the pre-configured AF session the AF must request creation of a new AF session that is specific to the DS-TT MAC address and subsequently must subscribe for TSN-related events. The current use of “if available” in the quoted excerpt also seems wrong, because without it the AF will not be able to request creation of a new AF session. Annex F.1 (5GS Bridge information reporting): Step 2 in the call flow in Figure F.1 states that the TSN-related information received by the SMF upon PDU Session Establishemnt is further conveyed to the PCF (step 2a) and to the TSN AF (step 2b). The figure is misleading in that step 2b (Npcf_PolicyAuthorization_Notify) is delivered on the pre-configured AF session, subsequent to which the TSN AF must request creation of a new AF session and subscribe for for notification on TSN related events. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.16.5.1: Clarified in step 2 that upon reception of Npcf_PolicyAuthorization_Notify on the pre-configured AF session, the AF shall request creation of a new AF session and shall subscribe for notifications on TSN related events. Annex F.1: Clarified that step 2b (Notify) is used over a pre-configured AF session, and added two more steps (Create and Subscribe, in step 2c and 2d, resp.). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Important assumptions on how Npcf_PolicyAuthorization_Notify is used (initially over a pre-configured AF session, subsequently over a DS-TT MAC address specific AF session, with a mandatory creation of a new AF session + subscription for further TSN notifications) are all but missing from Stage 2 normative text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.5.1; Annex F.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Delete “Pre-configured AF Session” description” as there is no pre-configured AF Session defined in CT3. The PCF should send the notification based on the pre-cofigured TSN AF address. The pre-configured TSN AF address should be described in TS 23.503, see S2-xxxxxx Besides, move TSN related description from general SM Policy Association procedure to Annex F.1 to avoid duplicated description. Align with the new trigger name (i.e. new 5GS bridge) agreed last meeting. |  |  |  |  |  |  |  |  |

***BEGIN CHANGES***

4.16.5.1	SMF initiated SM Policy Association Modification

The SMF may initiate the SM Policy Association Modification procedure if a Policy Control Request Trigger is met.

Figure 4.16.5.1-1: SMF initiated SM Policy Association Modification

For local breakout roaming, the interaction with HPLMN (e.g. step 2) is not used. In local breakout roaming, the V-PCF interacts with the UDR of the VPLMN.

1.	When a Policy Control Request Trigger condition is met the SMF requests to update (Npcf_SMPolicyControl_Update) the SM Policy Association and provides information on the conditions that have been met.

If the SMF is notified by NRF that the stored PCF instance is not reachable, it should query the NRF for PCF instances within the PCF set and select another instance (see clause 6.3.1.0 of TS 23.501 [2]).

2.	When an AF has subscribed to an event that is met due to the report from the SMF, the PCF reports the event to the AF by invoking the Npcf_PolicyAuthorization_Notify service operation.

When integration with TSN applies (see clause 5.28 in TS 23.501 [2]), the interaction between TSN AF and PCF is described in Annex F.1.

3.	If the PCF determines a change to policy counter status reporting is required, it may alter the subscribed list of policy counters using the Initial, Intermediate or Final Spending Limit Report Retrieval procedures as defined in clause 4.16.8.

4.	The PCF makes a policy decision as described in TS 23.503 [20]. The PCF may determine that updated or new policy information needs to be sent to the SMF.

If the SMF reported accumulated usage for the PDU session in step 1 the PCF deducts the value from the remaining allowed usage for the subscriber, DNN, and S-NSSAI in the UDR by invoking Nudr_DM_Update (SUPI, DNN, S-NSSAI, Policy Data, Remaining allowed Usage data, updated data) service operation.

If the SMF reported accumulated usage for a MK(s) in step 1 the PCF deducts the value from the remaining allowed usage for the MK in the UDR by invoking Nudr_DM_Update (SUPI, DNN, S-NSSAI, Policy Data, Remaining allowed Usage data, updated data (including MK(s))) service operation.

When new PCF instance is selected in step 1, the new PCF should invoke Nbsf_Management_Update service operation to update the binding information in BSF.

5.	The PCF answers with a Npcf_SMPolicyControl_Update response with updated policy information about the PDU Session determined in step 4.

***NEXT CHANGE***

F.1	5GS Bridge information reporting

Figure F.1-1: 5GS Bridge information reporting

Identities of 5GS Bridge and UPF/NW-TT ports are pre-configured on the UPF based on deployment.

1.	PDU Session Establishment as defined clause 4.3.2.2.1-1 is used to establish a PDU Session serving for TSC.

During this procedure, the SMF selects a UPF, which supports functions as defined in clause 5.28.1 of TS 23.501 [2], for the PDU Session.

During this procedure, the SMF receives the UE-DS-TT residence time, DS-TT MAC address for this PDU Session and port management capabilities from the DS-TT/UE in PDU Session Establishment request, and receives the allocated port number for DS-TT Ethernet port, port number(s) for the serving NW-TT Ethernet port(s) and Bridge ID in N4 Session Establishment Response message. The SMF requests the UPF/NW-TT to report the time comparison between TSN GM time and 5G GM time. The UPF allocates the port number for DS-TT and decides port number for NW-TT, Bridge ID after receiving N4 Session Establishment Request message.

2.	The SMF sends the information received in step 1 to the TSN AF via PCF to establish/modify the 5GS Bridge. The TSN AF stores the binding relationship between 5GS Bridge ID, MAC address of the DS-TT Ethernet port, the port number of the DS-TT Ethernet port, port numbers of NW-TT Ethernet ports for the 5GS Bridge and also updates 5GS bridge delay as defined in clause 5.27.5 of TS 23.501 [2] for future configuration.

If the SMF has reported UE-DS-TT Residence Time and/or port numbers of the serving NW-TT Ethernet port(s), then the PCF also provides the UE-DS-TT Residence Time and/or port numbers of the serving NW-TT Ethernet port(s) to the AF, the AF calculates the bridge delay for each port pair, i.e. composed of (DS-TT Ethernet port, NW-TT Ethernet port), using the UE-DS-TT Residence Time for all NW-TT Ethernet ports serving the PDU session.

The AF may provide a Port Management Information Container, MAC address reported for the PDU Session and related port number in response. If the SMF has reported that new 5GS Bridge information is available and no AF session exists for this PDU session yet, then the PCF informs a pre-configured AF using the Npcf_PolicyAuthorization_Notify service operation of 5GS Bridge ID, the port number of the DS-TT Ethernet port, and MAC address of the DS-TT Ethernet port for the PDU Session.

When the AF receives the Npcf_PolicyAuthorization_Notify message from the PCF, the AF shall use the Npcf_PolicyAuthorization service described in clause 5.2.5.3 to request creation of a new AF session specific to the received MAC address of the DS-TT Ethernet port (step 2c). The AF shall then use the Npcf_PolicyAuthorization service to subscribe for notifications for TSN related events over the newly established AF session (step 2d).

3.	If supported according to the port management capabilities received from DS-TT, TSN AF retrieves txPropagationDelay and Traffic Class table from DS-TT. TSN AF also retrieves txPropagationDelay and Traffic Class table from NW-TT.

4.	If DS-TT supports neighbor discovery according to the port management capabilities received from DS-TT, then TSN AF:

-	provides DS-TT port neighbor discovery configuration to DS-TT to configure and activate the LLDP agent in DS-TT;

-	subscribes to receive Neighbor discovery information for each discovered neighbor of DS-TT (see Table 5.28.3.1-1 in TS 23.501 [2].

If DS-TT does not support neighbor discovery, then TSN AF:

-	provides DS-TT port neighbor discovery configuration to NW-TT to configure and activate the LLDP agent in NW-TT to perform neighbor discovery on behalf of DS-TT;

-	subscribes to receive Neighbor discovery information for each discovered neighbor of DS-TT from NW-TT (see Table 5.28.3.1-1 in TS 23.501 [2].

TSN AF:

-	writes NW-TT port neighbor discovery configuration to NW-TT to configure and activate the LLDP agent in NW-TT;

-	subscribes to receive Neighbor discovery information for each discovered neighbor of NW-TT (see Table 5.28.3.1-1 in TS 23.501 [2]).

5.	TSN AF receives notifications from NW-TT on discovered neighbors of DS-TT and NW-TT.

6.	The TSN AF constructs the above received information as 5GS Bridge information and sends them to the CNC to register a new TSN Bridge or update an existing TSN Bridge.

***END OF CHANGES***