3GPP TSG-SA/WG2 Meeting #140e	S2-2004832

# Elbonia, Aug, 2020

Source:	ZTE

Title:	KI#1,Sol#22, solution update

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes update on solution 22

- Discussion

See S2-2004833

2	Proposal

It is proposed to agree the following update on solution 22.

******* start of 1st change ********

6.22	Solution #22: DNS based EAS discovery supporting session breakout

This solution is for Key Issue #1 on DNS based EAS discovery supporting session breakout.

6.22.1	Description

6.22.1.1	Deployment assumption of the solution

Centralized DNS (C-DNS) server is centrally deployed by MNO or 3rd party and responsible for resolving the UE DNS queries into a suitable (Edge) Application Server IP address.

Localized DNS (L-DNS) resolvers/servers may be locally deployed within edge hosting environment, and responsible for resolving the UE DNS queries into a suitable EAS IP address within the Local DN. The L-DNS resolvers/servers may or may not have connectivity with C-DNS server depending on the deployment. The L-DNS resolvers/servers may be deployed by MNO or 3rd parties.

NOTE 1:	The C-DNS server and/or L-DNS resolvers/servers may use an anycast address.

NOTE 2:	The C-DNS server or L-DNS resolvers/servers may contact any other DNS servers for recursive queries, which is out of control of 5GS and not described in the paper. The C-DNS server or L-DNS resolvers/servers can be different from the authoritative DNS server for the target domain name.

Figure 6.22.1.1-1: Target deployment scenario of the solution

The solution requires a new Functionality, an enhanced DNS Forwarder here referred to as "LDNSR". LDNSR supports Edge AS Discovery using DNS using knowledge of the 5GC connectivity of the UE. The LDNSR interacts with the 5GC as Application Function.

NOTE 3:	If the LDNSR is separately deployed, it is located before the N6 NAT, if any.

6.22.1.2	PDU session establishment

When the UE sets up a PDU session a PSA is allocated: this PSA may be a central PSA.

ULCL/local PSA insertion can be triggered by user traffic ("dynamic ULCL/Local PSA insertion") or by other means before user traffic ("pre-established ULCL/Local PSA insertion"):

-	Pre-established ULCL/Local PSA insertion can be done before the UE sends out actual DNS queries or traffic to be locally routed, e.g. at PDU Session establishment or be triggered by AF influence on routing related PCC rules. SMF sets up filters in ULCL for steering to Local PSA of both the edge application traffic and the DNS messages towards L-DNS.

When the L-DNS resolvers/servers does not have connectivity with C-DNS servers, specific mechanisms need to nevertheless support DNS translation for domains not corresponding to the edge resources (DNS requests not targeting the edge application domain). Alternative options are further described in clause 6.22.2.

NOTE 1:	Encrypted DNS traffic requires L3/L4 filters in ULCL (e.g. DNS Query destination IP address is used) for the steering:. Same filters can be used at all locations if L-DNS uses Anycast IP address.

-	Dynamic ULCL/Local PSA insertion is triggered by the DNS messages for EAS FQDN resolution. It requires LDNSR involvement and different modes are possible. LDNSR behaviour is described below.

NOTE 2:	The pre-established option can be combined with options using LDNSR if it is not wanted that all DNS traffic is handled in the L-DNS in deployment.

6.22.1.3	DNS resolution with Pre-established UL CL/L-PSA and L-DNS

5G core functions (PCF, UDR) can be configured by AF traffic influence routing with the list of DNAIs and IP addresses where the edge application servers are deployed and the traffic filters corresponding to the service to be steered. These apply to all services to be steered including DNS messages. The traffic filters may include the IP address or FQDN of the Local DN deployed service, L-DNS IP address.

Traffic can be steered based on ULCL filters for all DNS messages (DoH, DoT, Do53), and traffic to applications (to EAS).

In the configuration phase, traffic filters for each Local DN deployed service and the DNAI are configured using AF traffic influenced routes. PCF stores the list of traffic filters for each DNAI. This uses 3GPP Rel-16 mechanisms

During PDU session establishment, traffic filters are installed in ULCL using 3GPP Rel-16 mechanisms.

The UE may be configured with DNS resolver IP address per UE interface(e.g. per (DNN, S-NSSAI) using PCO, or use application specific DNS configurations (DoH, VPN), This may further be refined by other solutions.

The pre-established traffic filters are used to steer DNS requests to the closest Local DN or DNAI for EAS discovery. The L-DNS may add an edns-client-subnet (ECS) EDNS0 option before using recursive requests to reach an authoritative DNS server for the target domain.

6.22.1.4	DNS resolution before and after Dynamic UL CL/L-PSA insertion

The UE is configured with a DNS server during PDU session establishment by SMF via PCO; this may further be refined by other solutions. The DNS server can be C-DNS server or can be the LDNSR. 5G core functions has not been configured by AF traffic influence routing and the ULCL/L-PSA is dynamically inserted during DNS query procedure.

Editor's note:	It's FFS whether both above options of DNS configuration on UE need to be supported.

If the C-DNS server address is configured on the UE, the network deployment should ensure the DNS requests from the UE go via a LDNSR.

LDNSR is assumed to have visibility of the DNS parameters.

The EAS discovery can happen in two phases:

-	Phase 1: Before Dynamic UL CL/L-PSA insertion:

Option 1: LDNSR receives UL DNS query and inserts an ECS option into the DNS query and sends to C-DNS. The ECS option may include an IP address/prefix which is related with the user location and requested FQDN.

Option 2a: LDNSR receives UL DNS query, and it forwards the UL DNS Query to a L-DNS. The L-DNS address is related with the user location and requested FQDN.

Option 2b: LDNSR receives UL DNS query determines the L-DNS address but cannot send traffic to the L-DNS directly. This is further described in clause 6.22.2

For above options, after receiving the DNS response, the  LDNSR sends AF request to PCF to influence traffic routing towards Local DN. The PCF authorizes the AF request and triggers the SMF to insert UL CL/L-PSA towards the DNAI. . In order to support different scenarios, the LDNSR may determines the target DNAI based on the EAS IP address in DNS response and send the target DNAI, the EAS IP address in DNS response and/or FQDN to the PCF or to SMF directly when the LDNSR has received the SMF address during previous DNS query procedure..

As an optimization and if DNS request is sent in clear, SMF may optionally configure UL CL to local route some subsequent DNS queries based on FQDNs supported by the Local DN/DNAI. This corresponds to option 3 described as part of phase 2 below.

-	Phase 2: After Dynamic UL CL/L-PSA is inserted:

Unless configured to be locally routed (see option 3), LDNSR receives subsequent DNS Queries.

For queries related to that same FQDN from same UE, LDNSR can either redirect the query to L-DNS or follow the procedure as described in Options 1&2 above (e.g. when previous decisions should be reconsidered).

Editor's note:	the text above assumes that the LDNSR keeps a context related with the PDU Session of the UE where it keeps track of the DNS requests sent by a UE in a PDU Session. It is FFS how this can be done when the LDNSR is not naturally associated with the PDU Session (in case the LDNSR would not be located in SMF or UPF). Even if the DNS queries are from same source IP, it may still be possible that the IP address has been reallocated to another UE.

For requests related to new FQDNs, the process is as described in Phase 1. The information of the already inserted UL CL/ Local PSA can also be considered in Option 1 & 2 above.

When LDNSR is configured to apply redirect for an FQDN, ULCL has also been configured by SMF to locally divert to Local PSA the DNS Queries to L-DNS (e.g. on Destination IP address).

Option 3: If in Phase 1 SMF configured a FQDN query can be local routed on the UL CL, then the subsequent DNS queries for the FQDN will be locally routed to the Local DNS Resolver. The Local DNS Resolver will:

a)	Either send the DNS query to a L-DNS Server(this can be done by various way listed in sol 4 and solx [intel sol]), so that the L-DNS Server can add ECS option based on its own configuration (sol 4/ sol x [intel sol]) and do recursive DNS resolution if the L-DNS Server cannot resolve the EAS IP address.

b)	Or forward the query to C-DNS server, which assume that the deployment should ensure DNS query packet has topologically correct UE source address when DNS resolver gets it(sol 5: NAT at UPF-PSA rewrites the UE address to an address referring to the L-PSA (and the reverse on the downstream packet)).

NOTE:	Options 3 assumes that either ULCL steering is based on L3/L4 information or ULCL has visibility of the DNS traffic.

The above options for the EAS discovery using LDNSR can be described in Figure 6.22.1.4-1.

Figure 6.22.1.4-1: Options for the EAS discovery using LDNSR for PDU session breakout

6.22.2	Procedures

L-DNS server/resolver

Figure 6.22.2-1

The AF provides EAS deployment information to UDR via Nnef_TrafficInfluence, This includes FQDNs, IP addresses/prefixes, Local DNS resolver/server IP address per DNAI. The SMF gets the information from PCF (PCC rule) during PDU session establishment. During PDU Session Establishment procedure, the address of C-DNS server or LDNSR is provided by SMF via PCO to UE. Steps 1 - 6 are processed for each DNS request.

For the "pre-established" ULCL/L-PSA scenario, the SMF performs UL CL/L-PSA selection and insertion based on UE location and sets up uplink filter rule on UL CL. Steps 1, 7 are processed for each DNS request.

1.	The UE sends a DNS query including the requested FQDN.

2.	If an ULCL exists and the DNS query matches the uplink filter rule on UL CL, then step 3 to 6 are skipped. Otherwise the PSA1 UPF forwards the received DNS query to the LDNSR.

Editor's note:	This forwarding depends on the SA WG2 decision on how to support LDNSR (in SMF, in UPF, as a stand-alone entity)

3.	The LDNSR contacts SMF and provides FQDN and then:

Option 1: the LDNSR retrieves IP address corresponding to the DNAI selected by SMF based on UE location and requested FQDN. or

Option 2a (with direct connectivity between LDNSR and L-DNS): the LDNSR  retrieves UE location and  the address of L-DNS server serving the DNAI selected by the SMF.

Option 2b (without direct connectivity between LDNSR and L-DNS): the LDNSR requests the SMF to forward the DNS query via the user plane (after step 5). The SMF determines the DNAI and L-DNS address based on the UE location and requested FQDN.

Editor's note:	Whether and how SMF indicates LDNSR whether to contact SMF based on e.g. UE location is FFS.

NOTE 1:	If the LDNSR is integrated into the UPF, the UPF provides the above parameters without UE's IP address.

4a-4b:	Option 1: The LDNSR adds the IPv4 subnet or address or IPv6 prefix retrieved from SMF as ECS option as specified in RFC 7871 and sends it to C-DNS server. The C-DNS returns the DNS response including EAS IP address.

4c-4d:	Option 2a (with direct connectivity between LDNSR and L-DNS): the LDNSR sends DNS query to the L-DNS server and get the DNS response including the EAS IP address. After receiving the DNS response, the LDNSR provides SMF with IP address of EAS.

For both 4a-4b and 4c-4d, After receiving the DNS response, the LDNSR provides SMF with IP address of EAS and FQDN. Then, SMF decides whether to trigger the insertion of the ULCL/Local PSA .

Editor's note:	The conditions for LDNSR to send EAS IP address after receiving DNS response to SMF is FFS.

NOTE 2:	If the LDNSR is integrated into the SMF, the above interaction with SMF can be omitted.

5.	The SMF decides DNAI and performs UL CL/L-PSA selection and set up. The DNAI and UL CL/Local PSA2 is selected based on the information provided by LDNSR to ensure the selected local PSA and EAS are corresponding to same DNAI.

Option 2b: If the LDNSR requested the SMF to forward the DNS query in step 3, the SMF sends the DNS query to the address of the local DNS server via the Local PSA2. The SMF configures the Local PSA2 to forward the DNS response to the SMF. When receiving the DNS response, the SMF forwards it to the LDNSR. After receiving the DNS response, the LDNSR provides the SMF with IP address of EAS. The SMF may perform update of the ULCL/L-PSA based on this information. When receiving subsequent DNS queries, the LDNSR may provide the SMF with the new IP address of EAS and the SMF may perform update of the ULCL/L-PSA based on this information.

Optionally, the SMF may configure the L-DNS address in ULCL to be diverted to L-PSA. In order to redirect subsequent DNS queries directly to the L-DNS, the LDNSR acts as an iterative DNS server and redirects the DNS Query to the L-DNS selected for the UE Location by responding with a referral to the UE. This triggers the UE to send a new DNS query towards the L-DNS.

As an optimization, the LDNSR may redirect subsequent DNS queries for the FQDN to the L-DNS. For that, the traffic to that L-DNS needs to be configured in ULCL to be diverted to L-PSA.

As an optimization, the SMF may configure uplink forwarding rules on UL CL to route the traffic and subsequent DNS queries for FQDNs corresponding to the DNAI to the Local PSA2.

6.	The LDNSR sends DNS response to UE via PSA1.

7.	Option 3a and "pre-established" case: Based on the uplink forwarding rules, if an ULCL exists and the query matches the uplink filter rule on UL CL, the UL CL route the DNS query to local PSA2, and then local N6 routing forwards to Local DN. If no filter is present in ULCL, the DNS request is forwarded to the C-DNS.

The DNS query may be routed to a local DNS resolver which then resolves the DNS Query or forwards it for resolution. It may derive an ECS option (using locally configured value or the source address/prefix of the received DNS request), then send the DNS query to A-DNS server using the ECS option.

Alternatively, the DNS query may be routed to C-DNS server via Local N6 or DN. The deployment should ensure the DNS query packet has topologically correct UE source address when DNS resolver gets it, e.g. via NAT in Local DN.

The local DNS resolver or C-DNS server sends DNS response to the UE by retracing the forward path.

Option 3b:

a.	The local DNS resolver modifies the packet's destination IP address (corresponding to a default DNS server) to that of the L-DNS and stores the original IP address (Default-DNS-IP) and the packet's source IP address (corresponding to UE's IP address) to its own (i.e. the local DNS resolver's) IP address and stores the original source IP address (UE-IP) for later processing.

b.	The local DNS resolver then forwards the modified DNS request to the L-DNS and processes as follows:

-	If the L-DNS can resolve the IP address for the requested FQDN of EAS, it responds to the local DNS resolver with the desired IP address of the local EAS.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS and the L-DNS is not connected to the C-DNS, it responds to the local DNS resolver with DNS Response without including the IP address of EAS or indicating that it could not resolve the FQDN.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS but it is connected to a C-DNS, it communicates with the C-DNS to recursively resolve the EAS IP address.

c.	The local DNS resolver receives a DNS Response including empty value:

-	The local DNS resolver forwards the original DNS request (the one that had originally arrived from the UL CL/BP UPF) back to the UL CL/BP UPF.

-	The UL CL/BP UPF forwards the original DNS Request from local DNS resolver to the remote PSA UPF via N9.

NOTE 3:	Option 1 and 2 can be still applicable in Phase 2 if the requested FQDN is not configured to be local routed on the UL CL or the DNS traffic is encrypted.

6.22.3	Impacts on services, entities and interfaces

Editor's note:	This clause will be finalized when the EN of the previous clauses are closed.

******* end of 1nd change ********