3GPP TSG-WG SA2 Meeting #141E	S2-2007390

Elbonia, October 12 – 23, 2020	(revision of S2-20xxxx)

Source:	Qualcomm Incorporated

Title:	Evaluation and Conclusion for KI#12: NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes the conclusion of KI#12.

# 1. Introduction

This contribution proposes evaluation and conclusion on KI#12: NWDAF-assisted RFSP policy.

# 2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.12	Key issue #12: NWDAF-assisted RFSP policy

According to the table 6.0-1, solution #4, #40, #42, #43 and #44 are proposed for key issue #12 and Solution #43 is merged into solution #4.

Solution #4 proposes to enhance the Observed Service Experience analytic feature, and also proposed a new analytics ID Service in use analytics, PCF will combine the ouput of Observed Service Experience and the activated application information to determine the suitable RFSP policy.

Solution #40 proposes to reuse output of the User Data Congestion Analytics by the PCF to decide the RFSP index.

Solution #41 proposes to define a new analytics ID for “Radio/Frequency usage” to collect the QoS information for the specific S-NSSAI. The output of the analytics includes the number of user per frequency/ RAT and data volume on RAN, but it is not clear how does NWDAF determin these analytics results based on the input information which does not include any per RAT/frequence related information.

Solution #42 proposes to collect the subscribed RFSP index and the authorized RFSP index to the NWDAF for analytics. PCF has already received the authorized RFSP index from AMF, it is not clear what is the output of the analytics and what is the benefit for the PCF to use the output.

According to the above analytics, it is proposed to use solution#4 and solution #40 in the normative work.

# 8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

## 8.12	Key issue #12: NWDAF-assisted RFSP policy

It is concluded to base normative work on Key issue #12 in solution#4 and solution #40.

* * * * End of Changes * * * *