# SA WG2 Meeting #S2-141E	S2-2006978r01

# 12 - 23 October, 2020, Electronic, Elbonia

Title:	[Draft] LS on PC5 DRX operation

Release:	Rel-17

Work Item:	FS_eV2XARC_Ph2

Source:	SA2

To:	RAN2

Cc:	RAN1

Contact Person:

Name:	LaeYoung Kim

Tel. Number:

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 is progressing FS_eV2XARC_Ph2 (Study on architecture enhancements for 3GPP support of advanced V2X services – Phase 2) in TR 23.776 (The latest version of TR 23.776 can be found in https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/Latest_SA2_Specs/Latest_draft_S2_Specs).

SA2 would like to ask RAN2 the following questions as follows:.

Q1) For anycast type, whether the existing PC5 QoS parameters is sufficient determine the PC5 DRX parameters in RAN? Any additional information is required?

Q2) Once the AS determines the PC5 QoS parameters in RAN, whether the AS layer can provide the PC5 DRX related information to V2X layer, e.g., the DRX cycle, ON duration and starting point of the ON duration?

Q3) Whether aligning the PC5 DRX and the Uu DRX is needed?

2. Action:

To RAN2 group:

ACTION: 	SA2 kindly asks RAN2 to feedback the answer above.

3. Dates of Next TSG-SA2 Meetings:

3GPP TSG SA2#142E			16 – 20 November 2020	eMeeting