3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007617

Elbonia, October 12 – 23, 2020

Source:	Orange

Title:	KI #12: Evaluation and conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and conclusions on KI#12 “NWDAF-assisted RFSP policy”.

# Discussion

This contribution proposes conclusions for Key issue #12 “NWDAF-assisted RFSP policy”.

This contribution identifies the themes proposed in the various solutions, and assesses the impacts of each solution on the existing 5G CN.

| #Solution | Description |
| --- | --- |
| #4: Service Behaviour analytics and Service Experience analytics for NWDAF-assisted RFSP policy | The PCF determines RFSP value based on category of subscriber (e.g. gold), Observed Service Experience (extended per RAT type and frequency), and new analytics on “application in use”. It is compatible with solution #43. |
| #40: User Data Congestion Analytics for NWDAF-assisted RFSP policy | The PCF determines RFSP value based on the existing analytics “User Data Congestion” with new analytics filter S-NSSAI |
| #41: RAT/Frequency usage analytics | The PCF determines RFSP value based on new analytics on RAT and frequency usage (number or users and data volume), with new analytics filter RAT type combined with S-NSSAI and Area of Interest. |
| #42: NWDAF-assisted RFSP Policy Configuration | The PCF determines RFSP value based on the following input data: subscribed RFSP index, session information, optional RAN information. |
| #43: RFSP index value using Analytics Id Service Experience | The PCF determines RFSP value based on Observed Service Experience, extended to retrieve OAM input data per RAT/Frequency. |

# 2.	Proposal

It is proposed to apply the following changes to the TR 23.700-91.

* * * * Start of changes * * * *

## 7.12	Key issue #12: NWDAF-assisted RFSP policy

The Key Issue # 12 aims to define various mechanisms to be implemented by NWDAF and NFs to optimize on RAT and frequency selection: relevant data collection for the NWDAF from various NFs, NWDAF outputs to consumer NFs, and usage of such analytics for relevant action by the consumer NFs.

The 5 different solutions use or improve existing analytics, or create new analytics, based on the following input data:

A)	Use of RAT/frequency related information (solutions 4, 41, 42, 43)

B)	Use of session information, more or less detailed by type of application (solutions 4, 42)

C)	Use of existing analytics on User Data Congestion (solutions 40)

D)	Use of Observed Service experience (solutions 4, 43)

Table 7.12-1: Classification of solutions for KI#12

| #Solution | Description | Analytics IDs | Combination |
| --- | --- | --- | --- |
| #4: Service Behaviour analytics and Service Experience analytics for NWDAF-assisted RFSP policy | The PCF determines RFSP value based on category of subscriber (e.g. gold), Observed Service Experience (extended per RAT type and frequency), and new analytics on “application in use”. It is compatible with solution #43. | Analytics OSE extended per RAT type/frequency + new analytics “Application in use” | A+D (+B) |
| #40: User Data Congestion Analytics for NWDAF-assisted RFSP policy | The PCF determines RFSP value based on the existing analytics “User Data Congestion” with analytics filter S-NSSAI. | User Data Congestion | C |
| #41: RAT/Frequency usage analytics | The PCF determines RFSP value based on new analytics on RAT and frequency usage (number or users and data volume), with new analytics filter RAT type combined with S-NSSAI and Area of Interest. | New analytics “RAT/Frequency usage” | A |
| #42: NWDAF-assisted RFSP Policy Configuration | The PCF determines RFSP value based on the following input data: subscribed RFSP index, session information, optional RAN information. | New analytics based on RAN information; session information | A+B |
| #43: RFSP index value using Analytics Id Service Experience | The PCF determines RFSP value based on Observed Service Experience, extended to retrieve OAM input data per RAT/Frequency. | Analytics OSE extended per RAT type/frequency | A+D |

It appears that several solutions identify the need to provide analytics on RAT/Frequency usage, as a necessary basis of PCF decisions.

Moreover, other complementary criteria are proposed to be taken into account such as session information (possibly per application), Observed Service Experience, or User Data Congestion.

* * * * Next changes * * * *

## 8.12	Key issue #12: NWDAF-assisted RFSP policy

The specification of new NWDAF analytics on the use of the RAT / frequency is recommended for normative work. For the sake of keeping modularity, these new analytics dedicated to the use of RAT/frequency should be specified separately from any other analytics. Therefore, it is proposed to take solution 41 as baseline for normative work.

* * * * End of changes * * * *