SA WG2 Meeting #141e	S2-2007767r01

# Oct 12th – 23rd, 2020 ; Elbonia                   		 	(revision of S2-200)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	update to Solution 6.2.3.1

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: update to Solution 6.2.3.1

# 1 Discussion

# 2 Proposal

It is proposed to update TR 23.748 as follows

FIRST CHANGE (all text is new)

#### 6.2.3.2	Procedure for provisioning local DNS Server Address in the case of ULCL/BP

Figure 6.2.3.1-1 illustrates the procedure for configuring the DNS server address to the UE, in the case of ULCL /BP is used for the PDU session. The edge DN should have connectivity with central DN because the edge and central DN are the same DN, as specified in TS 23.501 [2] clause 5.6.4.

Figure 6.2.3.2-1: Procedure for Local DNS address provisioning to UE in the case of ULCL or BP

1.	During UE mobility, the UE performs Service Request or Registration procedure if CM-IDLE. If UE is CM-Connected and mobility occurs, Handover procedure is performed.

2.	The AMF invokes Nsmf_PDUSession_UpdateSMContext as specified in TS 23.502 [3].

3.	Based on the UE location information received from the step 2 and the mapping table between local DNS and its serving areas, the SMF performs Addition of additional PDU Session Anchor and Branching Point or ULCL procedure, or performs Change of additional PDU Session Anchor for IPv6 multi-homing or ULCL, or Simultaneous change of ULCL or Branching Point and additional PSA. Above procedures follows as specified in clause 4.3.5 in TS 23.502 [3]

In this step, SMF may determines to update the address of the DNS server handling UE requests based on the UE location.

For ULCL case, SMF configures the ULCL UPF routing rules during this procedure. The traffic routing rules include filtering out the DNS request packet by examining the destination IP address as the local DNS address configured on the UE, or the port number is 53, and offloading DNS request to edge Data Network.

4.	The local DNS address is provided to the UE via the ePCO in the PDU Session Modification Command. The UE updates the DNS server address accordingly.

For IPv6 multi-homing, the SMF can also provide DNS address to UE through RA message; in case of conflict with DNS addressing received over ePCO, the DNS address received from ePCO has a higher priority than the DNS address received from RA/RS message.

NOTE 1:	The local DNS can work as the authoritative DNS for the applications using EC service . The local DNS has connection with the authoritative DNS and can work as DNS Resolver for applications non-using EC service.

NOTE 2: 	The per FQDN control of the DNS resolution is done by the local DNS whereas SMF controls the usage of the EC services per application through the provisioning of the traffic routing rules in UL CL / BP. Both configurations need to be aligned so that users enjoy certain Application at the edge: the SMF configures the PDU Session ULCL /BP (and UE related rules for BP) Traffic steering rules whereas, it is the Local DNS Resolver  that controls whether the DNS Resolution for the Application FQDN in a PDU Session is resolved into a local EAS.

Editor’s Note: How the Local DNS applies subscriber differentiated DNS resolution control is FFS . It is FFS whether the local DNS can act as a LDNSR.

NEXT CHANGE (2)

NEXT CHANGE (3)