3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008746

Elbonia, November 16 – 20, 2020, 2020 	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Update the conclusion for KI#2

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	ATSSS / Rel-17

Abstract: This paper updates the Interim Conclusions for KI#2.

- Introduction/Discussion

This paper updates the Interim Conclusions for KI#2 based on the Evaluation of MP-QUIC based solution in S2-2008744 (KI#2 Solution #7 - Solution update), and S2-2008745 (Evaluation on KI#2 for Ethernet traffic). The following two ENs are removed accordingly.

Editor’s Note: It is FFS how many MP-QUIC Connections are established between UE and UPF, e.g. in relation to traffic flows, QoS Flows and steering modes.

Editor's Note: Whether the QUIC-based steering functionality can be applied to Ethernet MA PDU Sessions is FFS.

2. Text Proposal

It is proposed to adopt the following changes in the TR 23.700-93.

# * * * * First change * * * *

8.2	Conclusions for KI#2: Additional Steering Functionalities

The ATSSS steering functionality defined in Rel-17 shall be based on "Multipath QUIC" and shall apply the following principles:

1.	A QUIC-based steering functionality shall be specified, which shall use the QUIC protocol defined by IETF in draft-ietf-quic-transport [6], draft-ietf-quic-recovery [7], draft-ietf-quic-tls [18], draft-ietf-quic-invariants [25] and the following extensions:

(a)	the unreliable datagram extension, specified in draft-ietf-quic-datagram [8]; and

(b)	the multipath extension, which will be specified by IETF (e.g. possibly based on the draft-deconinck-multipath-quic [10]).

Editor's note:	It is FFS whether the QUIC-based steering functionality will apply other IETF protocols, such as the MASQUE protocol defined in draft-ietf-masque-connect-udp [20] and the tunnel protocol defined in draft-piraux-quic-tunnel [9].

2.	The QUIC-based steering functionality shall be optional for the UE and the network. The UE shall indicate whether it supports QUIC-based steering functionality during the MA PDU Session establishment procedure.

3.	The QUIC-based steering functionality may be used in parallel with ATSSS-LL and MPTCP functionalities for a MA PDU Session.

4.	The QUIC-based steering functionality shall be able to support traffic splitting per-packet and to measure the RTT and the packet loss rate per QoS flow by using the statistics exposed by the QUIC protocol. There is no need for the QUIC-based steering functionality to use the PMF protocol.

5.	The QUIC-based steering functionality shall be able to use the QUIC protocol to determine when an access becomes unavailable / available (e.g. it may use the QUIC liveness testing or a similar mechanism).

NOTE 1:	Although the PMF protocol is not required by the QUIC-based steering functionality, the PMF protocol could be required when the ATSSS-LL functionality is also applied. If the QUIC-based steering functionality is applied in parallel with ATSSS-LL, then it is possible to determine the access availability by using the PMF protocol.

6.	When the network decides to activate the QUIC-based steering functionality for an MA PDU Session, the network shall provide to UE:

a.	two IP addresses/prefixes called "link-specific multipath QUIC" addresses (one for each access); and

b.	the UPF IP address(es) and port(s), where the QUIC connections should be setup to.

7.	The QUIC-based steering functionality should introduce minimal overhead.

8.  The MP-QUIC Connection is established between the UE and the UPF per PDU session. This MP-QUIC Connection can be shared by all the IP flows to be applied with the QUIC-based steering functionality.

9.  The QUIC-based steering functionality is not applicable to Ethernet MA PDU session.

Editor's note:	It is FFS how the overhead can be minimized. For example, additional headers could be removed at the transmitter and re-inserted at the receiver or they could be compressed.

Editor's note:	Further details of the QUIC-based steering functionality are FFS and whether it can be supported in Rel-17 depends on IETF progress on the QUIC multipath extensions.

Editor's note:	It is FFS whether and how double encryption of data packets can be avoided.

# * * * * End of changes * * * *