SA WG2 Meeting #143E e-meeting	S2-2100858

Elbonia, February 24 – March 09, 2021

Source:	vivo, Nokia, Nokia Shanghai Bell, Lenovo, Motorola Mobility

Title:	Introducing UE behaviour for Edge Computing in SSC mode 2 and SSC mode 3

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution introduces a new UE behaviour for Edge Computing in SSC mode 2 and SSC mode 3

1	Introduction

As per TR 23.748 clause 9.2.2 " Conclusions for Key Issue #2: UE based EAS rediscovery ", it introduces new UE behaviour to support edge computing characteristic. The UE can remove the old EAS information associated with the released PDU Session and can reselect a new EAS after the UE receives a new IP address for SSC mode 2 and SSC mode 3.

This new UE behaviour in edge relocation should be reflected according to the summarised conclusions in TR 23.748 clause 9.2.2.

2	Proposal

It is proposed to adopt the following changes into TS 23.548.

********** Start Changes(All new text) **********

6.2.2.3	UE based EAS rediscovery for SSC mode 2/3

The following procedure is triggered by SMF in order to change the PDU Session Anchor serving a PDU Session of SSC mode 2/3 for a UE.

During this procedure, for SSC mode 2/3, the UE may apply following behaviour: when the UE detects the PDU Session release (IP address change), the UE can remove the old DNS cache related to released PDU session, for example, the old Edge Application Server address information. And UE can reselect a new EAS after the UE receives a new IP address.

Figure 6.2.2.3-1: UE Behaviour in Procedure of Changing of SSC mode 2/3 PSA for a PDU Session

1.	For SSC mode 2 and SSC mode 3, one of following event in 1a or 1b applies:

1a: The same procedure as the step 1-2 defined in clause 4.3.5.1 of TS 23.502 [3] that changing of SSC mode 2 PDU Session Anchor with different PDU Sessions

1b: The same procedure as the step 1-4 defined in clause 4.3.5.2 of TS 23.502 [3] that changing of SSC mode 3 PDU Session Anchor with multiple PDU Sessions

2.	When the UE detects the PDU Session release (immediate in SSC mode 2 or future in SSC mode 3), the UE can remove the old DNS cache related to old PDU session, for example, Edge Application Server address information. And UE can reselect a new EAS after the UE receives a new IP address.

3.	According to the event in step 1, one of following event applies:

3a: The same procedure as the step 3 defined in clause 4.3.5.1 of TS 23.502 [3].

3b: The same procedure as the step 5-6 defined in clause 4.3.5.2 of TS 23.502 [3].

*************** End Changes ***************