3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2101435

Elbonia, February 24 – March 09, 2021	(revision of S2-2100930)

Source:	Huawei, HiSilicon

Title:	New TS: On the MBS Session join and establishment of 5MBS

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds MBS Session join and establishment to the new TS

- Background and Introduction

In the skeleton of the new TS, there is a section for "MBS Session join and establishment".

- Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change* * * *

7	MBS procedures

7.2	MBS procedures for multicast Session

7.2.1	MBS join and Session establishment procedure

7.2.1.1	General

Session Join procedure is used by UEs to inform the 5GC of the UE interest in an MBS Session.. The user plane management is described in clause 6.6.

Editor’s notes: Whether UE join triggers MBS Session Establishment is FFS

7.2.1.2	MBS join and Session establishment procedure

The following steps are executed before the UE requests to join the MBS session:

- The MBS Session has been configured.

- The UE registers in the PLMN and establishes a PDU session.

- The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via announcement.

Figure 7.2.1.2-1: PDU Session modification for multicast

1.	To join the multicast group, the UE sends the PDU Session Modification Request (MBS Session ID). MBS Session ID indicates the multicast group that UE wants to join.

2.	Per the received MBS Session ID, the SMF recognize this is MBS Session join request. The SMF authorizes MBS Session join request, see clause 6.1.1.

3.	If SMF has no information about the multicast context for the indicated MBS Session, SMF checks at the NRF whether a multicast context for the indicated MBS Session exists in the system, by using Nnrf_NFDiscovery request (MBS Session ID). If a multicast context already exists in the NRF, the NRF responses with Nnrf_NFDiscovery response (MB-SMF ID).

Editor's note: Whether SMF acting as the MB-SMF is needed and how it works if needed is FFS.

Editor's note: More consideration on how to prevent denial of service attack type situation when first UE joining the multicast group, triggers the MB-UPF to join the multicast tree towards the content provider is FFS.

4.	By using Nsmf_MBSSession_Create request (MBS Session ID), SMF interacts with MB SMF to retrieve multicast QoS flow information of the indicated MBS session.

5.	SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response(N2 SM information (PDU Session ID, MBS Session ID, MB-SMF ID, multicast QoS flow information, updated PDU Session information, mapping between unicast QoS flow and multicast QoS flow information), N1 SM container (PDU Session Modification Command) to:

-	create a MBS session context for the indicated MBS session in the RAN, if it does not exist already; and

-	inform about the relation including the mapping information between the multicast context and the UE's PDU session to RAN.

Based on operator policy, the SMF may prepare for individual delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session’s QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

Editor's note: Details information included in N2 SM information will be aligned with RAN3.

Editor's note: Whether it needs to inform about the relation including the mapping information between the multicast context and the UE's PDU session to UE is FFS.

6.	The N2 message, which includes the PDU session modification command information is sent to the RAN.

If the MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise, 5GC shared MBS traffic delivery is adopted.

Editor's note: How the NG-RAN’s 5MBS capability is made known is FFS.

The NG-RAN uses the MBS Session ID to determine that the PDU Session Modification procedures corresponds to the indicated multicast session.

If the multicast QoS information is received, the associated unicast QoS flow information is not used to allocate the radio resource.

NOTE x: It is NG-RAN that decides whether radio resource is allocated or not.

When the NG-RAN receives an MBS Session ID but MBS Session context does not exist for that MBS Session ID, the NG-RAN use the included MBS Session QoS information to allocate resources to serve this multicast session. Otherwise the indicated MBS Session has been established before. The NG-RAN can use those allocated resource for MBS Session data packet transferring to UE.

7.	The RAN, AMF, SMF, and MB-SMF performs resources reservation for individual delivery and shared delivery.  The SMF obtains the 5MBS capability of target NG-RAN via the accepted QFI information and determines the delivery mode between 5GC and RAN.

Editor's note: Details of establishing the tunnel of 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery is FFS.

7.2.1.3	MBS join and Session establishment procedure involving MBSF

Editor's note: Details are FFS.

# * * * * End of changes * * * *