SA WG2 Meeting #143-E	S2-2100xxx

Elbonia, 24 Feb – 09 Mar 2021

Source:	Apple, Samsung, ZTE, vivo, Huawei?

Title:	TS 23.304: Introduction of Functional Entity- ProSe Application Server

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This contribution presents the text proposal for introducing ProSe Application Server definition in 3GPP TS 23.304.

# 1. Background

According to the conclusions for KI#1 (ProSe Direct Discovery), KI#5 (Support direct communication path selection between PC5 and Uu) and KI#8 (Support of PC5 Service Authorization and Policy/Parameter Provisioning), the ProSe Application Server is a functional entity in the 5G ProSe Architecture.

This pCR presents the text proposal for ProSe Application Server Functional entity definition.

# 2. Text Proposal

It is proposed to update the following text in the TS 23.304.

## *** First change ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ProSe	Proximity based Services

RPAUID	Restricted ProSe Application User ID

PDUID	ProSe Discovery UE ID

## *** Next change ***

### 4.3.8	ProSe Application Server

The ProSe Application Server supports the following functionalities.

For Direct Discovery,

-	Storage of ProSe Discovery UE ID and metadata;

-	Maintains permission information for the restricted ProSe Direct Discovery using RPAUIDs;

-	Mapping of RPAUID and PDUID for restricted ProSe Direct Discovery;

-	Interaction with 5G DDNMF for ProSe Direct Discovery, including

-	Allocation of the ProSe Restricted Code Suffix pool, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Restricted Code Suffix, if restricted Direct Discovery with application-controlled extension is used;

-	Provision the 5GC and the UE with parameters for ProSe Direct Discovery over PC5;

For ProSe Direct Communication,

- Provisioning parameters for Direct Communication (e.g. path selection preference for ProSe services between Uu and PC5 to UDR) to 5GC and UE.

For ProSe Group Discovery and Communication,

- Provisioning parameters for Group Member Discovery to UE;

- Provisioning information for Group management.

## *** End of the change ***