3GPP TSG-SA WG2 Meeting #143-e	S2-210wxyz

- Electronic meeting, 24 February - 9 March 2021

Source:	OPPO

Title:	Mobility support of 5MBS

Spec:	3GPP TS 23.xxx

Agenda item:	8.9

Document for:	Approval

Abstract of the contribution:  This PCR is to introduce the description for the mobility support of 5MBS.

1. Reason for Change

The PCR is based on the KI#7 conclusion in the TR 23.757 and to introduce the general description for the mobility support of 5MBS.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.xxx.

* * * * First Change * * * *

## 6.X	Mobility support of MBS service

The following scenarios are supported for the mobility of MBS service:

-	the UE moves from a NG-RAN node that supports 5MBS to a target NG-RAN node that supports 5MBS;

-	the UE moves from a NG-RAN node that supports 5MBS to a target NG-RAN node that does not support 5MBS; and

-	the UE moves from a NG-RAN node that does not support 5MBS to a target NG-RAN node that supports 5MBS.

The minimization of data loss may be supported during the handover from a NG-RAN that supports 5MBS to a target NG-RAN node that supports 5MBS.

Editor's note:	The details e.g. data forwarding will be checked after the RAN WGs January meeting.

To support the handover from a NG-RAN node that supports 5MBS to a target NG-RAN node that does not support 5MBS:

-	the mapping information between the unicast QoS flow and the multicast QoS flow is provided to the NG-RAN and the UE during the PDU session establishment procedure or the PDU session modification procedure for the PDU session associated with the MBS session;

-	the SMF shall activate the N3 tunnel of the PDU session associated with the MBS session after the handover procedure; and

-	the SMF and the MB-SMF shall also activate the N9 tunnel between the UPF and the MB-UPF for 5GC Individual MBS traffic delivery method after handover procedure.

To support the handover from a NG-RAN node that does not support 5MBS to a target NG-RAN node that supports 5MBS:

-	the 5GC shall switch to the 5GC shared MBS traffic delivery method after handover procedure.

* * * * End of Changes * * * *