3GPP TSG-SA2 Meeting # 148E	 S2-2108386r01

# Elbonia,  November 15 - 22, 2021												 (Revision of S2-2107164)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3135 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF+PGW-C assigned PDU Session ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Cisco |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CT3 LS S2-2107031/C3-214527 discusses the following scenario regarding SMF+PGW-C assigned PDU Session ID”: - UE firstly establishes a PDN connection via MME and MME assigns a Default EPS bearer ID1 to the PDN connection. The SMF+PGW-C will calculate a PDU Session ID1 with the value of 64 + Default EPS bearer ID1 according to Table 5.4.2-1 of TS 29.571, e.g. if Default EPS bearer ID is 5, then the calculated PDU Session ID1 is 69. And then the SMF+PGW-C sends the PDU Session ID1 to the PCF as a parameter to identify the PDU session for the SUPI, DNN and S-NSSAI. - UE handovers to the ePDG and the ePDG assigns a new Default EPS bearer ID2. The Default EPS bearer ID2 may be the same as the Default EPS bearer ID1 or different from the Default EPS bearer ID1. CT3 understands the SMF+PGW-C doesn’t calculate a new PDU session ID, since there is no explicit requirement to do so. - UE may establish an additional PDU connection to the same DNN and S-NSSAI via MME while the UE keeps the PDN connection via ePDG. As the MME may assign the same value of Default EPS bearer ID3 as the Default EPS bearer ID1, the SMF+PGW-C would allocate the same PDU session ID3 as the PDU session ID1 (i.e. the Default EPS bearer ID3 is 5, and the PDU Session ID3 will be 69). When the SMF+PGW-C sends the PDU Session ID3 to the PCF, PCF can’t identify this is a different PDU Session than PDU Session ID1 and may reject the request from the SMF+PGW-C incorrectly. For the question in bullet 2 above, the understanding from CT3 is correct that SMF+PGW-C doesn’t calculate a new PDU session ID when the UE handovers between EPS and EPC/ePDG and such behavior is proposed to be kept. For bullet 3) above, the described UE behavior (i.e. UE may establish an additional PDU connection to the same DNN and S-NSSAI via MME while the UE keeps the PDN connection via ePDG) should not happen per clause of 4.12 of TS 23.402, see below: - Multiple PDN connections for a given APN and UE can be supported with the restrictions that all PDN connections for a given APN and UE shall use the same access network and shall all be moved to a new access network during handovers. For multiple PDN connections towards different APNs, if the following scenario ever happens, the same PDU Session ID value could be assigned by the SMF+PGW-C(s) (our info however shows that such scenario is not seen in any real deployment): - UE establishes PDN connection towards APN1 via MME, default EBI is 5, and SMF+PGW-C allocated PSID 69 (as described in bullet 1) above); - UE handovers to EPC/ePDG, PSID 69 continues to be used; - UE then establishes PDN connection towards APN2 via MME, MME may use default EBI 5 again. In this case, the same PSID 69 assigned by the SMF+PGW-C may be assigned, regardless of the same or different SMF+PGWs are involved. In SA2#147E, there are different views expressed whether there is a real issue to resolve and different options are discussed: View-1: No issue is seen in real deloyment View-2: There is an issue to resolve, and 3 options are discussed: Option-1 SMF+PGW-C report PDU Sesssion ID change at EPS and EPC/ePDG handover. Option-2 Involve UDM Option-3 Surrounding NF use PDU Session ID and APN/DNN to differentiate the PDU Session For Option-1, besides the impact on surrounding NFs to handle PDU Session ID change, it changes the rationale that PDU Session ID is not changed during the lifetime of a PDN connection. For Option-3, there is a concern that it may not work properly because PDU Session ID may be used as resource URI by the surrounding NF. For Option-2, it is considered a reasonable approach by several companies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper implements Option-2 as follows: If handover between EPS and EPC/ePDG is required, - the SMF+PGW-C may query the UDM for any PDU Session ID(s) that are already assigned by the same or different SMF+PGW-C by invoking Nudm_UECM_Get service operation. - When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C may perform UDM registration using the Nudm_UECM_Registration service operation. Add a note clarifying the scenario thata same PDU Session ID value could be assigned by SMF+PGW-C for multiple PDN connections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of duplicated PDU Session ID for different PDN connections in certain scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.5, 4.11.0a.x (new), 5.2.3.1, 5.2.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

#### 4.11.0a.5	PDN Connection Establishment

During establishment of non-emergency PDN connection in the EPC, the UE and the SMF+PGW-C exchange information via PCO as described in clause 5.15.7 of TS 23.501 [2]. For UE with 5GC NAS capability disabled (i.e. N1 mode is disabled), the UE may also allocate a PDU Session ID and send it to the SMF+PGW-C via PCO. If the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the SMF+PGW-C provides an S-NSSAI to the UE, the SMF+PGW-C should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs, and that the S-NSSAI is not subject to Network Slice-Specific Authentication and Authorization, by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the SMF+PGW-C discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]). If the SMF+PGW-C is in a VPLMN, the SMF+PGW-C uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values. If the S-NSSAIs supported by the SMF+PGW-C are all subject to NSSAA, then the SMF+PGW-C should reject the PDN connection establishment. If the selected S-NSSAI is subject to NSAC and EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NSAC_NumberOfUEsUpdate services operation and/or the Nnsacf_NSAC_NumberOfPDUsUpdate services operation to check if the selected S-NSSAI is available as described in clause 4.11.5.9. The SMF+PGW-C uses the Nudm_SDM_Subscribe service operation to subscribe the change of the Session Management Subscription data. If the SMF+PGW-C is notified from UDM with subscription data change, the SMF+ PGW-C takes actions for the PDN connection as described in clause 5.17.2.1 of TS 23.501 [2].

As described in TS 23.548 [74], during establishment of a PDN connection, a UE that hosts EEC(s) may indicate to the SMF+PGW-C, in the PCO, that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s). If the UE indicated in the PCO that it supports the ability to receive ECS address(es) via NAS, the SMF+PGW-C may provide the ECS Address Configuration Information (as described in clause 6.5.2 of TS 23.548 [74]) to the UE in the PCO. The SMF+PGW-C may derive the Edge Configuration Server Information based on local configuration, the UE's location, and/or UE subscription information.

The SMF+PGW-C may use the bearer modification procedure without bearer QoS update to send the UE a PCO with updated ECS Address Configuration Information as defined in clause 6.5.2 of TS 23.548 [74] to the UE.

During establishment of non-emergency PDN connection in the EPC, if SMF+PGW-C is selected for a UE that has 5GS subscription, the SMF may be configured to obtain the subscribed IP index from UDM as part of subscription data using the Nudm_SDM_Get service operation (the SMF+PGW-C discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]).

During establishment of non-emergency PDN connection in the EPC, if SMF+PGW-C is selected for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN and if the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C+PGW-C may proceed as specified in first paragraph of this clause or select any S-NSSAI associated with the APN of the PDN connection. The SMF+PGW-C shall not provide any 5GS related parameters to the UE.

NOTE 1:	The SMF+PGW-C knows that the UE does not support 5GS NAS if the UE does not provide PDU Session ID in PCO (see clause 5.15.7 of TS 23.501 [2]).

During establishment of emergency PDN connection:

-	The SMF+PGW-C is to be derived from the emergency APN or to be statically configured in the Emergency Configuration Data in MME.

-	5GC interworking support with N26 or without N26 is determined based on UE's 5G NAS capability and local configuration (in the Emergency Configuration Data in MME).

-	The S-NSSAI configured for the emergency APN in SMF+PGW-C is not sent to the UE by the SMF+PGW-C. One S-NSSAI is configured for the emergency APN.

During establishment of non-emergency PDN connection and emergency PDN connection, if SMF+PGW-C is selected for a UE that does not support 5GC NAS, the SMF+PGW-C creates unique PDU Session ID for each PDN connection of the UE.

-	The unique PDU Session ID can be created based on the EPS Bearer IDs assigned by the MME for the PDN Connections associated with the UE and not be in the range of PDU Session ID values that can be created by a 5GC NAS capable UE.

-	If handovers between EPS and EPC/ePDG (as specified in clause 8.6 of TS 23.402 [26]) is supported, the uniqueness of PDU Session ID is ensured in the following way:

-	the SMF+PGW-C queries the UDM for any PDU Session ID(s) that are already registered in the UDM for the UE by invoking Nudm_UECM_Get service operation.

-	the SMF+PGW-C creates a PDU session ID that does not collide with the received PDU session ID(s) .

-	When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C performs UDM registration using the Nudm_UECM_Registration service operation.

NOTE 2: When, a UE handovers a PDN connection with APN1 established via MME to ePDG, and then establishes a second PDN connection with APN2 via MME, a same PDU Session ID value could be assigned by the SMF+PGW-C for the second PDN if  SMF+PGW-C does not query the UDM.

When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C provides the ID of the PCF ID selected for the PDN connection in the UDM using the Nudm_UECM_Registration service operation.

A SMF+PGW-C may support L2TP as described in clause 4.3.2.4. In this case step 1 and step 7 of Figure 4.3.2.4-1 correspond to a PDN Connection establishment and a SMF+PGW-C replaces the SMF in that Figure.

To support User Plane Integrity Protection with EPS and policies that Require User Plane integrity protection to be used, at PDN connection establishment, the MME shall indicate to the SMF+PGW-C (via the Serving GW) whether the UE and the MME support User Plane Integrity Protection in EPS. If the MME and the UE support User Plane Integrity Protection, then the SMF+PGW-C informs the MME of the User Plane integrity protection policy (Required, Preferred, Not Needed) applicable to the PDN connection on a per-EPS bearer basis.

******************** Next Change ********************

#### 4.11.0a.x	PDN Connection Release

When at PDN Connection Establishment, the SMF+PGW-C has registered with the UDM (see clause 4.11.0a.5), the SMF+PGW-C shall deregister with UDM during the PDN Connection Release procedure.

****************** NEXT CHANGE ***************

### 5.2.3	UDM Services

#### 5.2.3.1	General

The following table illustrates the UDM Services and Service Operations.

Table 5.2.3.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscriber Data | Get | Request/Response | AMF, SMF, SMSF, NEF, 5G DDNMF |
| Management | Subscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF |
| (SDM) | Unsubscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF |
|  | Notification | Subscribe/Notify | AMF, SMF, SMSF, NEF, GMLC, 5G DDNMF |
|  | Info | Request/Response | AMF, NEF |
| UE Context Management (UECM) | Registration | Request/Response | AMF, SMF, SMSF |
|  | DeregistrationNotification | Subscribe/Notify | AMF |
|  | Deregistration | Request/Response | AMF, SMF, SMSF |
|  | Get | Request/Response | NEF, SMSF, GMLC, NWDAF, SMF |
|  | Update | Request/Response | AMF, SMF |
|  | PCscfRestoration | Subscribe/Notify | AMF, SMF |
| UE | Get | Request/Response | AUSF |
| Authentication | ResultConfirmation | Request/Response | AUSF |
| EventExposure | Subscribe | Subscribe/Notify | NEF (NOTE), NWDAF |
|  | Unsubscribe |  | NEF (NOTE), NWDAF |
|  | Notify |  | NEF (NOTE), NWDAF |
| Parameter | Update | Request/Response | NEF, AMF |
| Provision | Create | Request/Response | NEF |
|  | Delete | Request/Response | NEF |
|  | Get | Request/Response | NEF |
| NIDDAuthorisation | Get | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| ServiceSpecificAuthorisation | Create | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| NOTE: Other NFs are allowed to consume the service based on roaming agreement or operator policy. |  |  |  |

****************** NEXT CHANGE ***************

##### 5.2.3.2.4	Nudm_UECM_Get service operation

Service operation name: Nudm_UECM_Get.

Description: The NF consumer requests the UDM to get the NF ID or SMS address of the NF serving the UE.

Inputs, Required: UE ID, NF Type.

Inputs, Optional: Access Type, Analytics ID(s) (if NF Type is NWDAF), indication to retrieve PDU Session information.

-	Access Type is included when the NF type indicates SMSF.

Outputs, Required: NF ID or SMS address of the NF corresponding to the NF type requested by NF consumer. If the NF Type requested by NF consumer is AMF, the Access Type corresponding to the AMF is also included. Analytics ID(s), if NF Type is NWDAF.

Outputs, Optional: SUPI or GPSI or both, PDU Session information.

For more information on retrieving PDU Session information, refer to clause 4.11.0a.5.

****************** END OF CHANGE ***************