SA WG2 Meeting #149e	S2-xxx

Elbonia, Feb 14 - 25, 2022	(revision of S2-xxx)

Source:	vivo

Title:	Key Issue on verification of URSP rule enforcement in UE

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: This contribution introduces key issue on verification of URSP rule enforcement in UE

# 1. Introduction

According to the SP-211649, the study on Enhancement of 5G UE Policy targeted at R18 allows enhancement to current design of UE policy. And for the WT#4, it describes:

-	#4: Investigate whether and how the network can be made aware when the UE enforces URSP rule and investigate whether and what actions the 5G network can perform.

Issues and requirement:

For the current procedure, the 5GC designs URSP rules according to AF guidance or subscription data. And 5GC delivers URSP rules to UE for matching application traffic to dedicated PDU session or establish new PDU session with the PDU session parameter indicated in matched URSP rules.

But for a dedicated application traffic on UE, it seems the potential matched traffic descriptor or RSDs are multiple. Also, the operators don’t know the details status of the PDU sessions that carry the application traffic. It is useful for operator to know the execution of URSP rules in UE, to further optimize the design of URSP rules for a dedicated application or traffic. Because due to some reasons, some of the URSP rules in UE may have low priority or probability to be used for the dedicated application traffic. The MNO need to know the execution status of specific URSP rules in UE, to make a further optimization and design.

Proposal:

Add a KI in the new TR for the FS_eUEPO, to study how to verify the URSP rule enforcement in UE:

- How to the 5GC aware of the dedicated URSP rules enforcement in UE?

- Whether and how the 5GC behave or perform actions according to the actual URSP rules enforcement in UE?

# 2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-85.

- * * * * First change, all new texts* * * *

## 5.X	Key Issue #X: Introducing traffic category in URSP rules

### 5.X.1	General description

For the current procedure, the 5GC designs URSP rules according to AF guidance or subscription data. And 5GC delivers URSP rules to UE for matching application traffic to dedicated PDU session or establish new PDU session with the PDU session parameter indicated in matched URSP rules.

But for a dedicated application traffic on UE, it seems the potential matched traffic descriptor or RSDs are multiple. Also, the operators don’t know the details status of the PDU sessions that carry the application traffic. It is useful for operator to know the execution of URSP rules in UE, to further optimize the design of URSP rules for a dedicated application or traffic. Because due to some reasons, some of the URSP rules in UE may have low priority or probability to be used for the dedicated application traffic. The MNO need to know the execution status of specific URSP rules in UE, to make a further optimization and design.

The following aspects shall be studied to whether and how to verify the URSP rule enforcement in UE:

- How to the 5GC aware of the dedicated URSP rules enforcement in UE?

- Whether and how the 5GC behave or perform actions according to the actual URSP rules enforcement in UE?

- * * * * End of changes * * * *