3GPP TSG-SA2 Meeting #149E	S2-2203992

Online, 14th – 25th February 2022

Source:	Rakuten Mobile

Title:	New Solution: PSAP resolution using NWDAF with finer granularity than cell level

Document for:	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA Ph3 /Rel-18

1	Discussion

In case of the emergency services call if the cell covers very large area e.g. NR satellite access, TA/Cell level location info. is not enough to determine route an emergency call to the corresponding PSAP.

TS23.167 specifies the case using LRF/RDF to retrieve location and routing information. In this case the location information for the UE can be used, but there are several cases that the UE’s recent location is not stored and so on. In such cases the stored and calculated information by NWDAF e.g. the mobility information of the UE, OAM info, the other UEs location data can be available to determine the corresponding PSAP.

Figure 7.2: Emergency Session Establishment procedure with using LRF/RDF

2	Proposal

It is proposed to adopt the following changes into TR23.700-81.

* * * * Start of Changes * * * *

6	Solutions

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | X |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |  |  |  |
| 8 |  | X |  |  |  |  |  |  |  |  |
| 9 |  | X |  |  |  |  |  |  |  |  |
| 10 |  |  | X |  |  |  |  |  |  |  |
| 11 |  |  | X |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |
| 13 |  |  |  |  | X |  |  |  |  |  |
| 14 |  |  |  |  | X |  |  |  |  |  |
| 15 |  |  |  |  | X |  |  |  |  |  |
| 16 |  |  |  |  |  | X |  |  |  |  |
| 17 |  |  | X |  |  | X |  |  |  |  |
| 18 |  |  |  |  |  |  | X |  |  |  |
| 19 |  |  |  |  |  |  | X |  |  |  |
| 20 |  |  |  |  |  |  | X |  |  |  |
| 21 |  |  |  |  |  |  |  | X |  |  |
| 22 |  |  |  |  |  |  |  | X |  |  |
| 23 |  |  |  |  |  |  |  | X |  |  |
| 24 |  |  |  |  |  |  |  | X |  |  |
| 25 |  |  |  |  |  |  |  |  | X |  |
| 26 |  |  |  |  |  |  |  |  | X |  |
| 27 |  |  |  |  |  |  |  |  | X |  |
| x |  |  |  |  |  |  |  |  | X |  |

* * * * Next Change * * * *

6.X	Solution #X: PSAP resolution with finer granularity of location information

6.X.1	Description

This solution addresses Key issue #9: Enhancement of NWDAF with finer granularity of location information. This solution proposes a use case about PSAP (Public Safety Answering Point) resolution for emergency services with finer granularity of location information.

6.X.2	Functional description

At a high level, the functional description of this solution is as follows

- UE initiates an emergency session request with including emergency URI and Cell Global Identification (CGI) with existing procedure.

- If required, IMS network access to the NWDAF to retrieve the PSAP destination with the estimated UE's location with finer granularity than Cell/TA level

- The IMS network uses the routing information returned by the NWDAF to route the emergency session request towards the appropriate PSAP.

Input data (example)

- UE ID

- Cell Global Identification (CGI)

- UE location info (if available)

Output data (example)

- PSAP destination

6.18.3	Procedures

Figure 4.10-1: Emergency Session Establishment procedure with using NWDAF

6.18.4	Impacts on services, entities and interfaces

Existing Emergency service procedure will be updated to acquire the PSAP destination with the finer granularity of location info than Cell ID/TA level.

NWDAF:

-	The NWDAF collects and analysis data from LCS and the other NFs to determine UE location and routing information.

-	The new Analytic ID supporting the PSAP resolution use case needs to be supported.

Editor's note:	The name of new analytic ID is FFS.

* * * * End of Changes * * * *