SA WG2 Meeting #S2-151E	S2-2204030

16 - 22 May, 2022, E-Meeting

Source: 	NEC

Title: 	Addressing ENs of solution 10

Document for: 	Agreement

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract of the contribution:.The objective of this pCR is to address the ENs added to the solution 10.

# Discussion

The objective of this pCR is to address following ENs.

Editor's note:	The impact of termination of temporary slice to NFs other than AMF is FFS.

Editor's note:	It is FFS whether a slice which does not have associated validity timer can be configured with validity timer using UPU procedure.

Editor's note:	If the UE is in connected state, how the PDU session of the network slice is gracefully released is FFS.

# Proposal

Add the following changes to TR 23.700-08.

* * * First Change * * * *

## 6.10	Solution #10: Associating a validity timer with a temporary slice

### 6.10.1	Introduction

The solution addresses second part of the key issue #3 " Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices " which states that there can be case where a network slice is deployed temporarily for a particular time period

The network (e.g. AMF, SMF, NSSF or other NFs) associates a validity timer for each S-NSSAI which is deployed temporarily either by explicit signalling (e.g. the NFs gets validity timer from the UDM) or through O&M procedure. This validity timer for each temporary S-NSSAI is sent to the UE e.g. in the configured NSSAI list during the registration procedure. When the UE receives the S-NSSAI with a validity timer in the configured NSSAI, the UE runs a validity timer. The UE will register to the S-NSSAI when the validity timer is running. In both CM-IDLE once the validity timer expires the UE locally removes the S-NSSAI from the allowed NSSAI list and the UE locally release the PDU session associated with the S-NSSAI. The network (e.g. AMF) also runs the validity timer for the S-NSSAI, once the validity timer expires the network removes the S-NSSAI from the allowed NSSAI list and locally release the PDU sessions associated with the S-NSSAI. In CM-CONNECTED state, on expiry of the validity timer, the AMF initiates PDU session release procedure. The network may also associate a S-NSSAI with validity timer if the S-NSSAI was not associated with any validity timer. The UE provides the capability to the AMF which provides to the UDM or other network function that it supports configured NSSAI with validity timer. The UE not supporting the timer doesn't get the validity timer in the configured NSSAI. In addition of UDM other entities can also provide the validity timer. This covers both the case where the timer is set in the serving network (for the serving S-NSSAI) and from the subscription (UDM-based, for the subscribed S-NSSAI).

### 6.10.2	Procedures

Figure 6.10.2-1: Handling of a temporary network slice

The detailed procedure are as described below:

0.	The UE can be preconfigured with configured NSSAI with validity timer. Configured NSSAI can also come in UE Configuration Update message.

NOTE 1: A S-NSSAI can be added or removed dynamically as per the current procedure.

1.	The UE initiates registration procedure and sends registration request message.

2-3.	The network may trigger authentication procedure and security mode command procedure.

4-6.	The AMF registers to the UDM for the UE. the UDM provides the subscribed NSSAI to the AMF. In the subscribed NSSAI the UDM associate a validity time for each temporary S-NSSAI.

NOTE 2:	The validity timer for a temporary S-NSSAI can also be provided by other NSSF.

7.	The AMF sends configured NSSAI = Subscribed NSSAI to the UE in the registration accept message.

8.	The UE stores the configured NSSAI.

9.	The sends registration complete message to the AMF.

10a.	The UE starts t1. The UE initiates registration to the S-NSSAI 1 while the timer t1 is running. Once the t1 expires the UE removes the S-NSSAI 1 from the allowed list locally. The UE removes the S-NSSAI 1 from the configured NSSAI list.

10b.	AMF starts t1 is. The AMF accepts registration to the S-NSSAI 1 while the timer t1 is running. Once the t1 expires the UE removes the S-NSSAI 1 from the allowed list locally.

NOTE 3:	The value of the timer can be hours, days or months and details can be captured in stage 3.

### 6.10.3	Impacts on services, entities and interfaces

UE

-	Handling of validity timer for a S-NSSAI.

AMF, SMF, NSSF

-	Handling of validity timer for a S-NSSAI.

UDM

-	UDM needs to provide the validity timer for a temporary S-NSSAI for a UE to the AMF.