SA WG2 Meeting #S2-152e(e-meeting)	S2-2206624

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI #5: solution evaluation and conclusion for key issue#5

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: this pCR is proposed to add evaluation and conclusion for key issue#5.

1. Discussion

This pCR is proposed to add evaluation and conclusion for key issue#5.

Key issue#5 has been discussed for three meetings and there are four solutions related to the key issue. This pCR analyses the common ground and differences between the solutions. Because the differences of solutions need further discussion, the conclusion proposed by this pCR is based on the common ground of the solutions.

2. Proposal

It is proposed to add evaluation and conclusion for key issue#5.

* First change *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.x	Key Issue #5: Support of location services for UEs accessing via a mobile base station relay

There are four solutions related to key issue#5, i.e. solution#7, solution#8, solution#14 and solution#15.

The common ground of the solutions includes:

-	The LMF obtains MBSR location to calculate target UE location.

-	The LMF decides that the target UE connects to MBSR based on information received from the AMF. The AMF provides the information to LMF when it determines that the UE connects to MBSR based on the additional ULI received from NG-RAN as described in solution#7, #8 and #14. However, it is not described in solution#15 that how the AMF makes the decision. Furthermore, solution#15 also includes another way for LMF to make the decision, i.e. based on UE positioning capability which is enhanced to include MBSR access provided by UE via LPP. But it is also not described that how the UE determines that it connects to MBSR.

The difference of the solutions is that the way for LMF to obtain the MBSR ID is different. In solution#7, #8 and #15, LMF obtains the MBSR ID using NRPPa message. In solution#14, LMF obtains MBSR ID from AMF which obtains MBSR ID from UDM. Solution#14 impacts AMF, UDM, so it impacts more NFs than the other solutions.

8	Conclusions

Editor's note:	This clause provides the conclusions for the study.

8.x	Conclusion for Key Issue #5: Support of location services for UEs accessing via a mobile base station relay

It is concluded that:

-	LMF obtains MBSR location to calculate target UE location when it determines that the target UE connects to MBSR.

* End of change *