3GPP TSG-SA WG2 Meeting #152-e	S2-2206662r01

# E-Meeting, 17th – 26th Aug 2022

Title:	[draft]LS Out on RAN dependency of FS_eNS_Ph3

Response to:	None

Release:	Rel-18

Work Item:	FS_eNS_Ph3

Source:	SA2

To:	RAN2, RAN3

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA2 starts the Rel-18 study FS_eNS_Ph3 study in TR 23.700-41(link). This study includes 6 key issues and 39(to be updated) solutions so far. SA2 is now evaluating all solutions and try to conclude the study at Oct meeting. Since some solutions have RAN dependency, SA2 would like to ask RAN2 and RAN3 on their views on the following aspects:

For key issue #1: Support of Network Slice Service continuity

- In solution 3, the impacts on the NG-RAN are following:

-	During Xn based handover, the target NG-RAN indicates to AMF in Path Switch Request to replace the congested network slice with a different network slice.

- In solution xx(as agreed in S2-2205875rxx), the impacts on the NG-RAN are following:

-	The NG-RAN detects that the target gNB does not support one of the UE’s network slices and select to do NG HO.

For Key Issue #3:  Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices

- In solution 9, the impacts on NG-RAN are following:

-	Broadcast one or more additional TAIs via a new information element in SIB

-	Support new messages in the NG-AP and Xn-AP as well as F1AP procedures to share support of additional secondary TAI, or indicating the cell where the UE is support >1 TAC and what the TAC and which ones the primary/secondary values are.

- In solution 11, the impacts on NG-RAN are following:

-	The NG-RAN is configured with a slice availability on a per-cell basis and inform AMF in NG Setup

- In solution 24, the impact on NG-RAN are following:

-	The NG-RAN is configured with temporarily/periodically available S-NSSAIs per TA basis and inform AMF the AMF in NG-Setup.

-	The NG-RAN determines when certain DRBs are no longer active for certain sessions and locally release/deactivate the DRBs without further signalling.

- In solution 29, the impact on NG-RAN are following:

-	The NG-RAN is able to trigger handover procedure to a supporting TAI when the UE is using certain S-NSSAIs (partially allowed in the Registration Area) upon transition from CM-IDLE mode to CM-CONNECTED mode

- In solution xx(as agreed in S2-2205876rxx), the impacts on the NG-RAN are following:

-	NG-RAN indicates to AMF during NG SETUP (CONFIGURATION UPDATE) that some NSAGs are supported in some cells of a TA

-	NG-RAN supports extended Mobility Restriction List (that is extended with per S-NSSAI and per cell restrictions

- In solution yy(as agreed in S2-2205563rxx), the impacts on the NG-RAN are following:

-	The NG-RAN is configured Constrained Service Area information for the network slice. The Constrained Service Area per S-NSSAI includes the Cell IDs of the cell(s)

-	Prevent the UE from entering into a cell belonging to the CSANS unnecessarily when any PDU session related to the S-NSSAI has an active user plane

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 kindly asks RAN2 and RAN3 to provide feedback on the aspects above before SA2#153e Oct meeting

3. Date of Next SA2 Meetings:

3GPP TSG SA2#153e		10-14,Oct, 2022	E-Meeting

3GPP TSG SA2#154e		14-18, Nov,2022	E-Meeting