SA WG2 Meeting #SA2-152E 	S2-2206772

17 – 26 August, 2022 Electronic meeting

Source:	China Mobile

Title:	New Solution: Supporting ECN in 5GS for XR/media Enhancements

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes a solution update for Sol#43

# Discussion

# Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

## 6.X	Solution #X: Supporting ECN in 5GS for XR/media Enhancements

### 6.X.1	Key Issue mapping

The solution addresses Key Issue #3: 5GS information exposure for XR/media Enhancements.

### 6.X.2	Description

The ECN technology has been describe in Sol#41,#43 and#46. There are some differences between the three solutions but the main idea is the same that introducing ECN bits in the 5GS to indicate the node congestion happen.

While some aspects should be further clarified.

1) PCF enhancement

When AF provides QoS parameters requirement for specific XR/media traffic, the request for utilizing ECN can also be included. AF may also provide ECN marking instruction information e.g. support for adapting data rate based on ECN marking for L4S, desired data rate for application to perform properly while ensuring Quality of experience for end user. PCF can generate corresponding policy based on these instruction information, and send the policy to RAN or UPF to help the ECN marking.

2) UE capability indication

UE application layer supports ECN marking for L4S.

3) 5G Mobility consideration

### During NG Handover, if the target NG-RAN is no longer congested, it can stop marking ECN for L4S and the application may start to gradually increase the data rate.

### 6.X.3	Impacts on services, entities and interfaces

AF:

-	Provide ECN marking instruction information e.g. support for adapting data rate based on ECN marking for L4S, desired data rate for application to perform properly while ensuring Quality of experience for end user

PCF:

-	Generate corresponding policy based on AF instruction information, and send the policy to SMF and RAN or UPF to help the ECN marking.

UE application layer:

-	Supports ECN mechanism.

# * * * *end of change * * * *