3GPP TSG-WG SA2 Meeting #154	S2-2210464

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI #3, conclusion update

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This contribution proposes to update KI#3 conclusion.

1. Introduction/Discussion

L4S is one of promising end-to-end congestion control mechanism and getting more supported. However, there are still chances to fail to connect during the handover procedure.

For example, if the UE can be used L4S service through the Source-RAN, due to UE mobility, the UE can be handover from Source-RAN to Target-RAN. Depends on Target-RAN implementation, it can be possible to provide limited functionality on the Target-RAN for supporting congestion notification through the User plane such as the L4S/ECN marking or the congestion information put into GTP-U header insertion.

In this paper proposes that NG-RAN provides the congestion information to PSA UPF via control plane signalling as represented by the red line in Figure 1. NG-RAN performs reporting congestion information to SMF via N2 message. SMF received reporting congestion information from NG-RAN and it can be provides the congestion information to PSA UPF for ECN marking through the N4 rule update.

Figure 1. Scenario on UE handover to a Target-RAN which provides limited functionality

In current KI#3 conclusion describes that congestion information only received via GTP-U header for ECN marking in PSA UPF. The author of this paper believes that congestion information with control plane may be enhanced to help operators handle such a scenario.

Proposal: PSA UPF performs ECN marking for uplink and downlink IP layer of the received packets based on latest reported congestion information from NG-RAN via the control plane.

2. Proposal

It is proposed to update KI#3 conclusion as clause 8.3 of TR 23.700-60 v1.2.0.

# * * * * 1st change * * * *

8.3	Conclusions for Key issue#3: 5GS information exposure for XR/media Enhancements

The following bullet points summarize the principles for the way forward to support current congestion level information exposure:

-	5G System may use ECN marking for the purpose of Low Latency, Low Loss and Scalable Throughput services L4S according to [37] and [62] for uplink and/or downlink QoS Flows via one of the following two methods. A QoS Flow level explicit indication may be provided to PSA UPF to enable the ECN marking for the purpose of L4S.

-	Method1: To support L4S, NG-RAN performs ECN marking according to [37] and [62] for uplink and downlink in IP layer of the received packets.

NOTE 1:	The criteria for RAN to determine (e.g. its congestion level) when to perform the marking is up to RAN implementation.

-	Method2: PSA UPF performs ECN marking according to [37] and [62] for uplink and downlink IP layer of the received packets based on latest reported congestion information from NG-RAN via GTP-U header. When no congestion/congestion ends, the PSA UPF stops ECN marking.

In Method2, if there is no UL packet when report is needed (e.g. for DL congestion), NG-RAN may generate an UL Dummy GTP-U Packet for such a reporting.

In a scenario where NG-RAN does not support ECN marking or report congestion information via GTP-U header, NG-RAN reports congestion information to SMF via Control plane (N2 message). After receiving congestion information from NG-RAN, SMF performs N4 rule update associated QoS flow with reported congestion information. PSA UPF performs ECN marking according to [37] and [62] for uplink and downlink IP layer of the received packets based on latest reported congestion information from NG-RAN via the control plane signaling (N2 message and N4 rule).

NOTE 2:	The specification of the mobility scenario for both methods is left to normative phase.

-	For both methods, ECN marking for L4S is per QoS flow. In order to map a packet flow that can be subject to ECN marking for L4S to a QoS flow with ECN marking for L4S support, the traffic detection is used at the UPF. For traffic detection, the packet filters can either reuse existing IP-5 tuples, or ECT(1).

NOTE 3:	If the network operator want to apply the ECN marking for L4S, it shall guarantee that any sender (UE or Server) requesting classic ECN congestion control will not tag its packets with the ECT(1) in order to avoid conflicted usage of ECT(1) in L4S. Otherwise, L4S is not supported in network.

NOTE 4:	Supports for L4S and for exposure of congestion information is pending RAN WG's feedback on the feasibility of RAN judgment and/or exposure of the corresponding info (e.g. per QoS flow congestion information).

-	5G System also may support API based exposure of congestion level information towards AF as following:

-	The following information may be exposed by RAN:

-	QNC for GBR QoS Flow: data rate cannot be guaranteed.

-	RAN provides the congestion information to PSA UPF enabling PSA UPF to perform API exposure towards the AF and ECN marking for L4S;

-	AF uses Nnef_AFSessionWithQoS to subscribe the above exposure to NEF/PCF, same as local exposure mechanism defined in TS 23.548 [61].

-	Exposure path of Network Exposure defined in clause 6.4 of TS 23.548 [61] is reused with extensions of GTP-U header and UPF/L-NEF services to exposure the above information.

-	Exposure path of RAN/UPF reporting congestion level information via SMF/PCF/NEF is also supported.

The following bullet points summarize the principles for the way forward to support exposure for other network information:

-	Data rate, delay difference and round trip delay of QoS flow may be exposed to AF:

-	Data rate may be measured and exposed by PSA UPF. Exposure path defined in clause 6.4 of TS 23.548 [61] is reused to expose the above information. Exposure path of UPF reporting via SMF/PCF/NEF is also supported.

-	The RAN may support exposing the above data rate information via SMF/PCF/NEF.

-	AF may request to be notified when the delay difference between two QoS Flows exceeds a threshold. The delay measurement for individual QoS Flows is based on QoS monitoring in clause 5.33.3 of TS 23.501 [2].

-	Round trip delay for multiple QoS flows of the XR service (e.g. the UL and DL are separated into two flows) can be obtained and exposed by the PSA UPF via the exposure path defined in clause 6.4 of TS 23.548 [61] or via SMF/PCF/NEF.

Editor's note:	It is FFS whether to expose the Normal data transmission interruption event to AF.

-	Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 in TS 23.288 [59]) to AF.

# * * * * End of changes * * * *