SA WG2 Meeting #154	S2-2210845

# 14 – 18 November 2022, Toulouse, France

Source:	Lenovo

Title:	KI#10: Resolve EN in solution #74

Document for:	Approval

Agenda Item:	9.23.1

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract of the contribution: This paper address EN in section 6.74.1.

# 1		Discussion

This contribution aims to resolve the editor note in section 6.74.1 by providing further details on how solution #74 and solution #73 (which was the former contribution S2-2206132) are aligned.

# 2		Proposal

The following solution is proposed.

******************************** First change (all new text) *******************************

# 6	Solutions

## 6.74	Solution #74: Supporting NWDAF interactions with MDAF

### 6.74.1	Description

This solution is based on the MDA functional overview and service framework in Figure 6.73.1-1 (i.e., Figure 5.1-1 as defined in TS 28.104 [15]) re-using the existing interfaces, i.e., Nnwdaf and MDAS (MDA MnS).

Figure 6.73.1-1: MDA functional overview and service framework, TS 28.104 [15]

When a consumer issues an NWDAF Analytics ID request that relies on management data analytics input from an MDAF or MDA MnS producer, this request needs to trigger once received at a particular NWDAF a subsequent request or subscription towards the appropriate MDAF. The interfaces that can be used for this interaction are the Nnwdaf for issuing a request or subscription to MDAF or MDA MnS producer and the MDAS interface for reporting to the NWDAF, as documented in clause 5.1 of TS 28.104 [15].

A consumer request needs to be analysed and processed by the recipient NWDAF to issue the subsequent subscription request, i.e., by deriving the corresponding subscription attributes, towards the MDAF or MDA MnS producer. The new subscription request may include the following attributes:

-	MDA Type: It refers to the desired analytics capability, as specified in clause 8.1.1 of TS 28.104 [15], e.g., slice throughput analytics, slice coverage analysis, considering the use cases and requirements for MDA capabilities as documented in clause 7.2 of TS 28.104 [15]. The MDA type can be selected based on a pre-configured mapping between an Analytics ID and the neccessry input that include the MDA type. Multiple MDA types can be potentially carried in a single request depending on the implementation of MDAF. For each MDA type the consumer can selecet specific MDA outputs, i.e., out of a pre-determined list of potential output results, as specified in caluse 9.3.2 of TS 28.104.

-	Relative time schedule: Considering the time schedule indicated by the consumer, the recipient NWDAF can estimate the corresponding time schedule calculating the upper bound time needed to receive the response from the respective MDAF or MDA MnS producer that relies upon. To achieve this:

i.	each MDAF or MDA MnS producer can register in a MnS discovery service producer as described clause 5.1 of TS 28.537 [28] its capability in terms of the upper bound time in preparing an analytics result based on the available CPU, storage resources and the current workload.

ii.	alternatively, an MDAF can respond to the requesting NWDAF its capability in terms of the upper bound time in preparing an analytics result based on the available CPU, storage resources and the current workload.

iii.	the NWDAF needs to have an estimation of the communications latency in receiving management data analytics. This can be determined by sending a round trip probe message to estimate the expected latency.

-	Start/stop time: It refers to the time schedule for starting and stoping an analytics subscription request where multiple MDA reporting may take place each with a specified reporting time window.

-	Reporting time window: It refers to the desired reporting time window related to the management data analytics output from MDAF.

-	Reporting analytics samples: This indicates how the reporting of the management data analytics samples can be presented, e.g. average value, min-max value, discrete time-based samples, etc.

-	Analytics subscription type: Considering the request type from the consumer, i.e. on-demand request (immediate) or on-demand/periodic request at a specified timed, the NWDAF needs to select the appropriate timing related to the subsequent request towards the MDAF or MDA MnS producer.

-	Reporting method: It refers to the way of receiving the result from MDAF or MDA MnS producer. The NWDAF may request an MDAF or MDA MnS producer to report in a file format when the result has a relatively bigger size, stream-based reporting that suits real-time data or notification based where the consumer receives a notification to fetch the data.

-	Area of Interest: Can be explicitly specified by the consumer and in this case the NWDAF requests the same area capability, i.e. be capable to produce analytics in the specified area, to MDAF or MDA MnS producer.

-	Network Slice: It refers to information (i.e., NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [29]).

-	Target UE(s): NWDAF focuses on a particular UE ID (Subscription Permanent Identifier - SUPI), while the management plane, i.e. MDAF or MDA MnS producer considers the average UE in a particular location. Specifically, when NWDAF needs MDAF or MDA MnS producer output, e.g. resource utilization prediction, in relation to a UE or a group of UE, that reside in a particular area, it needs to translate the location of UE(s) into an area in the form of cells or TAs.

-	Filtering conditions: Refers to the conditions included in a consumer request, which indicate the report triggering criteria. The recipient NWDAF needs to identify where filtering conditions can be applied and include any of them that relate to the subsequent request towards MDAF or MDA MnS producer.

The NWDAF also needs to perform the discovery of the appropriate MDAF or MDA MnS producer needed, by contacting the respective MnS discovery service producer as in clause 5 of TS 28.537 [28]. MDAF or MDA MnS producers need to register themselves to the respective Service Directory with their management capabilities described by MnS information or MnS profile.

NOTE:	The details for the MDA MnS producer discovery by the NWDAF may be based on the existing procedure defined in TS 28.537 [28].

### 6.74.2	Procedures

If a consumer requires analytics from NWDAF that rely on input from MDAF the following apply.

Figure 6.74.2-2: Supporting NWDAF interworking with MDAFrequest and response

Precondition: Initially MDAF(s) or MDA MnS producers register their capabilities, i.e., MnS information or MnS profile as described in clause 5 of TS 28.537 [28] to a MnS discovery service producer. The MnS discovery service producer may contain all or partial information related to the capabilities of MDA MnS producer.

1	An analytics consumer issues a request using either on-demand request or subscription method towards the selected NWDAF as described in TS 23.288 [5].

2.	The recipient NWDAF uses the Analytics Request Processing or Analytics Logic (that resides within the same function or can alternative be a separated NF) to determine the requirements of the received analytics request to identify the corresponding attributes (i.e. as mentioned above: Analytics ID, Relative time schedule, Reporting time window, etc.).

3.	The recipient NWDAF performs the discovery of the corresponding MDAF or MDA MnS producer. The discovery step can take place in two different ways:

i.	the recipient NWDAF requests the Service Directory indicating the desired capabilities for a target MDAF or MDA MnS producer in step 3a and receives a reply in step 3b suggesting the set of MDAF or MDA MnS producer including the corresponding address to select from,

ii.	the recipient NWDAF only receives basic profile information, i.e., the address, from the Service Directory in steps 3a and 3b and needs in step 3c to request from each suggested MDAF or MDA MnS producer its capabilities before being able to select the desired one(s).

4.	If the discovery process returns an empty set, i.e. with no results that can fulfil the desired capabilities, then the NWDAF may optionally negotiate with the analytics consumer, i.e. to change or downgrade, the requirements of the analytics request. The requirements that the NWDAF may negotiate with the consumer if the desired one(s) cannot be fulfilled may include the:

a	Reporting time window where the NWDAF can propose a new reporting time window based on the received MDAF or MDA MnS producer options set provided by the discovery process.

b	Analytics subscription type where the NWDAF can degreade the analytics subscription type, e.g. from an on-demand immediate response to a response estimating an additional latency, based on the received set of MDAF or MDA MnS producer options provided by the discovery process.

If a solution cannot be reached in the negotiation step or if the negotiation step is not supported, then the process is terminated at this point.

5.	If the suggested set of MDAF or MDA MnS producer that fulfils the desired capabilities is nonempty or a negation step assisted in reaching a solution, then the recipient NWDAF selects the desired MDAF or MDA MnS producer out of the suggested set and then prepares the format the corresponding MDA request, i.e. by deriving the requirements or corresponding attributes of the MDA request.

6.	The MDA request is issued automatically towards the selected MDAF or MDA MnS producer.

7.	The MDAF or MDA MnS producer prepares the MDA result.

8.	The MDAF or MDA MnS producer returns the MDA responds providing the desired analytics output results.

9.	The recipient NWDAF collects additionally raw data from other data sources, which may include NFs, MnSs, repository functions, e.g. NRF, UDR, URM, data collection entities, e.g. DCCF, ADRF, to calculate the requested analytics result.

10.	The NWDAF prepares the analytics result.

11.	The NWDAF provides the analytics response towards the analytics consumer that includes the requested result.

### 6.74.3	Impacts on services, entities and interfaces

-	NWDAF contains or interact with the Analytics Request Processing or Analytics Logic to determine the attributes of the MDAF on-demand request or subscription.

-	NWDAF discovers of the appropriate MDAF or MDA MnS producer form the MnS discovery service producer.

-	NWDAF requests or subscribes to the desired OAM related analytics from the corresponding MDAF or MDA MnS producer.

-	NWDAF may optionally negotiate with the analytics requirements related to a consumer request.

******************************** End of change *******************************