SA WG2 Meeting #S2-155	S2-2302406

# Athens, Greece, Feb 20 – 24, 2023                                              revision of S2-2301473

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3792 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF support of 5GS Packet Delay Variation monitoring based on QoS monitoring mechanism and exposed to AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, [Tencent, Samsung, Nokia Nokia Shanghai Bell] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the conclusion of 8.7 in TR 23.700-60 for Key Issue #7 policy enhancements to minimize jitter has been concluded, 5GS should support to monitor the 5GS jitter value i.e. Packet Delay Variation based on AF request with utilizing the QoS monitoring method. Resoving the EN about RT latency |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GS support to monitor the 5GS Packet Delay Variation value based on AF request with utilizing the QoS monitoring method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF cannot get the Packet Delay Variation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

5.37.X		 5GS Packet Delay Variation monitoring and reporting

5GS Packet Delay Variation means the variation of UL, DL or RT packet delay measurements between UE and PSA. AF may send the requirement for Packet Delay Variation monitoring to 5GS as described in TS 23.503[12] clause 6.1.3.xx. Based on the QoS monitoring mechanism described in clause 5.33.3, the 5GS transmission packet delay variation is calculated or monitored by PCF and then reported to AF.

When PCF gets the Packet Delay Variation requirement, the related QoS monitoring policy is generated for a service data flow. Then PCF triggers the QoS monitoring procedure, and gets the UL, DL or RT packet delay from SMF. PCF may calculate the 5GS Packet Delay Variation between UE to PSA UPF based on the UL, DL or RT packet delay values and then reports the value to AF/NEF.

NOTE: By implementation, the PCF calculates the 5GS transmission Packet Delay Variation based on QoS monitoring report of packet delay values.

Editor’s Note: Whether all QoS monitoring needs can be addressed with the limitation of one QoS Monitoring control information per PCC rule is FFS.

<<< End of changes >>>