3GPP TSG-WG SA2 Meeting #155	S2-2302426

Athens, Greece, February 20 - 24, 2023	(revision of S2-230xxxx)

Source:	Lenovo

Title:	Use of NSSF to determine the Alternative S-NSSAI

Document for:	Discussion and approval

Agenda Item:	9.11.2

Work Item / Release:	eNS_Ph3 / Rel-18

Abstract: this paper proposes to agree on the impacts required for the NSSF to support KI#1 conclusions.

# 1.	Introduction/Discussion

## 1.1.	Why the NSSF is considered for Alternative S-NSSAI determination

One of the main questions which should have been answered within KI#1 are: a) what triggers the network slice replacement and b) how to determine the available Alternative S-NSSAIs.

Let us first look on the scenarios which were documented in the KI#1 description:

1)	No mobility scenario:

Scenario 1a): network slice is overloaded in NG-RAN.

Scenario 1b): network slice or network slice instance is overloaded or undergoing planned maintenance in CN (e.g. network slice termination).

Scenario 1c): network performance of the network slice cannot meet the SLA.

2)	Inter RA Mobility scenario:

Scenario 2a): network slice is not supported in the target RAN node.

Scenario 2b): network slice in target RAN node is overloaded.

Scenario 2c): network slice is not supported in the target CN.

Scenario 2d): network slice or network slice instance is overloaded in the target CN.

This key issue is to study whether and how to provide service continuity for PDU sessions in network slices in the above scenarios 1b), 1c) and 2d).

Analysis of the scenarios:

- In the scenario 1b), the first part of the scenario "network slice or network slice instance is overloaded" can be solved by the existing mechanism like the NAS-level congestion control. But assuming that a new solution is desired, the NSSF can make use of the NWDAF analytics “Slice load level related network data analytics” (see TS 23.288 clause 6.3). If the NSSF determines that the S-NSSAI is loaded on system-level, the NSSF can determine on one or more Alternative S-NSSAIs which have the same/similar area of service/coverage and offering similar characteristics. The NSSF can inform the associated AMFs about the slice replacement.

It is more interesting to analyse the latter part "network slice or network slice instance is… undergoing planned maintenance in CN (e.g. network slice termination)".  For the "planned maintenance in CN (e.g. network slice termination)" it means that the OAM system configures the NFs serve the slice in the 5GC (e.g. AMFs or NSSF) that the S-NSSAI is to be terminated as described in TS 28.541 where for graceful and gradual termination of NSIs and NSSIs the SHUTTING DOWN state is used. Apparently, this is system-level configuration, and therefore, provides a system-level trigger that the network slice is to be terminated. Based on this termination trigger, the NSSF can determine on one or more Alternative S-NSSAIs which have the same/similar area of service/coverage and offering similar characteristics, as the NSSF has system level overview of the network slice deployment.  The NSSF can provide the Alternative S-NSSAIs to the AMF. The AMF determines a single Alternative S-NSSAI applicable for each UE which is currently registered to the old S-NSSAI.

- For scenario 1c), the NSSF can subscribe for NWDAF analytics related to the “Service Experience for a Network Slice” (see TS 23.288 clause 6.4).  This is pre Rel-18 available functionality. Based on the NWDAF analytics, the NSSF may determine (on system-level) that there are issues with the Network Slice performance. The NSSF may determine that the impacted S-NSSAI can be replaced by one or more Alternative S-NSSAIs and may provide the Alternative S-NSSAIs to the AMF.

Proposal 1:	the NSSF has system-level overview of the deployment of the old S-NSSAI and can determine one or more Alternative S-NSSAIs having the same area coverage and similar characteristics. The NSSF provides the Alternative S-NSSAI(s) to the AMFs serving the old S-NSSAI.

Proposal 2:	When receiving the Alternative S-NSSAI(s), the AMF determines a single Alternative S-NSSAI applicable for each UE which is currently registered to the old S-NSSAI.

NOTE:	The criteria in the AMF to select an Alternative S-NSSAI can be based on one of the following:

(A) the AMF can consider whether some of the Alternative S-NSSAIs are part of the UE’s Subscribed S-NSSAIs, and if yes, then select it;

(B) the AMF may distribute the UEs among the available Alternative S-NSSAIs, so that no overload in the Alternative S-NSSAI occur.

## 1.2.	Analysis of roaming architecture

The NSSF can determine an Alternative S-NSSAI in both non-roaming case and in roaming case. Specifically in roaming case, the H-NSSF can determine that the HPLMN S-NSSAI needs to be replaced. The H-NSSF can use the existing be roaming interface and the existing service operations with the V-NSSF.

The Figure 1 shows the reference architecture which can be applied to use the NSSF for determining the Alternative S-NSSAI in roaming case. The AMF is connected with the NSSF, in roaming case with the visited NSSF (e.g. V-NSSF), via the N22 reference point. The V-NSSF is connected with the NSSF in the home PLMN (e.g. H-NSSF) via the N31 reference point. The H-NSSF may be connected with the network data analytics function (NWDAF) in the HPLMN via the N34 reference point.

Figure 1:  Reference architecture for the NSSF in roaming case

The signalling flow, which can apply for such roaming case is shown in Figure 2 steps 5 – 9, i.e. scenario B.

The NSSF currently supports the following functionality (according to clause 6.2.14 from TS 23.501):

-	Selecting the set of Network Slice instances serving the UE;

-	Determining the Allowed NSSAI and, if needed, the mapping to the Subscribed S-NSSAIs;

-	Determining the Configured NSSAI and, if needed, the mapping to the Subscribed S-NSSAIs;

-	Determining the AMF Set to be used to serve the UE, or, based on configuration, a list of candidate AMF(s), possibly by querying the NRF;

-	The NSSF may provide support for Network Slice restriction and Network Slice instance restriction based on NWDAF analytics.

In order to support the conclusions from KI#1, the NSSF functionality can be enhanced with the following high-level principles (please note that the detailed description is provided in Figure 2):

-	Determining that an (old) S-NSSAI has to be replaced (according to proposal 1).

NOTE 1:	This may be triggered by the OAM system (e.g. when a NSI is to be decommissioned or when the NSI doesn’t fulfil the SLAs/KPIs) or by NWDAF analytics (e.g. about the overload or congestion in user plane or in control plane).

NOTE 2: Such possible triggers to the NSSF are shown for information in step 2 and 7 in the Figure 2.

-	Determining one or more Alternative S-NSSAIs which can replace the (old) S-NSSAI (according to proposal 1).

NOTE 3:	The determination may be based on information received by the OAM, or by selecting potential S-NSSAIs with similar characteristics locally configured in the NSSF, and/or considering the NWDAF analytics about the load of the potentials alternative S-NSSAIs.

-	A consumer NF (e.g. AMF, V-NSSF) subscribes for notification event whether an S-NSSAI (out of a set of S-NSSAIs) needs to be replaced by the alternative S-NSSAI. Correspondingly, the NSSF sends a notification to the consumer NF (e.g. AMF, V-NSSF) to indicate that an S-NSSAI needs to be replaced by the alternative S-NSSAI(s).

NOTE 4:	Such signalling is exemplified in Figure 2:

-	in steps 1 and 3 in non-roaming case or in roaming case ; and

-	in steps 5, 6, 8, 9 in roaming case for HPLMN S-NSSAI replacement. The AMF when sending Nnssf_NSSAIAvailability_Subscribe request can already include the HPLMN ID, so that the V-NSSF can resolve the H-NSSF.

-	A consumer NF (e.g. AMF, V-NSSF) requests the NSSF to provide an Alternative S-NSSAI for a specific (old) S-NSSAI. Such signallling is shown in Figure 2 in steps 11 – 13.

NOTE 5:	Such signalling is exemplified in Figure 2:

Based on the above

Proposal 3:	the existing services Nnssf_NSSAIAvailability_Subscribe and Nnssf_NSSAIAvailability_Notify are re-used and enhanced to include notification for the replacement of an S-NSSAI with an Alternative S-NSSAI.

NOTE 6:	The Nnssf_NSSAIAvailability Subscribe/Notify service may needs to be enhanced for roaming case, considering that the V-AMF can include the PLMN ID in the Nnssf_NSSAIAvailability_Subscribe request, so that the V-NSSF can resolve the H-NSSF.

Proposal 4:	the existing services Nnssf_NSSelection_Get is re-used and enhanced with a new indication for requesting an Alternative S-NSSAI for a specific (old) S-NSSAI.

# 2.	Detailed signalling flow

The Figure 2 shows a signalling flow how the NSSF can trigger the AMF to perform S-NSSAI replacement procedure. The Figure 2 is shown for information, it is self-explanatory and will not be described step-by-step. The Figure 2 shows three scenarios:

Scenario A:	the replacement of the S-NSSAI with Alternative S-NSSAI which is triggered/determined in the NSSF of the serving PLMN: this is applicable to non-roaming case and roaming case with LBO. An example signalling flow in shown in the upper part of Figure 2 in the steps 1 – 4.

Scenario B:	the replacement of the HPLMN S-NSSAI with Alternative HPLMN S-NSSAI triggere/determined in the hNSSF and signalled to the vNSSF. This applies to roaming case with HR PDU Sessions. An example signalling flow in shown in the lower part of Figure 2 in the steps 5 – 10.

Scenario C:	the AMF queries the NSSF to provide an Alternative S-NSSAI to a specific S-NSSAI.

The service operations Nnssf_NSSAIAvailability_Subscribe and Nnssf_NSSAIAvailability_Notify are available already and can be re-used in the exchange between the AMF and V-NSSF, as well as between V-NSSF and H-NSSF.

A new event ID needs to be introduced to subscribe for replacement for any S-NSSAI served by the AMF, as shown in steps 1 and [5 + 6].  Correspondingly, the NSSF can notify the NF consumer in steps 3 and [8 + 9]. The notification from the NSSF to the NF consumer in can have different values:  e.g. start/trigger an S-NSSAI replacement and stop an S-NSSAI replacement (the latter shown in grey text in the steps 3 and 8).

The procedure at the bottom of Figure 2 (steps 11 – 13) shows how the AMF may use the Nnssf_NSSelection_Get request operation to request from the NSSF an Alternative S-NSSAI or NSI. The query of NSI is already available in the existing Nnssf Get service as described in step 4 in clause 6.15.3 of TR 23.700-41. So,

Figure 2:  Signalling flow for NSSF triggering S-NSSAI replacement in non-roaming and roaming cases.

# 2. Proposal

For the case where the NSSF determines the Alternative S-NSSAI, the following proposal should be documented in the normative phase:

Proposal 1:	the NSSF has system-level overview of the deployment of the old S-NSSAI and can determine one or more Alternative S-NSSAIs having the same area coverage and similar characteristics. The NSSF provides the Alternative S-NSSAI(s) to the AMFs serving the old S-NSSAI.

Proposal 2:	When receiving the Alternative S-NSSAI(s), the AMF determines a single Alternative S-NSSAI applicable for each UE which is currently registered to the old S-NSSAI.

Proposal 3:  the existing services Nnssf_NSSAIAvailability_Subscribe and Nnssf_NSSAIAvailability_Notify are re-used and enhanced to include notification for the replacement of an S-NSSAI with an Alternative S-NSSAI.

Proposal 4:  the existing services Nnssf_NSSelection_Get is re-used and enhanced with a new indication for requesting an Alternative S-NSSAI for a specific S-NSSAI.