3GPP TSG-SA2 Meeting #155	S2-2303072

# Athens, Greece, February 20th – 24th, 2023								(revision of S2-2302002)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0604 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support the Maintenance of Federated Learning Process in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson], Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Resolution of EN “The details of the judgement process is FFS”: Server NWDAF is enabled to solicit training status from client NWDAF which failed to send the report by the reporting time window. Principle 5 and NOTE 2 listed in the conclusion in clause 8.8 of TR 23.700-81 as follows: “During the FL procedure, the NWDAF containing MTLF as FL server may trigger reselection, addition or removal of FL clients and may issue a new FL client discovery via NRF, based on local policy or status of FL clients, e.g. load, availability, capability, latency, accuracy, etc. FL clients can join or quit FL operation dynamically in the execution phase.” “NWDAF containing MTLF as FL Server can monitor the accuracy level of the trained model and exclude a Client NWDAF from the FL group …” Based on these conclusions, the procedures to support maintenance of Federated Learning in FL execution phase are added to TS 23.288, including FL server triggers reselection, addition or removal of FL client(s), discovers FL client(s) via NRF, and FL client joins or quits FL operation dynamically. Revision 4 adds the support for Server NWDAF to solicit FL status to Client NWDAF which did not send interim training information by the indicated time, and generate an FL report including information of number of clients participating in the FL process, number of iterations, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure to TS 23.288 to support FL server to reselect FL client(s), discover FL client(s) via NRF, and to support FL client to join or quit Federated Learning operation dynamically. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for support maintenance of Federated Learning process in FL execution phase will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new) * * *

#### 6.2C.2.Y Procedures for Maintenance of Federated Learning Process

This clause specifies how to maintain a Federated Learning process in FL execution phase, including Server NWDAF triggering reselection, addition or removal of Client NWDAF(s), discovering new Client NWDAF(s) via NRF, and Client NWDAF(s) joining or quitting Federated Learning operation dynamically.

In Federated Learning execution phase, Server NWDAF monitors the status changes of Client NWDAF(s), and may reselect Client NWDAF(s) based on their updated status, availability, and/or capability, etc.

NOTE 1: Server NWDAF checks if there is a need to carry on the FL execution phase and then reselects FL members for the next iteration if needed.

Figure 6.2C.2.Y-1: Procedure of Server NWDAF reselecting Client NWDAF(s), Client NWDAF(s) joining or quitting Federated Learning operation dynamically in Federated Learning execution phase

The procedure for Server NWDAF reselecting Client NWDAF(s), Client NWDAF(s) joining or quitting Federated Learning operation dynamically is as follows:

0.	Server NWDAF registers into NRF about the Federated Learning procedure with the following parameters:

-	Analytics ID.

1a.	Server NWDAF may get the updated status of current Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502) in the Federated Learning execution phase.

Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the Server NWDAF the status changes of the current Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current Client NWDAF could be availability changes, capability changes (e.g., it will not support FL anymore, etc.).

1b. The current Client NWDAF(s) may inform Server NWDAF to quit the Federated Learning operation.

1c. Server NWDAF may get the information of the new Client NWDAF(s) dynamically via NRF by subscribing to the event that a new NWDAF registers itself as Client NWDAF (Nnrf_NFManagement _NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502).

1d. Server NWDAF may subscribe for NF load analytics of the Client NWDAF(s).

1e. Client NWDAF(s) may report interim local ML model information before the maximum response time, including accuracy of local model, characteristics of local dataset, etc.

2.	If a Client NWDAF does not report interim local ML model information before the maximum response time, the Server NWDAF may send an FL status request including the expected time units for local training to complete, expected degree (low, medium, high) of delay in interim local ML model information, etc.

3.	The Client NWDAF sends the FL status response to Server NWDAF indicating the time units needed to complete the local training and to send the local training report.

4.	Based on the FL status response in step 3, the Server NWDAF may send an extended maximum response time to Client NWDAF, before which it has to send the local training report. Otherwise, the Server NWDAF may indicate Client NWDAF to skip reporting for this iteration.

5.	Server NWDAF checks Client NWDAF(s) status based on the received information, judges whether reselection of Client NWDAF(s) for the next roundof Federated Learning is needed. The judgement is based on the updated status of the Client NWDAF(s), including the availability, capability, status of FL training, how many times interim local ML model information was not sent in time and the average delay, and etc. The Server NWDAF may assign a confidence score to the Client NWDAF, which is used to determine whether to re-select a Client NWDAF.

6.	[If re-selection is needed as judged in step 2.] If step 1c is not performed, Server NWDAF may discover new candidate Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502. Server NWDAF reselects Client NWDAF(s) from the current Client NWDAF(s) and the new candidate Client NWDAF(s) (found in steps 1c or 3). For the new candidate Client NWDAF(s), the interaction between Server NWDAF and Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1.

7.	Server NWDAF sends termination request to the Client NWDAF(s) which will be removed from the Federated Learning process, optionally indicating the reason. Client NWDAF(s) terminate(s) Federated Learning operations if received termination request from the Server NWDAF and may perform further action to be qualified to participation in FL training in the next cycles.

Editor’s Note: The further actions by Client NWDAF is FFS.

Editor’s Note: What service will be used by Server and Client NWDAFs to terminate Federated Learning operation is FFS.

8.	After N iterations, the Server NWDAF may store an FL training report and optionally confidence scores of each Client NWDAF in the ADRF. The FL training report contains information about the FL training process, such as duration, number of iterations, min and max number of Client NWDAFs, number of iterations for each Client NWDAF, number of times a Client NWDAF joined and leaved the FL group, average interim ML model information report time, etc.

* * * End of Changes * * *