3GPP TSG-SA2 Meeting #156E	S2-2304567

# April   17 - 21, 2023, Elbonia										 								(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0097 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ground-based DAA for an area enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A2X adaptations Procedures needs to be described.Policy for allowed PLMNs shall be on the A2X level and not per individual service type. Clarification of Editor’s note: The AAM will using mechanisms outside of 3GPP scope detect the location of the UAVs within the area and based on that request corresponding NRID information from a RID-USS etc using a data network connection. How the data network connection is implemented is outside of 3GPP scope but can be either fixed or mobile. The rights needed for the AAM to access the RID-USS and retrieve the Remote-ID is determined maintained by the UTM outside the scope of 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adapted to show that A2X services are used. Policy for Authorization of allowed PLMNs for the specific service removed Procedures for service added Editors note removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7, 5.7.1, 5.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.7	Ground-based DAA for an Area

### 5.7.1	Functional Description

This clause provides a network-assisted (ground based) DAA mechanism. It is applicable for a specific area, such as a stadium or arena where UAVs are used e.g. for filming an event. It is based on a ground-based entity, Area Airspace Manager (AAM), that is able to detect UAVs in the specific area and provide local steering policies to the individual UASes in order to avoid the UAVs crashing into each other or different structures etc. The policy may e.g. include allowed flying zones and positions allowed for the specific UAV. The policy may also apply to a specific outdoor area, e.g. an event, where specific measures to avoid collision between drones are established locally.

NOTE 1:	The policies provided by the AAM is application specific steering/DAA policies that are out of scope of 3GPP. The same or different local policies can be provided to the UAVs in the arena. UAVs can, for example, be given different fly zones in the arena just like people have different seating in an arena.

The high-level principles of the network-assisted (ground based) DAA are:

-	The arena/area has a ground-based entity Area Airspace Manager (AAM). The AAM includes one or more UEs enabled for use of PC5. The AAM may also have a direct connection to the Data Network.

-	For the applicable airspace of the area/arena the AAM defines individually adapted local collision avoidance rules for correspondingly located UAVs.

NOTE 2:	How the AAM can determine the local steering policy for collision avoidance is out of scope of 3GPP. It may, e.g., be based on maps of the area/arena  and detailed information about the facilities together with an awareness of the current usage need for the airspace.

-		Provisioning of AAM local collision avoidance rules to a UAV/UE must comply with the policies for PC5 operations received from the 5GC or being preconfigured in the UE.

-	Detected UAVs are identified by their coordinates and by the Remote ID as retrieved by Broadcast Remote ID (BRID) or Network Remote ID (NRID) mechanisms dependent on the method used by the UAV.

NOTE 3:	How the AAM can scan the UAVs in specific area is out of scope of 3GPP.

NOTE 4:	How the AAM retrieves the Remote ID for detected UAVs is out of scope of 3GPP. For example, in case of BRID different UAVs can use different methods, such as WLAN, Blutooth or PC5 direct communication, for broadcating the Remote ID and in case of NRID the AAM will, based on the geolocation of the UAV, retrieve the Remote ID from the USS at application layer. For the NRID case the AAM will typically have to be authorized by the UTM to be allowed to retrieve the Remote ID.

-	Based on the retrieved Remote ID, the AAM activates PC5 communication with each detected UAV by triggering establishment of a unicast link based on procedures in 23.287 [X] clause 6.3.3 using the UAV's Remote-ID as Application-Layer-ID (i.e. Target User Info) in the Direct Communication Request message.

For Direct Communication over PC5 the UAV and AAM shall comply with the authorization and provisioning principles described in clause 4.2.1.2.2 including the following considerations:

-	The default destination Layer-2 ID to be used for initial signalling to establish a unicast connection for the A2X service.

-	Parameters for Groupcast are not applicable.

-	Both NR and LTE PC5 are supported.

NOTE 5: LTE PC5 can be used to broadcast DAA messages by UAVs.

-	The AAM uses PC5 unicast link to provide each UAV present in the arena/area with local DAA policies as user traffic.

-	A UAV that receives local collision avoidance related policies from the AAM over PC5 may forward the policies to its UAV-C so that the UAV-C can steer the UAV accordingly by use of C2 communication (e.g. over Uu, PC5, or other means) in order to enforce the local policies and avoid collisions.

NOTE 6:	Whether and how to forward the policies from UAV to its UAV-C and how UAV-C uses the policies is out of scope of 3GPP.NOTE 7:	A UAV-C that receives local collision avoidance policies from an AAM, can inform the AAM about its network address (e.g. IP address or URL) in order to enable direct AAM to UAV-C communication via Data Network. How a UAV-C communicates with the AAM is out of scope of 3GPP.

Figure 5.7.1-1: Logical architecture for ground based DAA

### 5.7.2	Procedures

Figure 5.7.2-1: High-level procedure for Ground-based DAA for Arena

Prerequisites: The AAM/UE and the UAVs/UEs are configured to use the ProSe service AAM-UAS-Influence.

- The UAVs/UEs listens for signals on the correspondingly destination Layer-2 ID configured for the used service type in accordance with chapter 4.2.1.2.2.

- The AAM can scan by e.g., making use of ward pointing radars and cameras etc. the airspace over the area/arena for UAVs. For each detected UAV it determines the coordinates. 
How this is done is out of 3GPP scope.

- The AAM retrieves for each detected UAV the corresponding Remote-ID using the method applicable for the specific UAV. Methods which may be used includes broadcast of Remote-ID via PC5, WiFi and Bluetooth, and also Network Remote ID.  
How this is done is out of the 3GPP scope.

- For each detected UAV/UE the AAM establishes a PC5 direct communication link with the discovered UAV for AAM to UAS interaction by performing the Layer-2 link establishment procedure as described when Target User Info is included in clause 6.3.3.1 of 3GPP TS 23.287 (Fig 6.3.3.1-1 step 2, 3 4a, 5a and 6). The AAM application layer provides a service type indicating the A2X service “Ground-based DAA for an area” and the retrieved Remote-ID as target UE’s Application Layer ID. As a result of this procedure a PC5 unicast direct communication link enabling bidirection data exchange is set up between the application layer in the AAM and the UAV application layer of the UAV/UE having the specified Remote-ID.

- Optionally the UAV/UE may, when the PC5 unicast direct communication link has been set up for NR, activate a corresponding bidirectional tunnel function extending the link to the UAV-C  e.g. via the communication means in use for C2 communcation. This enables packets received on the link (i.e. from the AAM) to be forwarded to the UAV-C and packets received from the UAV-C to be forwarded on the link (i.e. towards the AAM). Implementation aspects for this tunneling function are out of scope for 3GPP.

- Using the PC5 unicast direct communication link the AAM and the UAV establishes a bidirectional communication channel for exchange of messages. Optionally this channel can be extended to involve the UAV-C using the bi-directional tunnel. The protocol and implementation aspects for this step are out of scope for 3GPP.

- The UAV-C and/or the UAV may optionally and out of scope for 3GPP use the communication channel to provide application specific information to the AAM.

- Based on awareness of the current UAV traffic situation, local info and plans for the area/arena, the AAM determines locations and flight paths appropriate for the specific UAV such that collisions can be avoided and creates a corresponding application specific steering policy being out of scope of 3GPP.

- The UAM provides the determined steering policy to the specific UAV using the activated communication channel. Optionally it can be forwarded to the UAV-C which can return corresponding C2 commands. Protocols, semantics and syntax for handling this are application specific and out of 3GPP scope.

- The UAV is in accordance with received policy and using mechanisms that are out of scope for 3GPP steered to approved locations while avoiding collisions.

NOTE: 	A UAV-C that optionally receives local policies for DAA from an AAM, can inform the AAM of its address (e.g. IP address or URL) to enable direct AAM to UAV-C communication via Data Network.

* * * End of Changes * * *