3GPP TSG-WG SA2 Meeting #158	S2-2308415

# 21 - 25 August, 2023, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4256 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct PFD management based on PFD Determination analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 4.18.2.2 not complete PFD information related descriptions will arouse wrong perception, step1 missing Nnwdaf_AnalyticsInfo_Response service operation, step 2 may be optional if the PFD data is already available in NEF (PFDF) and step 4 may also be optional if the NEF(PFDF) determine not to create/update/delete PFD information, that is dot line figure should be used in step 2 and step 4 and correspding procedure description should be updated to be clear. And step 4 clause 4.18.2 needs to be corrected as clause 4.18.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 4.18.2.2 correct "PFD information subscribed from the NWDAF" as PFD information notified/responded from the NWDAF, also correct the figure and procedure description in step1, step 2 and step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and not complete NEF(PFDF) handling on PFD determination analytics will arouse wrong stage 3 implementation and wrong deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.18.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.18.2.2	PFD management based on PFD Determination analytics

Figure 4.18.2.2-1 shows the procedure that NEF (PFDF) determines the PFD information for the known Application Identifier(s), based on the PFD Determination analytics information notified/responded from the subscribed/requested NWDAF. The procedure enables the NEF(PFDF) to determine whether to create/update/delete PFD information corresponding to the known Application Identifier(s).

Figure 4.18.2.2-1: Procedure for PFD management based on PFD Determination analytics

1.	The NWDAF notifies/responds to PFD determination analytics to the NEF(PFDF) as Consumer NF with PFD Information defined in clause 6.16.4 of TS 23.288 [50].

2.	The NEF (PFDF) fetches the PFD information currently in use from UDR if not available in NEF (PFDF), as described from step 2 to step 3 of clause 4.18.3.1.

3.	The NEF (PFDF) compares the PDF information from UDR with PFD information from the NWDAF to determine whether to create/update/delete PFD information corresponding to the Application Identifier.

4.	If determined in step 3 to create/update/delete PFD information corresponding to the Application Identifier, the NEF (PFDF) invokes the Nudr_DM_Create/Update/Delete (Application Identifier, one or more sets of PFDs) to the UDR to create/update/delete PFD information corresponding to the Application Identifier, i.e. from step 3 to step 5 of clause 4.18.2.1 are executed. The NEF(PFDF) may forward new/updated/deleted PFD information to UPF via SMF to detect a known application described in clause 4.4.3.5.

*** End of Changes ***