SA WG2 Meeting #158	S2-2308816

Goteborg, Sweden, August 21 – 25, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1120 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Not reporting URSP rule enforcement other then connection capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-85, it is agreed that the UE reports the URSP rule enforcement to 5GC via NAS message, and only the connection capability that enforced should be reported to 5GC. But for some of the URSP rule, it contains a Traffic descriptor (containing one or more components) and this URSP rule is determined to be applicable when every component in the Traffic descriptor matches the corresponding information from the application. So, if a URSP rule is enforced, and the traffic descriptor in URSP rule is the combination of Connection Capabilities and other TD, that the UE can only report the Connection Capabilities to 5GC. Other components in this enforced URSP rule is not reported. Just like the example in Annex A in TS 23.503, if the traffic descriptor in a URSP rule containing “Traffic Descriptor: Application descriptor=App1” and “Connection Capabilities="internet",”, after this URSP rule is enforced by UE, the UE only reports the Connection Capabilities="internet" to 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a URSP rule in UE is enforced, and the traffic descriptor in this URSP rule containing one or more components, that the UE can only report the Connection Capabilities to 5GC and other components in this enforced URSP rule is not reported to 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will report all of the component in enforced URSP rule to network, but except the Connection Capability, some of other kinds of TD should not be reported to network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.6.2.4	Support of URSP rule enforcement reporting

This clause defines how and under what conditions the UE reports URSP rule enforcement to PCF so that PCF can be made aware when a given UE enforces specific URSP rule(s) and what actions the PCF may trigger upon the reception of such reporting.

In order to determine the URSP rule enforcement, for a UE indicating the capability of reporting URSP rule enforcement to network (see clause 4.2.2.2.2 of TS 23.502 [3]), the PCF may indicate in a URSP rule sent to the UE to send reporting of URSP rule enforcement (see clause 6.6.2.1).

A UE supporting URSP rule enforcement reporting shall report URSP rule enforcement to the SMF if a URSP rule includes an indication for reporting URSP rule enforcement and if Connection Capabilities is in the TD (see clause 6.6.2.1), when:

-	the UE associates a newly detected application to a new PDU Session based on URSP evaluation result (see clause 6.6.2.3) for such a URSP rule, the UE shall include in the PDU Session Establishment Request (see clause 4.3.2.2.1 of TS 23.502 [3]) the Connection Capabilities contained in the Traffic descriptor of the associated URSP rule, or

-	the UE associates a newly detected application to an existing PDU Session based on URSP evaluation result (see clause 6.6.2.3) for such a URSP rule, the UE shall send a PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) including the Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

NOTE 1:	UE reporting of URSP rule enforcement can increase the amount of signalling in the network. Use of this feature is recommended to be restricted to URSP rules for specific application traffic on specific UEs based on the deployment choices of the operator.

If the UE enforces several URSP rules for multiple applications, and these multiple applications' traffic are all associated to this PDU session, in order to reduce the number of uplink NAS messages, the UE may include more than one URSP rule enforcement report in one PDU Session Modification Request to 5GC (see clause 4.3.3.2 of TS 23.502 [3]).

NOTE 2:	A rule with the "match-all" traffic descriptor cannot contain Connection Capabilities in the Traffic descriptor. The format and values of the Traffic descriptor component type identifier are defined in clause 5.2 of TS 24.526 [19].

NOTE 3:	If a URSP rule that contains a Traffic descriptor with two or more components, a combination of Connection Capability and other Traffic Descriptor, is enforced by UE, only the Connection Capability shall be reported to network, other Traffic Descriptor is not reported.

The PCF receives reporting from URSP rule enforcement for a given UE via Policy Control Request Triggers (see clause 6.1.3.5).

When the PCF serving the PDU session is not the same as the PCF serving the UE, the PCF serving the UE subscribes to the PCF serving the PDU session to receive the reporting of URSP rule enforcement for a given UE via PCF event reporting (see clause 6.1.3.18 and the related procedure in clause 4.16.16 of TS 23.502 [3]).

For LBO roaming session case, the H-PCF for the UE sends the PCRT for the UE reporting of URSP rule enforcement to the V-PCF for the UE during the UE Policy Association Establishment or Modification.

The PCF for the UE may check whether the value of URSP rule enforcement and its PDU Session parameters (e.g. DNN/S-NSSAI) are compliant to the URSP rule of the UE. If the PCF for the UE found an inconsistency, the PCF for the UE may perform appropriate actions (e.g. initiating slice replacement procedure).

Policy control decisions based on awareness of URSP rule enforcement are described in clause 6.1.6.

* * * End of Change * * *