3GPP TSG-SA2 Meeting #154	S2-2308672

# November 14 – November 18, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4311 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on assistance for member UE selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR provides a series of corrections to the member UE selection framework and specific solutions. The reason for change is the need for an increase of clarity to the specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifications regarding the update of the filtering criteria without changing the initial list of target UEs - Corrections to the text in the list of filtering criteria: replace ‘selected UE’ with ‘candidate UE’, use of ‘NF service’ instead of ‘service operation’, clarification regarding the shown parameters as some may not be provided as filtering information but still be service operation input parameters, other minor editorials - Correction in 4.15.13.4 to clarify that Relative Proximity is an analytics ID - Correction in 5.2.6.32.4 not to restrict the filtered out UEs to more than one filter as it can also be useful when there’s only one filtering criterion provided - Miscellaneous editorial corrections throughout 4.15.13.0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 4.15.13.1, 4.15.13.2, 4.15.13.3.1, 4.15.13.3.1, 4.15.13.3.3, 4.15.13.4, 4.15.13.5.2, 4.15.13.6.1, 5.2.6.32.1, 5.2.6.32.25.2.6.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

#### 4.15.13.0	General

AF provides a list of target UE(s) with the initial subscription request and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve from 5GC NFs the information for the UE in the list of target UE(s). Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF. The AF may update the filtering criteria subsequently and notify the NEF. In that case, the AF does not need to include again the list of target UEs in the subscription request but instead includes a Subscription Correlation ID as indicated in clause 5.2.6.32.2.

The Member UE selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how the AF leverages the 5GC network exposure for Member UE selection is described in the informative Annex I.

#### 4.15.13.1	Member UE selection general information flow

This clause describes the procedures that are generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to Member UE selection and update

1.	AF subscribes to the Member UE selection assistance functionality by sending a first Nnef_MemberUESelectionAssistance_subscribe request including a list of target UE(s), one or more member UE filtering criteria as listed in Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may update the filtering criteria of the subscription by invoking Nnef_UEMemberSelectionAssistance_subscribe and providing a Subscription Correlation ID instead of the list of target UEs.

2a.	[OPTIONAL] NEF verifies the authorization of the AF Request and identifies which information needs to be collected for each UE in the list of target UE(s) and executes the corresponding service operations based on the Member UE filtering criteria provided by the AF, e.g. events, analytics ID(s),  notifications, etc..

2b.	[OPTIONAL] The NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the target UEs received in step 1 for the Member UE update using the updated filtering criteria.

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target UE(s). The interactions between the NEF and the 5GC NFs depend on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, the NEF consolidates all the information to derive the list(s) of candidate UE(s) based on the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s) considering the validity period(s) of the analytics ID used as Member UE selection criteria, which may be a subset of the time window(s) received from the AF.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information. See clause 5.2.6.32.4 for details.

#### 4.15.13.2	Member UE Filtering Criteria for 5GS assistance to Member UE selection

Table 4.15.13.2-1 provides a summary of the Member UE filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of Member UE filtering criteria

| Member UE filtering criteria | Description of filtering criteria | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the candidate UEs match or exceed the QoS of the filtering criteria | NF service: Nsmf_EventExposure or Nudm_EvenExposure, Filter/service operation input parameters: target=SUPI, Event=QoS Monitoring, traffic descriptor (e.g. Application ID) | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the candidate UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | NF service: Nsmf_EventExposure Filter/service operation input parameters: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI, Event ID=Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE and AF, e.g. the average and variance of End-to-end data volume transfer time | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter/service operation input parameters: Analytics ID = End-to-end data volume transfer time, target of Analytics reporting = GPSI(s) or SUPI(s) | 4.15.13.6 |
| UE current location | Indicate the certain area that the candidate UE should be currently located in. | NF service: Namf_EventExposure Filter/service operation input parameters: a list of GPSI(s) or SUPI(s) | 4.15.13.4 |
| UE historical location | Indicate the certain area that the candidate UE appeared in a historical period of time | NF service: Nnwdaf_AnalyticsSubscription Filter/service operation input parameters: Analytics ID = UE mobility, Analytics Filter information include "Visited AoI = Target AOI" and "Analytics target period = historical nomadic period” | 4.15.13.4 |
| UE direction | Indicate the candidate UEs should include different moving directions | NF service: Nnwdaf_AnalyticsSubscription Filter/service operation input parameters: Analytics ID= UE Mobility | 4.15.13.4 |
| UE separation distance (NOTE 1) | Indicate the candidate UEs should comply with a minimum separation distance between each other | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter/service operation input parameters: Analytics ID = Relative Proximity, Analytics Filter Information include "Proximity Attributes" | 4.15.13.4 |
| Service Experience | Indicates candidate UEs fulfilling certain Service Experience criteria e.g., MOS value | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter/service operation input parameters: Analytics ID=Service Experience, Analytics Filter Information= “S-NSSAI, DNN, Application ID, DNAI, AoI”, Service Experience Contribution weight reporting threshold (NOTE 2), Service Experience Type (NOTE 3) | 4.15.13.5 |
| DNN | Indicate the DNN of the candidate UE for the PDU Session used by the application | NF service: Nsmf_EventExposure Filter/service operation input parameters: a list of GPSI(s) or SUPI(s), Event ID=QFI allocation | 5.2.8.3 |
| NOTE 1: This criterion should only be applied when the number of UEs is in the range of 10's or less. NOTE 2: The Service Experience Contribution Weights signal the relative importance of each UE's Service Experience value (i.e. MOS), as defined in TS 23.288 [50]. For example, it might be that the service experience of a UE in relation to other UEs may not be as important e.g. because the data provided by such UE is not as critical to the service. NOTE 3: Indicates the type of service experience analytics, e.g. AI/ML traffic where a customized MoS apply. |  |  |  |

* * *Next Change * * *

##### 4.15.13.3.1	General

An AF may invoke Nnef_MemberUESelectionAssistance_Subscribe service operation with  QoS filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	QoS filtering criteria.

-	Optionally, an Area of Interest: desired location area of the candidate UEs.

The QoS filtering criteria includes:

-	a traffic descriptor (e.g. Application ID).

And one or more of the QoS parameters subject to QoS monitoring (see list in clause 5.45 of TS 23.501 [2]).

The determination of the list of UEs that match or exceed the QoS filtering criteria and are optionally located in the AoI in real time, for the duration of the subscription, is further described in clause 4.15.13.3.3.

* * *Next Change * * *

##### 4.15.13.3.3	Member UE Assistance with QoS filtering criteria for real-time QoS Monitoring

At the reception of Nnef_MemberUESelectionAsistance_Subscribe request of step 1, NEF determines to do real-time QoS monitoring for determining the list of UEs that match or exceeds the QoS filtering criteria in real time. Unless the QoS flow to be monitored is associated with a default QoS flow, QoS Monitoring must have been activated in SMF for that QoS Flow before (e.g. by AF Session with required QoS) (see clause 4.15.4.5.1 for details).

Figure 4.15.13.3.3-1: 5GC assistance to Member UE selection for real-time QoS monitoring

1.	AF subscribes to the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including its AF ID, the Application ID, optionally S NSSAI/DNN and the QoS filtering criteria.

2.	NEF uses the AF ID to verify the authorization of the AF request and identifies which information needs to be collected and executed based on the QoS filtering criteria provided by the AF.

3.	If S-NSSAI/DNN are not included in the request from the AF, the NEF derives the S-NSSAI and DNN which this Application ID has access to. NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination.

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: 'QoS monitoring', target UEs, optional a traffic descriptor (e.g. Application ID), optional indication for default QoS flows monitoring, S-NSSAI, DNN, Notification Target Address set to NEF, etc.). Alternatively, NEF may send an Nudm_EventExposure_Subscribe request to the UDM (not shown in Figure 4.15.13.3.3-1).

5.	The SMF may need to send an N4 Session Modification request to UPF for requesting the QoS monitoring for certain flows.

6.	UPF sends an Nupf_EventExposure_Notify request to NEF including an Event Exposure notification, according to the subscription received from SMF.

7.	Based on the Event Exposure reports received from the UPFs, NEF consolidates the received results and derives the list(s) of candidate UE(s) and additional information based on the QoS filtering criteria requested by the AF request.

8.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

#### 4.15.13.4	Specific procedure for the 5GC assistance to member UE selection based on the UE's current location, historical location and direction and UE separation distance

Figure 4.15.13.4-1: 5GC assistance to Member UE selection based on the UE's current and historical location, direction and separation distance

1.	AF requests 5GS assistance to support the Member UE selection by considering the UE's historical location, UE's current location, direction and UE separation distance. AF includes the UE lists and the following criteria as part of the member UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	UE Direction: Select the UE with the different direction in the FL coverage Area.

-	UE separation distance: Select UEs that are geographically separated, fulfilling the separation distance no smaller than the certain separation distance.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4.	The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility/Relative Proximity, AOI = Target AOI) with UE list received from step 4.

6-7.	The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Relative Proximity, Filters include "Visited AoI = Target AOI", "target period = historical nomadic period" and "proximity attributes"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period and NWDAF also provides a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility, the UE's direction and distance between UEs in the group/list.

8-9.	Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction and separated from each other fulfilling the minimum distance threshold as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify the AF of the given UE which is moving out the coverage.

* * *Next Change * * *

##### 4.15.13.5.2	Member UE Assistance with Service Experience filtering criteria

Figure 4.15.13.5.2-1: 5GC assistance to Member UE selection based on Service Experience

1.	AF subscribes to the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including the Application Identity, AoI, DNAI(s), Service Experience Type and contribution weights associated to location, time window, Application ID and Service Experience type.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the Service Experience filtering criteria provided by the AF.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI, DNN and DNAI(s) to which this Application ID have access. Due to the absence of a time window in the request from the AF, NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application Id or AF Id).

4-8.	NEF sends an Analytics request/subscribe to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe, including Analytics ID = Service Experience, Application ID, S-NSSAI, AoI, DNAI(s), and target UEs based on the initial list obtained from the AF. Then, procedures as specified in clause 6.4.6 of TS 23.288 [50] are followed.

9.	Based on the Analytics report received from the NWDAF, NEF consolidates results and derives the list(s) of candidate UE(s). The NEF may use the Service Experience type provided by the AF consumer in the filtering information and use operator policies to interpret a customized MoS. The NEF may use the contribution weight associated to an application and a Service Experience Type (e.g. AI/ML traffic) and apply them to a location and time window, as provided by the AF consumer, to be used as reporting thresholds when selecting candidate Member UEs, e.g. the NEF may select a UE with a specific Service Experience Type as a Member UE candidate, if the associated Service Experience fulfils the threshold for Service Experience filtering criteria provided by the AF.

10.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * *Next Change * * *

##### 4.15.13.6.1	General

An AF may invoke Nnef_UEMemberSelectionAssistance_Subscribe service operation with end-to-end data volume transfer time related filtering criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	End-to-end data volume transfer time filtering criteria: this may include the average end-to-end data volume transfer time for a specific data volume between UE and AF and/or the variance of the end-to-end data volume transfer time.

-	Optionally, an Area of Interest: location area of the candidate UEs.

-	Optional time windows for selecting the candidate UEs: start time and stop time.

* * *Next Change * * *

##### 5.2.6.32.1	General

Service description: The service provides the consumer with the capability to subscribe or unsubscribe the Member UE selection assistance information from the NEF or notify the Member UE selection assistance information by the NEF to the consumer. See Table 4.15.13.2-1 for the Member UE filtering criteria.

##### 5.2.6.32.2	Nnef_MemberUESelectionAssistance_Subscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Subscribe

Description: The NF consumer subscribes to receive the Member UE selection assistance information, or the subscription is updated if the same subscription is already defined in NEF.

Inputs, Required: Notification Target Address (+ Notification Correlation ID), at least one filtering criterion shown in Table 4.15.13.2-1.

Inputs, Conditional Required:

If no Subscription Correlation ID is provided in the subscription, Target of Member UE Selection Assistance Reporting (GPSI or a list of GPSIs) is required.

Inputs, Optional: Application ID, Subscription Correlation ID (in the case of update of the existing subscription), Expiry time, additional Member UE filtering criteria as shown in Table 4.15.13.2-1, time window(s) for selecting the candidate UEs, specific parameters depending on the Member UE filtering criteria as shown in Table 4.15.13.2-1, Periodicity (the periodicity of member update), maximum number of candidate UEs.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

* * *Next Change * * *

##### 5.2.6.32.4	Nnef_MemberUESelectionAssistance_Notify service operation

Service operation name: Nnef_MemberUESelectionAssistance_Notify

Description: NEF reports the Member UE selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information.

Inputs, Conditional Required:

At least one of the following inputs is required:

-	One or more list(s) of candidate UE(s).

Inputs, Optional: Recommended time window for performing the application operation per list of candidate UE(s) as described in clause 4.15.13.1, specific values of the parameters that gathered by the NEF for the Member UE filtering criteria per candidate UE,

a number for each filtering criterion that indicates the UEs in the initial list that do not meet the corresponding filtering criteria .

NOTE:	The number per filtering criterion can be an indication for AF to revise the corresponding filtering criterion.

Outputs, Required: Operation execution result indication.

* * *End of Changes * * *