3GPP TSG-SA WG2 Meeting #158	S2-2308986

# Goteborg, Sweden, August 21 - 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4820 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover considering PDU set based handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.NG-RAN’s PDU Set handling capability is notified by whether PDU Set QoS parameter is accept or not during PDU Session establishment procedure. So it would be better to take the same way of indication to SMF during handover(i.e. whether PDU Set QoS parameter is accept or not). Moreover, if the target NG-RAN doesn’t support PDU Set handling or PDU Set QoS parameter is not handed over, it doesn’t need to notify it to SMF. 2. To support PDU Set handing, PSA UPF perfroms PDU Set identification and marking and NG-RAN performs PDU Set handling as their PDU Set QoS parameter. Considering UE mobility, whenever UE may move to a NG-RAN not supporting PDU Set feature, there is no need to de-activate PDU Set identification and marking e.g. expected that UE moves soon to another NG-RAN supporting PDU Set feature. So such de-activation can be done when UE move out of TA or RA. 3. The remaing Editor’s note is removed as when PDU Set QoS handling is configured or determined by PCC rule in SMF, if SMF detects that the target NG-RAN node does not use PDU Set based handling, after handover SMF may trigger PDU Session modification procedure to activate PDU Set based handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. the target NG-RAN provides to the SMF with an indication of whether PDU Set QoS parameter is accepted or not if whether the target NG-RAN node supports PDU Set based handling. 2. deactivation of UPF’s PDU set identification and marking can be done when UE move out of TA or RA. 3. The remaing Editor’s note is removed. 4. in order to activate PDU Set based handling, after handover SMF may trigger PDU Session modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to treat PDU Set handling during handover is ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether PDU Set QoS parameter is accepted or not if the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the NG-RAN indications, the SMF may configure the PSA UPF to activate/deactivate the PDU Set identification and marking.

NOTE xx : If the source NG-RAN supports PDU Set QoS handling, but the target NG-RAN doesn’t supports, the SMF may keep activate PDU Set identification and marking of the PSA UPF for the corresponding QoS flows until UE moves out of TA or RA.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

When PDU Set QoS handling is configured or determined by PCC rule in SMF, if the target NG-RAN node provides indication of ‘PDU Set QoS parameter is not accepted’ or provides no indication on PDU Set QoS parameter during handover, SMF may trigger PDU Session modification procedure to activate PDU Set based handling.

********** End of Changes **********