3GPP TSG-SA2 Meeting #158	S2-2308996

# August 21 – 25, 2023, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0323 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Terminologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarification on terminologies regarding 5G ProSe UE-to-UE Relay. User Info ID is used in 5G ProSe UE-to-UE Relay Discovery as well, but in the definition of User Info ID, there is no usage case of 5G ProSe UE-to-UE Relay Discovery. Also, the definition of the UE-to-UE Relay Layer indication is missing in subcluase 3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on definition of User Info ID and adding the definition of the UE-to-UE Relay Layer indication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding the User Info ID and difficult to understand new terminology |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G ProSe-enabled UE: A UE that supports 5G ProSe requirements and associated procedures.

5G ProSe Direct Discovery: A procedure employed by a 5G ProSe-enabled UE to discover other 5G ProSe-enabled UEs in its vicinity based on direct radio transmissions between the two UEs with NR technology.

5G ProSe Direct Communication: A communication between two or more UEs in proximity that are 5G ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node.

5G ProSe UE-to-Network Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity to the network for 5G ProSe Remote UE(s).

5G ProSe Remote UE: A 5G ProSe-enabled UE that communicates with a DN via a 5G ProSe UE-to-Network Relay.

5G ProSe UE-to-UE Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity between 5G ProSe End UEs.

5G ProSe End UE: A 5G ProSe-enabled UE that connects with another 5G ProSe-enabled UE(s) via a 5G ProSe UE-to-UE Relay.

Application Layer ID: An identifier identifying a 5G ProSe-enabled UE within the context of a specific application. The format of this identifier is outside the scope of 3GPP.

Direct Network Communication: One mode of network communication, where there is no 5G ProSe UE-to-Network Relay between a UE and the 5G network.

Indirect Network Communication: One mode of network communication, where there is a 5G ProSe UE-to-Network Relay between a UE and the 5G network.

Member ID: An identifier uniquely identifying a member in the Application Layer managed group and that is managed by the ProSe application layer.

Mode of communication: Mode of communication to be used by the 5G ProSe-enabled UE over PC5 reference point, i.e. broadcast mode, groupcast mode or unicast mode.

Open ProSe Discovery: ProSe Direct Discovery without explicit permission from the 5G ProSe-enabled UE being discovered, according to TS 22.278 [7].

ProSe identifier: A globally unique identifier used to identify the ProSe Application associated with the ProSe operation in 5G ProSe Direct Discovery and 5G ProSe Direct Communication. In this Release, the "Application ID" defined in TS 23.303 [3] can be used as the ProSe identifier in 5G ProSe Direct Discovery and in a consequent 5G ProSe Direct Communication.

Restricted ProSe Discovery: ProSe Direct Discovery that only takes place with explicit permission from the 5G ProSe-enabled UE being discovered, according to TS 22.278 [7].

Relay Service Code: A Relay Service Code is used for the case of UE-to-Network Relay as well as for the case of UE-to-UE Relay. The definition for the case of UE-to-Network Relay is in TS 23.303 [3]. For the case of UE-to-UE Relay, the Relay Service Code is used to identify a connectivity service the 5G ProSe UE-to-UE Relay provides, and the authorized users the 5G ProSe UE-to-UE Relay would offer service to. The definition of values of Relay Service Code for the case of UE-to-UE Relay is out of scope of this specification.

UE-to-UE Relay Layer indicator: Information included in 5G ProSe UE-to-UE Relay Discovery parameters for indicating whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

User Info ID: The User Info ID is configured for Model A or Model B Group Member Discovery, 5G ProSe UE-to-Network Relay Discovery, and 5G ProSe UE-to-UE Relay Discovery either for public safety or commercial applications based on the policy of the HPLMN or via the ProSe application server that allocates it. The definition of values of User Info ID is out of scope of this specification.

For the purposes of the present document, the following term and definition given in TS 23.303 [3] apply:

Application Layer Group ID

Destination Layer-2 ID

Discovery Entry ID

Discovery Filter

Discovery Query Filter

Discovery Response Filter

Geographical Area

Local PLMN

Model A

Model B

Metadata Index Mask

ProSe Application ID

ProSe Application Code

ProSe Application Mask

ProSe Query Code

ProSe Response Code

ProSe Restricted Code

ProSe Restricted Code Prefix

ProSe Restricted Code Suffix

ProSe Discovery UE ID

ProSe Layer-2 Group ID

Restricted ProSe Application User ID

Source Layer-2 ID

For the purposes of the present document, the following term and definition given in TS 23.287 [2] apply:

NR Tx Profile

* * * End of Changes * * *