3GPP TSG-WG SA2 Meeting #158	S2-2309105

Goteborg, Sweden, Aug 21 – 25, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] Reply LS on INACTIVE eDRX above 10.24sec and SDT

# Response to:	LS (S2-2308320/R3-233347) on Reply LS on INACTIVE eDRX above 10.24sec and SDT from RAN3

# Release:	Release 18

# Work Item:	NR_RedCap_Ph2, NR_redcap_enh-Core

Source:	[Huawei to be] SA2

To:	RAN3

Cc:	CT4, RAN2

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	Steven DOT Wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

SA2 thanks RAN3 for the LS (R3-233347/S2-2308320) on “Reply LS on INACTIVE eDRX above 10.24sec and SDT” and would like to provide the following feedback.

1. 	RAN3 has agreed to include the RAN configured Paging Time Window (PTW) in the MT COMMUNICATION HANDLING REQUEST message, when the NG-RAN sends this message to request CN applying HLCOM.

SA2 thanks RAN3 for the information and will take this into account.

2.	Following SA2 and CT4 updates, RAN3 has agreed to include the QFI, PPI, ARP and PDU session ID of the QoS Flow in the DL DATA NOTIFICATION message from AMF to NG-RAN requesting connection resume. Concerning signalling 5QI, RAN3 is still discussing it and would like to check with SA2, if the signalled 5QI can be different than the one associated with the stored QFI in the RAN UE context.

SA2 would like to point out that the signalled 5QI is the same as the one associated with the stored QFI in the RAN UE context. Based on the QFI, the NG-RAN can get the ARP and 5QI accordingly and hence no need to include the ARP and 5QI in the DL DATA NOTIFICATION message.

3.	Furthermore, RAN3 has discussed the need to signal the DL data size in the DL DATA NOTIFICATION message to help NG-RAN making decision if MT-SDT paging can be performed based on the QFI associated with the configured SDT radio bearer. RAN3 would like to ask SA2 and CT4 whether it is feasible that DL data size can be signalled to the NG-RAN (e.g. for QoS flows that have buffered data in the CN) along with the paging differentiation parameters to assist NG-RAN for MT-SDT paging decision.

SA2 confirms that it's feasible to provide the DL data size to NG-RAN and will keep RAN3 informed if any progress.

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to take the above into account and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #159		October 09 – 13, 2023			Xiamen, CN

TSG-SA2 Meeting #159		November 13 – 17, 2023			Chicago, USA